3GPP TSG-RAN WG3 Meeting #111-e	R3-210661

E-meeting, 25 January – 4 February, 2021

Agenda item:	15.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Slice scope in QoE measurement activation and reports

Document for:	Text Proposal

# 1	Introduction

This paper contains in annex a text proposal for TR 38.890 clause 6.9 introducing missing details relative to handling of slice scope in QoE measurement activation and reports.

# Annex	- TP for TR 38.890

<<< start of changes >>>

## 6.9 	Per slice QoE measurement

### 6.9.1 	Scenario and requirements

Figure 6.9.1-1: An example of the same service type served by different slices

Figure 6.9.1-1 is an example of the same service type served by different slices. As shown in the figure, UE1 is served by Slice #1 and UE 2 is served by Slice #2. If the Service Level Agreement (SLA) of Slice #1 and Slice #2 are different, QoE of UE1 and UE2 should be different for the same service type.

Collecting QoE per slice is beneficial for both management system (e.g. OAM) and network functions (e.g. UDM, NWDAF, etc.). The requirements of per slice QoE measurement in 5G includes:

-	slice SLA maintenance and enforcement in OAM.

-	slice experience analysis and prediction in NWDAF.

-	better slice selection decision in NSSF.

### 6.9.2 	Solution

#### 6.9.2.1 	Configuration

The Slice Scope information should be included in the QoE configuration. The slice scope may consist of list of slice ids and slice id ranges. In case of management-based QMC activation the slice scope information corresponds to the scope of slices allowed by the CN. The gNB will select UEs having PDU sessions with slice ID within the slice scope information received from OAM. The slice scope information is also forwarded by the gNB to the UE, and the UE will use this information to select the appropriate application client session(s) for QoE measurements.

In case of signaling-based QMC activation, which may originate either in the H-PLMN or the V-PLMN (in case of roaming), the slice scope information corresponds to the scope of slices allowed by the CN or the scope of UE subscribed slices. The received slice scope information, within the QMC configuration information, is transparently forwarded by the gNB to the UE.

#### 6.9.2.2 	Mapping

Mapping of slice scope information received for QMC configuration takes place in the UE. If the received slice scope information corresponds to slices to which the UE is subscribed, the UE maps the slice scope onto the corresponding allowed slice(s).

#### 6.9.2.3 	Reporting

The slice identification should be included in the QoE report. The UE includes in the QoE report:

- the allowed slice id, which is the slice id of the PDU session on which the reported QoE measurement(s) are performed, and

- the corresponding subscribed slice id.

<<< end of changes >>>