3GPP TSG-RAN3 #111-e	R3-210686

25 January – 5 February 2021

Online

Agenda Item:	10.2.6

Source:	Ericsson

Title:	(TP for SON BL CR for TS 38.300): Conditional handover SON aspects

Document for:	Discussion, TP

# Introduction

In RAN #86 the new WID on enhancement of data collection for SON/MDT in NR was approved. In the following we will discuss the objectives of the WID and in particular focus on Conditional handover (CHO), bringing forward our proposals on topics to be studied, specifically in connection to failure cases and message forwarding.

# Discussion

## Introduction

In the New WID on enhancement of data collection for SON/MDT in NR [1] that was approved in RAN#86 and revised at RAN#88e, the objectives of the work item were presented. Specifically, the following was captured:

Support of data collection for SON features, including CCO, inter-system inter-RAT energy saving, 2-step RACH optimization, mobility enhancement optimization, and leftovers of Rel-16 SON/MDT WI (PCI selection, energy efficiency (OAM requirements), Successful Handovers Reports, UE history information in EN-DC, load balancing enhancement, MRO for SN change failure, RACH Optimisation enhancements) [RAN3, RAN2]

- Specification of the UE reporting necessary to enhance the network configuration [RAN2].

- Specification of the inter-node information exchange, including possible enhancements to S1/NG, X2/Xn, and F1/E1 interfaces [RAN3]

Data collection for mobility enhancement optimization is included among the objectives. Already in Rel-16 one of the WIs was about E-UTRAN and NR Mobility Enhancements putting forward the concept of CHO. Thus, SON related optimization for CHO can be henceforth studied.

In RAN3#109 online meeting following agreements have been made:

- FFS whether CHO specific failure types are needed. The existing definitions of too late handover /too early handover/ handover to wrong cell are the starting point for further study.

- From RAN3 point of view, in order to support MRO for CHO, more information is needed from UE. (FFS on the details).

- Study the contents of the RLF INDICATION or HANDOVER REPORT message to support MRO enhancements for CHO. In order to progress in this area it is necessary to converge on the CHO failure case definition.

In RAN3#110 online meeting following agreements regarding CHO were made:

- Cover CHO failure scenarios; whether to define CHO specific failure types or reuse the existing failure types with some necessary update is FFS.

- Data forwarding enhancements on HO to wrong cell is de-prioritized in this WI

- Resource optimization for Conditional Handover is FFS

MRO aims to address different failure types such as too late HO, HO to wrong cell, Too Early HO, for a regular HO. As part of Rel 16, and in TS 38.300, a number of failure types are defined. The following intra-system failure events are specified;

[Too Late Handover] An RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

[Too Early Handover] An RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

[Handover to Wrong Cell] An RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

Based on the detected failure type (which is enabled via RLF report transferred via Failure Indication and HO Report signals), proper actions can be taken by gNB to mitigate the detected failures. For example, in case of Too Late Handover, actions normally aim at performing handover earlier, while for Too Early Handover, actions normally aim at delaying the handover. In the case of Handover to Wrong Cell, actions could be taken either aiming at performing handover between source and target cell later or aiming at performing handover between source and reestablishment cell earlier.

In Release-17 Mobility Enhancements, 3GPP aims at improving the mobility robustness for LTE and NR. One of the use cases of mobility enhancements may be the Conditional Handover (CHO) feature, where the network prepares the UE for HO to multiple candidate cells in advance, before the source radio link becomes too weak and leads to a handover failure. CHO will decrease the risk of failures and increase the robustness. In the following we discuss the importance of the optimization for CHO parameters.

## Conditional HO parameters

In a typical CHO operation the network configures measurement reports whose trigger point for A3 event would be much earlier than a typical trigger point for handovers, to minimize the risk of transmission of a too late measurement report, and, upon that reception, the network determines to configure CHO for one or multiple target candidate cells. Each of these candidate cells needs to be prepared, as if they would be a handover candidate cell.

CHO can be configured for one or more target candidates, where each candidate is considered as a prepared cell.

When CHO is configured, for each target cell candidate the UE receives a CondReconfigToAddMod IE containing an RRC Reconfiguration to be applied upon the fulfillment of an execution condition, and a configuration for that execution condition, as shown below:

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Cond condReconfigAdd

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...

}

As shown above, the configuration of the execution condition is a set of one or two MeasID(s), each of these pointing to the measurement configuration whose reportConfig has as reportType set to condTrigerConfig IE, received as part of ReportConfigNR. As shown below an execution of a conditional HO can be triggered on a conditional A3 or an A5 type event.

Figure 1. Conditional HO triggering configuration based on A3 or A5 events.

If a single MeasID is configured for the execution condition associated to a given target candidate, the monitoring procedure is somewhat similar to the evaluation of event-triggered measurement reports In other words, the UE executes CHO if the execution condition is considered as fulfilled for the applicable cells for all measurements after layer 3 filtering taken during the corresponding Time to Trigger (TTT) defined for this event. If two MeasIDs are configured for the execution condition associated to a given target candidate, the UE executes CHO if both conditions are considered fulfilled. A typical use case for using two MeasID(s) is when the network sets the same event but with different trigger quantities like RSRP and RSRQ, so the UE only executes the handover when the target cell candidate is better in terms of both RSRP and RSRQ.

In CHO, for each target candidate one or two measID(s) can be configured.

In CHO, each configured MeasID(s) has similar parameters to event-triggered reporting configuration e.g. A3 or A5 threshold(s), TTT, etc.

In our understanding, mobility failure while conditional HO is configured may be seen as a costly exercise for the network as significant amount of network resources need to be reserved until a successful execution of CHO. Such resources may include RACH resources as well as enough memory for early data forwarding. Hence the above highlighted parameters (including condEventA3 and condEventA5) require careful consideration as proper configuration of such CHO parameters is important to limit the radio resource and capacity consumption that CHO may bring.

The CHO mechanism leverages significant amount of network resources to provide robust mobility. Hence any CHO failure is costly for the network.

CHO failures may happen due to wrong configuration of CHO triggering parameters (i.e., condTriggerConfig). Optimization of CHO parameters may require careful considerations, when it comes to mobility robustness optimization.

In the following, we exemplify some scenarios that highlight the potential failures occurring due to wrong configurations of the above-mentioned CHO parameters (in Figure 1) as well as selecting wrong candidate CHO target cells.

## Failure events due to CHO

In the following we exemplify different types of the failure events that may happen while a UE is configured with CHO. This in our understanding may require defining new failure types to enable the network detecting such failures, taking proper actions to diminish the probability of such failures.

- Too Late CHO execution – Re-establishment/Handover to a prepared cell:

In a scenario, as shown in Figure 2, a gNB may configure the conditional HO for a UE having cell B and cell C as a target candidate cells. It could happen that while the UE is performing the evaluation of execution conditions (as specified in 5.3.5.13.4, TS 38.331), RLF is detected and the UE initiates a re-establishment procedure (or a handover) towards a prepared cell such as cell B or cell C. This type of failure may indicate that the CHO execution thresholds were not properly set, although the candidate cells where correctly selected.

Such a scenario could be considered as a Too Late Handover for CHO, as RLF occurs while the UE is evaluating CHO conditions, leading to a re-establishment to a different cell (that is a prepared cell). In legacy, this is typically caused by mistuned parameters of an A3/A5 event e.g. thresholds and/or TTT which may be adjusted at source upon reception of an RLF report containing measurements for source and neighbour cells. Similarly, the inclusion of measurements for CHO target candidates in RLF reports may also be helpful for the tuning of A3/A5 events for CHO, so that CHO execution occurs before an RLF with source.

Figure 2: Too Late CHO execution – re-establishment/handover towards a CHO prepared cell.

- Too Late CHO execution – Re-establishment to an un-prepared cell:

In another scenario, as shown in Figure 3, a gNB may configure the conditional HO for a UE having cell B as a target candidate cell, while cell C is not prepared for CHO. It could happen that while the UE is performing the evaluation of execution conditions (as specified in 5.3.5.13.4, TS 38.331), RLF is detected and the UE initiates a re-establishment procedure towards cell C that is not a target candidate for CHO i.e. it is unprepared. Here not only the CHO command triggering thresholds are not configured properly, but also CHO target candidates may not be selected properly so the UE inevitably performed re-establishment towards a cell that is not prepared for CHO. This may mean that the target candidate cells were wrongly selected.

Figure 3: Too Late CHO execution – re-establishment toward an unprepared cell.

Having the above two cases (a and b) we may have a new failure event type as following:

- [Too Late CHO execution] An RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

- Re-establish cell is a prepared cell

- Re-establish cell is not a prepared cell

- Too Early CHO execution – Re-establishment to the serving/source cell:

In another scenario, as shown in Figure 4, a gNB may configure the conditional HO for a UE having cell B and cell C as a target candidate cell. It could happen that UE declares RLF while executing CHO toward a candidate cell or right after successful CHO execution and UE selects the source cell as a suitable cell and initiates re-establishment toward the source cell. This event may happen due to mistuned thresholds for CHO execution, but it does not depend on the candidate cells selection because the UE ends up in the same source cell.

Figure 4: Too Early CHO execution – re-establishment toward serving/source cell.

Having the above case, we may have a new failure type as following:

- [Too Early CHO execution] An RLF occurs shortly after a successful CHO from a source cell to a target cell or a CHO failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

- CHO to wrong cell – Re-establishment/handover to the prepared cell:

In yet another scenario, as shown in Figure 5, a gNB may configure the conditional HO for a UE having cell B and cell C as a target candidate cells. It could happen that UE declares RLF while executing CHO toward a candidate cell B or right after successful CHO execution and UE selects cell C which is a prepared cell for CHO. This event may happen due to mistuned thresholds for CHO execution, but the problem may not be the selection of candidate cells because UE ends up in another prepared cell.

Figure 5: CHO to wrong cell – Re-establishment/handover to the prepared cell.

- CHO to wrong cell – Re-establishment to an un-prepared cell:

In yet another scenario, as shown in Figure 6, a gNB may configure the conditional HO for a UE having cell B as a target candidate cell while cell C is not prepared for CHO. It could happen that UE declares RLF while executing CHO toward a candidate cell B or right after successful CHO execution to cell B. the UE selects cell C which is a not prepared cell for CHO. This event may happen due to mistuned thresholds for CHO execution, but also it may be about the fact that the candidate cells are wrongly selected as UE ends up in another prepared cell.

Figure 6: CHO to wrong cell – Re-establishment to an un-prepared cell.

Having the above case, we may have a new failure type as following:

- [CHO to Wrong Cell] An RLF occurs shortly after a successful CHO from a source cell to a target cell or a CHO failure occurs during the handover procedure; the UE attempts to select a cell other than the source cell and the target CHO cell to recover from failure via

- Re-establishment if the selected cell is not a CHO prepared cell

- Reconfiguration with sync if the selected cell is a CHO prepared cell

UE may fail at CHO execution and re-establishes a connection to a cell which may not be prepared for CHO. An optimization that allows a-priori preparation of the reestablishment cell for CHO may enhance the CHO performance causing a better experience for UE.

## Resolving the detected failure events

In our understanding, RAN 3 may need to define new failure events covering the above-mentioned failure types. In fact, we believe that if the RLF report provides sufficient amount of measurements for detection of such failures, the network may be able to tune the dedicated CHO parameters to avoid such CHO related failures.

- It is proposed that RAN3 considers the following failure types:

- [Too Late CHO execution] An RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to select a different cell to recover from the failure via

- Re-establishment if the selected cell is not a CHO prepared cell

- Reconfiguration with sync if the selected cell is a CHO prepared cell

- [Too Early CHO execution] An RLF occurs shortly after a successful CHO from a source cell to a target cell or a CHO failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

- [CHO to Wrong Cell] An RLF occurs shortly after a successful CHO from a source cell to a target cell or a CHO failure occurs during the handover procedure; the UE attempts to select a cell other than the source cell and the target CHO cell to recover from failure via

- Re-establishment if the selected cell is not a CHO prepared cell

- Reconfiguration with sync if the selected cell is a CHO prepared cell

## Resource optimization

In Observation 4, we highlighted the fact that the CHO mechanism leverages significant amount of network resources to provide robust mobility. It is imperative thus to actively focus on resource optimization.

As a first step we will strive to identify possible areas of optimization.

One area concerns the optimization of the number of prepared cells. Examples of possible resources reserved in candidate cells are e.g. C-RNTI and preambles. Other resources are DRB resources for critical services. Also, several candidate configurations in the same message increases the length and thus vulnerability of the RRCReconfiguration message.

At first the source would not have the necessary knowledge to choose the best cells to prepare. As a result, the source will prepare many different cells. The reservation of resources in the target would lead to an overload of the network, which is clearly undesirable. Therefore we need to investigate methods to optimize the number of prepared cells.

Based on the above we propose that RAN3 will study the optimization of the number of prepared cells.

- RAN3 to study the optimization of the number of prepared cells.

Currently there is no time limit for how long a candidate cell will be reserved for CHO. A time limit could help limit the amount of resources reserved for CHO. Such a timer could be set either in the source cell or the candidate cell and it could therefore be sent either from source cell to candidate cell or vice versa.

Therefore, we propose that RAN3 should study a timer deciding time limit for how long the resources in candidate cells should be occupied and how it can be communicated between source and candidate cell

- RAN3 to study a timer deciding time limit for how long the resources in candidate cells should be occupied and how it can be communicated between source and candidate cell.

Furthermore, it has to be decided if the UE also needs to know the timer value in order to independently remove a certain candidate cell or if the UE should be told explicitly by the candidate cell in a RRCReconfiguration message.

It should be considered how the UE will know about when to remove a certain candidate cell from its configuration.

Another area of interest has to do with optimization of early and late data forwarding. Early data forwarding is beneficial in reducing data interruption. On the other hand, early data forwarding will increase overhead on backhaul and memory usage in candidate cells. Since early data forwarding is costly, we need to study methods to optimize its usage.  Besides, late data forwarding facilitates PDCP SN continuity.

Important to note here that UE continues to receive packet even after reception of the CHO configuration, which means that certain amount of traffic will be duplicated while doing early data forwarding thus it is beneficial to include User Plane (UP) measurements in order to enable efficient resource optimization.

UE continues sending/receiving UP data after the reception of CHO configuration so certain number of packets will be duplicated while using early data forwarding thus including UP measurements might be beneficial for resource optimization

Considering the above we propose that RAN3 studies methods to optimize early and late data forwarding.

- RAN3 to study methods to optimize early and late data forwarding.

- RAN3 to create LS to RAN2 to request UP measurements.

Possible optimizations for early data forwarding are e.g.

- If several candidate cells are in the same node, then one possible optimization is to bundle traffic for one UE to these cells, namely, to use one data forwarding connection for the whole set of target cells

- Only forward traffic for critical services like voice

- Don’t set up forwarding traffic immediately, wait for other triggers, which could be e.g. another measurement report from the UE

# Conclusion

We made the following observations:

Observation 1	CHO can be configured for one or more target candidates, where each candidate is considered as a prepared cell.

Observation 2	In CHO, for each target candidate one or two measID(s) can be configured.

Observation 3	In CHO, each configured MeasID(s) has similar parameters to event-triggered reporting configuration e.g. A3 or A5 threshold(s), TTT, etc.

Observation 4	The CHO mechanism leverages significant amount of network resources to provide robust mobility. Hence any CHO failure is costly for the network.

Observation 5	CHO failures may happen due to wrong configuration of CHO triggering parameters (i.e., condTriggerConfig). Optimization of CHO parameters may require careful considerations, when it comes to mobility robustness optimization.

Observation 6	Having the above two cases (a and b) we may have a new failure event type as following:

[Too Late CHO execution] An RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

a-	Re-establish cell is a prepared cell

b-	Re-establish cell is not a prepared cell

Observation 7	Having the above case, we may have a new failure type as following:

[Too Early CHO execution] An RLF occurs shortly after a successful CHO from a source cell to a target cell or a CHO failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

Observation 8	Having the above case, we may have a new failure type as following:

[CHO to Wrong Cell] An RLF occurs shortly after a successful CHO from a source cell to a target cell or a CHO failure occurs during the handover procedure; the UE attempts to select a cell other than the source cell and the target CHO cell to recover from failure via

a-	Re-establishment if the selected cell is not a CHO prepared cell

b-	Reconfiguration with sync if the selected cell is a CHO prepared cell

Observation 9	UE may fail at CHO execution and re-establishes a connection to a cell which may not be prepared for CHO. An optimization that allows a-priori preparation of the reestablishment cell for CHO may enhance the CHO performance causing a better experience for UE.

Observation 10	It should be considered how the UE will know about when to remove a certain candidate cell from its configuration.

Observation 11	UE continues sending/receiving UP data after the reception of CHO configuration so certain number of packets will be duplicated while using early data forwarding thus including UP measurements might be beneficial for resource optimization

Based on the discussion, we propose the following:

Proposal 1	It is proposed that RAN3 considers the following failure types:

[Too Late CHO execution] An RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to select a different cell to recover from the failure via

a.	Re-establishment if the selected cell is not a CHO prepared cell

b.	Reconfiguration with sync if the selected cell is a CHO prepared cell

[Too Early CHO execution] An RLF occurs shortly after a successful CHO from a source cell to a target cell or a CHO failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

[CHO to Wrong Cell] An RLF occurs shortly after a successful CHO from a source cell to a target cell or a CHO failure occurs during the handover procedure; the UE attempts to select a cell other than the source cell and the target CHO cell to recover from failure via

a.	Re-establishment if the selected cell is not a CHO prepared cell

b.	Reconfiguration with sync if the selected cell is a CHO prepared cell

Proposal 2	RAN3 to study the optimization of the number of prepared cells.

Proposal 3	RAN3 to study a timer deciding time limit for how long the resources in candidate cells should be occupied and how it can be communicated between source and candidate cell.

Proposal 4	RAN3 to study methods to optimize early and late data forwarding.

Proposal 5	RAN3 to create LS to RAN2 to request UP measurements.

# References

TR 37.816 V16.0.0 (2019-07), 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on RAN-centric data collection and utilization for LTE and NR (Release 16)

TS 38.300

# Annex

TP for SON BL CR for TS 38.300

============ Start of 1st change ==============

### 15.5.x		Support for Mobility Robustness Optimization for Conditional Handover

#### 15.5.x.1 Connection Failure

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Conditional Handovers, or Conditional Handover to Wrong Cell. These problems are defined as follows

[Too Late CHO execution] An RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to select a different cell to recover from the failure via

- Re-establishment if the selected cell is not a CHO prepared cell

- Conditional Handover (Reconfiguration with sync) if the selected cell is a CHO prepared cell

[Too Early CHO execution] An RLF occurs shortly after a successful CHO from a source cell to a target cell or a CHO failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

[CHO to Wrong Cell] An RLF occurs shortly after a successful CHO from a source cell to a target cell or a CHO failure occurs during the handover procedure; the UE attempts to select a cell other than the source cell and the first selected target CHO cell to recover from failure via

- Re-establishment if the selected cell is not a CHO prepared cell

- Conditional Handover (Reconfiguration with sync) if the selected cell is a CHO prepared cell

In the definition above, the "successful CHO" refers to the UE state, namely the successful completion of the RA procedure.