3GPP TSG-RAN WG3 Meeting # 111-e	R3-210735

# Online,  January 25th 2021 - February 4th 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Local NG-RAN Node Identifier. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T, Verizon Wireless, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A standardized solution is needed to enable inter-vendor interoperable way for an NG RAN node to deduce the identity of another NG RAN node from the received I-RNTI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the definition of a new “Local NG-RAN Node Identifier”. Add a normative section to describe the use of the “Local NG-RAN Node Identifier” to resolve the old NG-RAN node from the I-RNTI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to support successful RRC Resume procedure in inter-vendor scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2, Annex xx (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 R3-210736 TS 38.423 R3-210728 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------  Start of Changes ---------------------------------------------------------

## 8.2	Network Identities

The following identities are used in NG-RAN for identifying a specific network entity:

-	AMF Name: used to identify an AMF.

-	NR Cell Global Identifier (NCGI): used to identify NR cells globally. The NCGI is constructed from the PLMN identity the cell belongs to and the NR Cell Identity (NCI) of the cell. The PLMN ID included in the NCGI should be the first PLMN ID within the set of PLMN IDs associated to the NR Cell Identity in SIB1, following the order of broadcast.

NOTE 1:	How to manage the scenario where a different PLMN ID has been allocated by the operator for an NCGI is left to OAM and/or implementation.

-	gNB Identifier (gNB ID): used to identify gNBs within a PLMN. The gNB ID is contained within the NCI of its cells.

-	Global gNB ID: used to identify gNBs globally. The Global gNB ID is constructed from the PLMN identity the gNB belongs to and the gNB ID. The MCC and MNC are the same as included in the NCGI.

NOTE 2:	It is not precluded that a cell served by a gNB does not broadcast the PLMN ID included in the Global gNB ID.

-	Tracking Area identity (TAI): used to identify tracking areas. The TAI is constructed from the PLMN identity the tracking area belongs to and the TAC (Tracking Area Code) of the Tracking Area.

-	Single Network Slice Selection Assistance information (S-NSSAI): identifies a network slice.

-	Network Identifier (NID): identifies an SNPN in combination with a PLMN ID.

-	Closed Access Group Identifier: identifies a CAG within a PLMN.

-	Local NG-RAN Node Identifier: used to identify an NG-RAN node within a set of NG-RAN nodes that can interoperate over the Xn-C interface.

-----------------------------------------------  Next Change ---------------------------------------------------------

Annex xx (normative):

Resolve old NG-RAN node from I-RNTI

The I-RNTI provides the new NG-RAN node a reference to the UE context in the old NG-RAN node.

To support the new NG-RAN node to resolve the old NG-RAN node from the I-RNTI, the new NG-RAN node should include in the first leftmost bits of the I-RNTI a Local NG-RAN Node Identifier as specified in TS 38.423.

-----------------------------------------------  End of Changes --------------------------------------------------------