3GPP TSG-RAN WG3 #111 electronic															R3-210768

Online, Jan 25 – Feb 5, 2021

Title: 	Some Considerations on How to Support CPAC

Source: 	CATT

Agenda item:	14.3

Document for:	Discussion and Decision

1. Introduction

According to the discussion of last RAN3 meeting, there are still the following open issues to be resolved regarding the CPAC procedure:

FFS on how to support CPAC replace, (SN modification procedures or SN Addition or others).

FFS on how to support multiple candidate PSCell preparation in CPAC:

- Option 1: prepare one PSCell in one CPAC procedure, use parallel CPAC procedures to prepare multiple PSCells.

Need to introduce an indicator to distinguish the triggering of different PSCell preparation for the same UE”.

- Option 2: prepare multiple PSCells in one CPAC procedure

FFS if multiple SN can be prepared in one SN initiated CPC procedure (SN Change Required).

FFS on how to handle the received CPC execution condition by the MN in case of SN initiated inter-SN CPC, pending to RAN2 progress.

It is pending to RAN2 on if it is needed for the Target SN to send CPAC success to the MN, and if it is needed, FFS on reusing HO Success or introduce a new class2 procedure.

FFS on F1AP and E1AP impacts.

FFS if conditional SN change can be prepared directly between the involved SNs (depends on availability of SRB3).

In this contribution, we focus on these open issues, and provide some analysis and proposals.

2. Discussion

Issue 1：FFS on how to support CPAC replace, (SN modification procedures or SN Addition or others)

The SN modification procedures can be used to modify part of the radio configuration instead of replacing it completely, whereas Handover Request + replace ID is used in CHO to completely replace the original configuration. Therefore, the SN Modification procedure is more suitable for changing CPAC configuration information from the perspective of signaling overhead and flexibility.

Proposal 1: To use the SN modification procedure to support CPAC configuration update.

Issue 2：FFS on how to support multiple candidate PSCell preparation in CPAC:

According to the email discussion of last RAN3 meeting [2], there are two solutions for consideration;

- Option 1: prepare one PSCell in one CPAC procedure, use parallel CPAC procedures to prepare multiple PSCells.

  - Need to introduce an indicator to distinguish the triggering of different PSCell preparation for the same UE”.

- Option 2: prepare multiple PSCells in one CPAC procedure

  - FFS if multiple SN can be prepared in one SN initiated CPC procedure (SN Change Required).

For DC scenarios, an UE signaling link can exist for long time of period, and any configuration update is done via the SN modification procedure. However, for the case of multiple PScells, if parallel CPAC procedures are adopted, which obviously has the issue of signaling overhead redundancy. Therefore, Option 2 has some advantages in terms of signaling overhead and flexibility.

Besides, for the case of multiple SNs, multiple SN addition procedures can be allowed for MN initiated CPAC procedure, and accordingly, multiple SNs prepared in one SN initiated CPC procedure should also be allowed.

Proposal 2：Multiple SNs can be prepared in one SN initiated CPC procedure (SN Change Required).

Issue 3：FFS on how to handle the received CPC execution condition by the MN in case of SN initiated inter-SN CPC, pending to RAN2 progress

According to the email discussion of last RAN3 meeting [2], there are two solutions for consideration;

- Option 1: MN provides the received CPC execution condition to the target SN

- Option 2: MN does not provide the received CPC execution condition to the target SN,

However, according to the LS from RAN2 [], “For SN initiated inter-SN CPC, the MN generates CPC. The source SN sets the execution condition and communicates it to the MN. The MN generates the conditional reconfiguration message including the execution condition(s) provided by the source SN and RRCReconfiguration provided by the candidate PSCell(s)”, apparently, the received CPC execution condition does not need to be signalled to the target SN. Therefore, Option 2 should be adopted.

Proposal 3：MN does not provide the received CPC execution condition to the target SN.

Issue 4：It is pending to RAN2 on if it is needed for the Target SN to send CPAC success to the MN, and if it is needed, FFS on reusing HO Success or introduce a new class 2 procedure.

According to the email discussion of last RAN3 meeting [2], there are three solutions for consideration;

- Option 1: Reuse HO Success Procedure

- Option 2: introduce a new class 2 procedure

- Option 3: UE indicates access directly to the MN (up to RAN2)

According to the discussion of RAN2, the signaling flow for CPAC is shown as below:

Step 4a: If at least one CPA candidate PSCell satisfies the corresponding CPA execution condition, the UE applies the stored corresponding configuration for the selected candidate PSCell and synchronizes to that candidate PSCell. The UE completes the CPA execution procedure by sending an RRCReconfigurationComplete message to the new PSCell via MN.

Therefore, Option 3 should be adopted.

Proposal 4：UE indicates access directly to the new PSCell via MN.

Issue 5：FFS if conditional SN change can be prepared directly between the involved SNs (depends on availability of SRB3)

This issue has association with whether to support direct interface between SNs, from the perspective of the deployment of heterogeneous network, it is unable to guarantee that there must be a logical interface between small BSs. Besides, for legacy SN change procedure, direct communication between SNs is also not supported, so for the CPAC case, such enhancement should be considered as low priority.

Proposal 5：Whether to prepare conditional SN change directly between the involved SNs should be considered as low priority.

3. Conclusion

After the above analysis, we provide the following observations and proposals

Proposal 1: To use the SN modification procedure to support CPAC configuration update.

Proposal 2：Multiple SNs can be prepared in one SN initiated CPC procedure (SN Change Required).

Proposal 3：MN does not provide the received CPC execution condition to the target SN.

Proposal 4：UE indicates access directly to the new PSCell via MN.

Proposal 5：Whether to prepare conditional SN change directly between the involved SNs should be considered as low priority.

4. Reference

[1] RP-201040 Revised WID on Further Multi-RAT Dual-Connectivity enhancements