3GPP TSG-RAN WG3 Meeting #111-e	R3-210802

- 25 Jan - 4 Feb 2021

- Online

Agenda item:	10.2.4

Source: 	ZTE, China Telecom, China Unicom

Title: 	Further Discussion on Inter-system Load Balancing in NR

Document for:	Discussion and Approval

Introduction

In the two previous RAN3 meetings, the inter-system load balancing in NR has been discussed and some agreements were reached.

Introduce Inter System Load Balancing mechanisms on the basis of the solution available in E-UTRAN

Introduce Inter System Load Balancing by means of mechanisms that resemble or reuse the SON Configuration Transfer IE for the purpose of configuring load balancing metrics and reporting load balancing measurements

Use S1: eNB CONFIGURATION TRANSFER, S1: MME CONFIGURATION TRANSFER, NG: UL RAN CONFIGURATION TRANSFER and  NG: DL RAN CONFIGURATION TRANSFER for the transfer of inter system load balancing via means of mechanisms that resemble or reuse the SON Configuration Transfer IEs. It is FFS whether further details on the signaling part need to be introduced

Adopt signaling of the Composite Available Capacity (Cell Capacity Class value and Capacity Value) for inter system MLB

Adoption of further MLB metrics is FFS

Event Based Reporting and Periodic Reporting (only in case specific conditions are met), are agreed to be supported for inter system MLB. The mechanism should avoid excessive signaling

Introduce a new mechanism for Inter System Status Request/Response/Update over NG: UL RAN CONFIGURATION TRANSFER and  NG: DL RAN CONFIGURATION TRANSFER, via modification of the Inter-System SON Information IE

Introduce a new mechanism for Inter System Status Request/Response/Update over S1: UL RAN CONFIGURATION TRANSFER and  S1: DL RAN CONFIGURATION TRANSFER, via reuse of the Inter-System SON Configuration Transfer IE

Support periodic inter system load reporting with periodicity not lower than 1000ms and threshold-based load reporting, subject to confirmation from CT

We do not support per slice load information for inter system load balancing in the current release

Support an explicitly signaled threshold configuration for inter system load information reporting; details are FFS

Agree to CAC encoding as defined in LTE, e.g. in TS36.413, as a starting point. Whether CAC is encoded according to the sender’s rules is FFS

Whether to support the Number of active UEs for inter system load balancing is FFS

It is FFS whether to support signaling of PRB utilization for inter system load balancing in the current release;

It is FFS whether to support signaling of the Number of RRC connections for inter system load balancing in the current release;

It is FFS whether to support signaling of the TNL Available Capacity for inter system load balancing in the current release;

To be continued...

In this contribution, we will further discuss the remaining open issues.

Discussion

In last meeting, it was agreed that the Inter-system SON Information IE in UL or DL RAN Configuration Transfer message over NG should be reused. And the principle of using this IE should follow the agreed structure in SON BL CR to TS 38.413[1], i.e. the Inter-System Resource Status Request IE should be included in the Inter-system SON Information Request IE within the Inter-system SON Information IE and the Inter-System Resource Status Reply IE should be included in the Inter-system SON Information Reply IE within the Inter-system SON Information IE. In addition, the Inter-system Resource Status Update IE should be included in the Inter-system SON Information Report IE within the Inter-system SON Information IE. And the detailed IE structure is shown in [2].

As described in TS 36.413, the Inter-system SON Configuration Transfer IE follows the definition of the same IE in TS 38.413. It seems that the inter-system load balancing for 36.413 is not needed to specify.

-----------------------------------------------------------TS 36.413-------------------------------------------------------------------------

9.1.16	 eNB CONFIGURATION TRANSFER

This message is sent by the eNB in order to transfer RAN configuration information.

Direction: eNB  MME.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| SON Configuration Transfer | O |  | 9.2.3.26 |  | YES | ignore |
| EN-DC SON Configuration Transfer | O |  | 9.2.3.26a |  | YES | ignore |
| Inter-system SON Configuration Transfer | O |  | OCTET STRING | Contains the Inter-system SON Configuration Transfer IE as defined in TS 38.413 [44]. | YES | ignore |

9.1.17	 MME CONFIGURATION TRANSFER

This message is sent by the MME in order to transfer RAN configuration information.

Direction: MME  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| SON Configuration Transfer | O |  | 9.2.3.26 |  | YES | ignore |
| EN-DC SON Configuration Transfer | O |  | 9.2.3.26a |  | YES | ignore |
| Inter-system SON Configuration Transfer | O |  | OCTET STRING | Contains the Inter-system SON Configuration Transfer IE as defined in TS 38.413 [44]. | YES | ignore |

-----------------------------------------------------------TS 36.413-------------------------------------------------------------------------

-----------------------------------------------------------TS 38.413-------------------------------------------------------------------------

9.3.3.33	Inter-system SON Configuration Transfer

This IE contains the configuration information, used by e.g., SON functionality, transmitted between an NG-RAN node and an eNB and additionally includes the node identifier of the destination of this configuration information and the node identifier of the source of this information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Transfer Type | M |  |  |  |
| >from E-UTRAN to NG-RAN |  |  |  |  |
| >>Source eNB-ID |  | 1 |  |  |
| >>>Global eNB ID | M |  | 9.3.1.165 |  |
| >>>Selected EPS TAI | M |  | EPS TAI 9.3.3.17 |  |
| >>Target NG-RAN node ID |  | 1 |  |  |
| >>>Global RAN Node ID | M |  | 9.3.1.5 |  |
| >>>Selected TAI | M |  | TAI 9.3.3.11 |  |
| >from NG-RAN to E-UTRAN |  |  |  |  |
| >>Source NG-RAN Node ID |  | 1 |  |  |
| >>>Global RAN Node ID | M |  | 9.3.1.5 |  |
| >>>Selected TAI | M |  | TAI 9.3.3.11 |  |
| >>Target eNB-ID |  | 1 |  |  |
| >>>Global eNB ID | M |  | 9.3.1.165 |  |
| >>>Selected EPS TAI | M |  | EPS TAI 9.3.3.17 |  |
| Inter-system SON Information | M |  | 9.3.3.34 |  |

-----------------------------------------------------------TS 38.413-------------------------------------------------------------------------

Proposal 1:  As the Inter-system SON Configuration Transfer IE in TS 36.413 is aligned with TS38.413, the stage 3 TP of TS 36.413 in not needed for Inter-system Load Balancing.

Considering the other load metrics for exchange between LTE and NR, after checking the IE tables of Resource Status Update message over X2 and Xn, it can be found that the TNL related load and Radio Resource Status have similar definitions in LTE and NR. However, the value format and granularity of these IEs are not same. For TNL related load, the corresponding IE in LTE, i.e. S1 TNL Load Indicator, is to indicate the status of S1 TNL load by using low load, medium load, high load and overloaded. While the corresponding IE in NR, i.e. TNL Capacity Indicator, is to indicate the offered and available capacity of Transport Network by using the integer value. In addition, for Radio Resource Status, the corresponding IE in LTE is only in cell level, while the corresponding IE in NR could be in both cell level and beam level. Therefore, it is not possible for the eNB and gNB to exchange the load metrics directly.

However, as the objective of the inter-system load balancing is to make the source gNB/eNB be aware of the load status of the target eNB/gNB. For the reporting direction from NR to LTE, the load metrics in NR, the Number of active UEs, RRC Connections, TNL Capacity Indicator and even PRB usage, could be considered as more straightaway for the eNB to obtain the load status of target gNB.

For example, considering one scenario that one eNB (SA) has two potential target gNBs (SA) for offloading, if the Number of active UEs can be transmitted from NG-RAN to E-UTRAN by inter-system load balancing, the eNB can have a better view to select the target gNB for offloading, i,e, the gNB with less number of active UEs.

Similarly, for the reporting direction form LTE to NR, in addition to the Composite Available Capacity Group, the other metrics of LTE could be exchanged from eNB to gNB, such as the S1 TNL load and PRB usage.

Proposal 2: The Number of active UEs, RRC Connections, TNL Capacity Indicator and PRB Utilization could be considered to transmit from gNB to eNB, while the S1 TNL Load and PRB Utilization could be considered to transmit from eNB to gNB.

Conclusion

Proposal 1:  As the Inter-system SON Configuration Transfer IE in TS 36.413 is aligned with TS38.413, the stage 3 TP of TS 36.413 in not needed for Inter-system Load Balancing.

Proposal 2: The Number of active UEs, RRC Connections, TNL Capacity Indicator and PRB Utilization could be considered to transmit from gNB to eNB, while the Hardware Load, S1 TNL Load and PRB Utilization could be considered to transmit from eNB to gNB.

The corresponding TPs are given in [2] and [3] respectively.

References

- R3-207241 BLCR to 38.413_Addition of SON features enhancement  Ericsson

- R3-210803 TP for BL CR 38.300 on Inter-system Load Balancing  ZTE, China Telecom, China Unicom

- R3-210804 TP for BL CR 38.413 on Inter-system Load Balancing  ZTE, China Telecom, China Unicom