3GPP TSG-RAN WG3 Meeting #111-e	R3-210804

- 25 Jan - 4 Feb 2021

- Online

Agenda item:	10.2.4

Source: 	ZTE, China Telecom, China Unicom

Title: 	(TP for SON BL CR 38.413) Inter-system Load Balancing

Document for:	Approval

Introduction

This contribution provides a TP for BL CR 38.413 on the inter-system load balancing.

Text Proposal for 38.413

First Change

9.3.3.36	Inter-system SON Information Report

This IE contains the configuration information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| >HO Report Information |  |  |  |  |
| >>Inter-system HO Report | M |  | 9.3.3.40 |  |
| >Failure Indication Information |  |  |  |  |
| >>Inter-system Failure Indication | M |  | 9.3.3.38 |  |
| >Energy Savings Indication |  |  |  |  |
| >>Cell State Indication | M |  | 9.3.3.aa |  |
| >Resource Status Reporting |  |  |  |  |
| >>Inter-system Resource Status Update | M |  | 9.3.3.cc |  |

Editor's Note: The ASN.1 changes for the above changes are not yet included

Next Change

9.3.3.xx	Inter-system SON Information Request

This IE contains the request information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Request | M |  |  |  |
| >NG-RAN Cell Activation |  |  |  |  |
| >>Cell Activation Request | M |  | 9.3.3.zz |  |
| >Inter-system Resource Status Reporting Initiation |  |  |  |  |
| >>Inter-system Resource Status Request | M |  | 9.3.3.dd |  |

Editor's Note: The ASN.1 changes for the above changes are not yet included

Next Change

9.3.3.yy	Inter-system SON Information Reply

This IE contains the reply information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Inter-system SON Information Response | M |  |  |  |
| >NG-RAN Cell Activation |  |  |  |  |
| >>Cell Activation Reply | M |  | 9.3.3.bb |  |
| >Inter-system Resource Status Reporting Initiation |  |  |  |  |
| >>Inter-system Resource Status Reply | M |  | 9.3.3.ee |  |

Editor's Note: The ASN.1 changes for the above changes are not yet included

Next Change

- 9.3.3.dd	Inter-system Resource Status Request

This IE contains the Inter-system resource status request information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | - | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | - | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | EACH | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity |  |  |  |
| CHOICE Inter-system Resource Status Reporting type | M |  | - | - |  |  |
| >Event based Reporting |  |  |  |  |  |  |
| >>Event based Inter-system Resource Status Request | M |  | 9.3.3.ff |  |  |  |
| >Periodic based Reporting |  |  |  |  |  |  |
| >>Reporting Periodicity | M |  | ENUMERATED (1000ms, 2000ms, 5000ms,10000ms, …) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. of cells that can be served by a NG-RAN node. Value is 16384. |

Editor's Note: The ASN.1 changes for the above changes are not yet included

Next Change

- 9.3.3.ee	Inter-system Resource Status Reply

This IE contains the Inter-system load balancing reply information to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Criticality Diagnostics | O |  | - |  | YES | ignore |

Editor's Note: The ASN.1 changes for the above changes are not yet included

Next Change

- 9.3.3.cc	Inter-system Resource Status Update

This IE contains the results of requested measurements for Inter-system load balancing to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.3.3.ff | indicating the overall available resource level in the cell in Downlink and Uplink |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. of cells that can be served by a NG-RAN node. Value is 16384. |

- 9.3.3.ff	Composite Available Capacity Group

The Composite Available Capacity Group IE indicates the overall available resource level in the cell in Downlink and Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Composite Available Capacity Downlink | M |  | 9.3.3.gg | For the Downlink | - |  |
| Composite Available Capacity Uplink | M |  | 9.3.3.gg | For the Uplink | - |  |

- 9.3.3.gg  Composite Available Capacity

The Composite Available Capacity IE indicates the overall available resource level in the cell in either Downlink or Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell Capacity Class Value | O |  | 9.3.3.hh |  | - |  |
| Capacity Value | M |  | 9.3.3.ii | ‘0’ indicates no resource is available, Measured on a linear scale. | - |  |

- 9.3.3.hh	Cell Capacity Class Value

The Cell Capacity Class Value IE indicates the value that classifies the cell capacity with regards to the other cells. The Cell Capacity Class Value IE only indicates resources that are configured for traffic purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell Capacity Class Value | M |  | INTEGER (1..100,...) | Value 1 shall indicate the minimum cell capacity, and 100 shall indicate the maximum cell capacity. There should be a linear relation between cell capacity and Cell Capacity Class Value. | - |  |

- 9.3.3.ii	Capacity Value

The Capacity Value IE indicates the amount of resources that are available relative to the total NG-RAN resources. The capacity value should be measured and reported so that the minimum NG-RAN resource usage of existing services is reserved according to implementation. The Capacity Value IE can be weighted according to the ratio of cell capacity class values, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity . Capacity Value should be measured on a linear scale. | - |  |

Editor's Note: The ASN.1 changes for the above changes are not yet included

Next Change

9.3.3.x1	Event based Inter-system Resource Status Request

This IE is used to request event based Inter-system resource status reporting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inter-system Resource Status Request Indication | M |  | ENUMERATED (above threshold, below threshold, between thresholds, offset, ...) |  |
| Inter-system Resource Status Request Lower Bound Threshold | C-ifInterSystemResourceStatusReportingOnThreshold |  | INTEGER (0..100) | This value represents the lower bound threshold for the Capacity Value at the load reporting cell triggering the inter -system resource status reporting |
| Inter-system Resource Status Request Upper Bound Threshold | C-ifInterSystemResourceStatusReportingOnThreshold |  | INTEGER (0..100) | This value represents the upper bound threshold for the Capacity Value at the load reporting cell triggering the inter -system resource status reporting |
| Inter-system Resource Status Request Offset | C-ifInterSystemResourceStatusReportingOnOffset |  | INTEGER(-100..100,..) | This value represents the delta offset for the Capacity Value, triggering the inter -system resource status reporting |

| Condition | Explanation |
| --- | --- |
| ifInterSystemResourceStatusReportingOnThreshold | This IE shall be present if the Inter-system Resource Status Request Indication IE is set to the value "above threshold ", “below threshold” or “between thresholds”. |
| ifInterSystemResourceStatusReportingOnOffset | This IE shall be present if the Inter-system Resource Status Request Indication IE is set to the value ”offset”. |

End of Changes