3GPP TSG-RAN WG3 Meeting #111e	R3-210842

# 25th Jan - 4th Feb 2021

Agenda item:	15.2

Source: 	China Unicom

Title: 	Further discussion on scenarios and mechanisms for per slice QoE measurement

Document for:	Discussion and Decision

# 1 Introduction

Scenarios and potential mechanisms of per slice QoE measurement were discussed at last RAN3 meeting and the agreements are as follows. This document is proposed for further discussion on scenarios and mechanisms for per slice QoE measurement.

- NR QoE should support per slice QoE measurement.

- RAN3 to study the feasibility and priority of typical scenarios of per slice QoE measurement.

- The Slice Scope should be included in the QoE configuration.

- RAN3 to study the mechanism to support mapping of QoE report and slice identification.

- The slice identification should be included in the QoE report.

To be continued:

- The feasibility and priority of typical scenarios of per slice QoE measurement.

- How and where to include the Slice Scope in the QoE configuration.

- The mechanism to support mapping of QoE report and slice identification.

- How and where to include the slice identification in the QoE report.

# 2 Discussion

- 2.1 Scenarios

The Scenarios of per slice QoE measurement were discussed at last RAN3 meeting [1] and the typical scenarios are shown in table 1.

Table 1 typical scenarios of per slice QoE measurement

| Scenario 1 | APP 1 –service type 1 –slice 1 APP 2 –service type 2 –slice 2 | Scenario 5 | APP 1 | –service type 1 –slice 1 –service type 2 –slice 2 |
| --- | --- | --- | --- | --- |
| Scenario 2 | APP 1 –service type 1 –slice 1 APP 2 –service type 2 –slice 1 | Scenario 6 | APP 1 | –service type 1–slice 1 –service type 2–slice 1 |
| Scenario 3 | APP 1 –service type 1 –slice 1 APP 2 –service type 1 –slice 2 | Scenario 7 | APP 1 | –service type 1–slice 1 –service type 1–slice 2 |
| Scenario 4 | APP 1 –service type 1 –slice 1 APP 2 –service type 1 –slice 1 | Scenario 8 | APP 1 | –service type 1 –slice 1 |

As specified in TS 23.503[2], one APP will select one single slice according to URSP. The QoS differentiation in a PDU session is guaranteed by multi QoS flows with different QFI.

If the UE determines that there is more than one existing PDU Session which matches (e.g. the selected Route Selection Descriptor only specifies the Network Slice Selection, while there are multiple existing PDU Sessions matching the Network Slice Selection with different DNNs), it is up to UE implementation to select one of them to use.

If none of the existing PDU Sessions matches, the UE tries to establish a new PDU Session using the values specified by the selected Route Selection Descriptor.

Observation 1: one APP will select one single slice according to NSSP in URSP.

Therefore, the scenario 5 and 7 can be low priority to be considered as they are not practical scenarios.

Proposal 1: the scenario 5 and 7 can be low priority as they are not practical scenarios.

- 2.2 mechanism

As specified in TS 23.502[3], the key procedure of PDU Session Establishment is shown in Figure 1.

Figure 1 UE-requested PDU Session Establishment

The PDU SESSION RESOURCE SETUP REQUEST Message is shown in Table 2[4] and the PDU SESSION ESTABLISHMENT ACCEPT Message is shown in Table 3 [5].

As shown in the tables, both UE and NG-RAN node can associate PDU session ID to S-NSSAI. However, note that the PDU SESSION ESTABLISHMENT ACCEPT Message is a NAS message, so only NAS can make it for UE. Based on specific implementation (including information exchange between AS and NAS), UE may perform the mapping of QoE measurement and slice identification.

Table 2: PDU SESSION RESOURCE SETUP REQUEST Message

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| RAN Paging Priority | O |  | 9.3.3.15 |  | YES | ignore |
| NAS-PDU | O |  | 9.3.3.4 |  | YES | reject |
| PDU Session Resource Setup Request List |  | 1 |  |  | YES | reject |
| >PDU Session Resource Setup Request Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>PDU Session NAS-PDU | O |  | NAS-PDU 9.3.3.4 |  | - |  |
| >>S-NSSAI | M |  | 9.3.1.24 |  | - |  |
| >>PDU Session Resource Setup Request Transfer | M |  | OCTET STRING | Containing the PDU Session Resource Setup Request Transfer IE specified in subclause 9.3.4.1. | - |  |
| UE Aggregate Maximum Bit Rate | O |  | 9.3.1.58 |  | YES | ignore |

Table 3: PDU SESSION ESTABLISHMENT ACCEPT Message

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION ESTABLISHMENT ACCEPT message identity | Message type 9.7 | M | V | 1 |
|  | Selected PDU session type | PDU session type 9.11.4.11 | M | V | 1/2 |
|  | Selected SSC mode | SSC mode 9.11.4.16 | M | V | 1/2 |
|  | Authorized QoS rules | QoS rules 9.11.4.13 | M | LV-E | 6-65538 |
|  | Session AMBR | Session-AMBR 9.11.4.14 | M | LV | 7 |
| 59 | 5GSM cause | 5GSM cause 9.11.4.2 | O | TV | 2 |
| 29 | PDU address | PDU address 9.11.4.10 | O | TLV | 7-31 |
| 56 | RQ timer value | GPRS timer 9.11.2.3 | O | TV | 2 |
| 22 | S-NSSAI | S-NSSAI 9.11.2.8 | O | TLV | 3-10 |
| 8- | Always-on PDU session indication | Always-on PDU session indication 9.11.4.3 | O | TV | 1 |
| 75 | Mapped EPS bearer contexts | Mapped EPS bearer contexts 9.11.4.8 | O | TLV-E | 7-65538 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 7-1503 |
| 79 | Authorized QoS flow descriptions | QoS flow descriptions 9.11.4.12 | O | TLV-E | 6-65538 |

Observation 2: both UE and NG-RAN node can associate PDU session ID with S-NSSAI.

The mechanism to map the QoE report and the slice identification was discussed at last RAN3 meeting[1] and the alternatives are as follows.

The feasibility of following alternatives is FFS and additional alternatives should not be precluded.

-  Solution 1: Mapping performed by NG-RAN node

-  Solution 2: Mapping performed by UE

Mapping performed by NG-RAN node

1.	OAM/CN transmit the QoE measurement configuration to NG-RAN node, including Slice Scope.

2.	NG-RAN node can map the Slice Scope to the ongoing PDU session list and send the QoE measurement configuration with the PDU session list to UE.

3.	UE receives the QoE measurement configuration and sends it to the corresponding application layer according to the PDU session list.

4.	UE sends the QoE report with PDU session ID to NG-RAN node.

5.	NG-RAN node can remap the PDU session ID back to slice ID and attach it in the QoE report.

6.	NG-RAN node forwards the QoE report with slice ID to the MCE.

Mapping performed by UE

1.	OAM/CN transmits the QoE measurement configuration to NG-RAN node, including Slice Scope.

2.	NG-RAN node checks the Slice Scope with all of the ongoing PDU sessions, and sends QoE measurement configuration to UE with qualified PDU session, including Slice Scope.

3.	UE receives the QoE measurement configuration and sends it to the corresponding application layer according to the Slice Scope.

4.	UE sends the QoE report with slice ID to NG-RAN node.

5.	NG-RAN node forwards the QoE report with slice ID to the MCE.

According to the above analysis, both of the solutions can work and the key differences between them include:

- Which node to perform the mapping of QoE measurement and slice identification.

- Whether the Slice Scope should be included in the configuration from NG-RAN node to UE AS.

- Where to place Slice Identification in report.

Figure 2-a per slice QoE measurement procedure with mapping performed by NG-RAN node

The procedure with mapping performed by UE is shown in Figure 2-a.

To avoid introducing extra information exchange in UE AS and NAS, the Slice Scope is unnecessary to include in the configuration from NG-RAN node to UE AS and the PDU session ID is included instead.

Considering that there is no application layer in the NG-RAN node, from a protocol integrity perspective, NG-RAN node is not permitted to add the Slice identification in QoE Report container. Therefore, Slice identification shall be placed outside the QoE Report container.

Proposal 2: for the solution with mapping performed by NG-RAN node, (a) Slice Scope is unnecessary to include in the configuration from NG-RAN node to UE AS and the PDU session ID is included instead, (b) Slice Identification shall be placed outside the QoE Report container.

Figure 2-b per slice QoE measurement procedure with mapping performed by UE

The procedure with mapping performed by UE is shown in Figure 2-b.

As UE AS needs to send the configuration to corresponding application layer according, the Slice Scope should be placed outside the QMC Config container for UE AS to do the correlation.

In the QMC activation and reporting in LTE solution, the QoE reference IE is used to specify the network request session and is included in QoE Report container. Similarly, Slice Identification is used to specify the network request slice and should be included in QoE Report container as well.

Proposal 3: for the solution with mapping performed by UE, (a) Slice Scope included in QoE measurement configuration should be placed outside the QMC Config container, (b) Slice Identification should be included in QoE Report container to specify the network request slice.

As it’s still during SI stage, additional alternatives should not be precluded.

Proposal 4: both of the alternative solutions can work and can be converged at WI stage, meanwhile, additional alternatives should not be precluded.

Proposal 5: capture the TP for TR 38.890[6] on per slice QoE measurement in attached Annex.

- Conclusion

In this paper, we discuss the mechanisms for per slice QoE measurement, the following are observations and proposals:

Observation 1: one APP will select one single slice according to NSSP in URSP.

Proposal 1: the scenario 5 and 7 can be low priority as they are not practical scenarios.

Proposal 2: for the solution with mapping performed by NG-RAN node, (a) Slice Scope is unnecessary to include in the configuration from NG-RAN node to UE AS and the PDU session ID is included instead, (b) Slice Identification shall be placed outside the QoE Report container.

Proposal 3: for the solution with mapping performed by UE, (a) Slice Scope included in QoE measurement configuration should be placed outside the QMC Config container, (b) Slice Identification should be included in QoE Report container to specify the network request slice.

Proposal 4: both of the alternative solutions can work and can be converged at WI stage, meanwhile, additional alternatives should not be precluded.

Proposal 5: capture the TP for TR 38.890[6] on per slice QoE measurement in attached Annex.

# 4 References

- R3-206900: " Summary of Offline Discussion on per slice QoE measurement ".

- 3GPP TS 23.503: " 3GPP TS 23.502: " Procedures for the 5G System (5GS); Stage 2".

- 3GPP TS 23.502: " Procedures for the 5G System (5GS); Stage 2".

- 3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

- 3GPP TS 24.501: " Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

- 3GPP TR 38.890 v0.2.0: " Study on NR QoE management and optimizations for diverse services ".

# 5. Annex- TPs to be captured in TR

### 6.9.2 	Solution

#### 6.9.2.1 	Configuration

The Slice Scope information should be included in the QoE configuration from CN/OAM to NG-RAN node.

Editor’s NOTE: The Slice Scope in the QoE configuration is FFS in WI stage.

#### 6.9.2.2 	Mapping

Editor’s NOTE: The mechanism to support mapping of QoE report and slice identification may converge in WI stage.

The procedure of mapping QoE report and slice performed by NG-RAN node is as follows:

1.	OAM/CN transmit the QoE measurement configuration to NG-RAN node, including Slice Scope.

2.	NG-RAN node can map the Slice Scope to the ongoing PDU session list and send the QoE measurement configuration with the PDU session list to UE.

3.	UE receives the QoE measurement configuration and sends it to the corresponding application layer according to the PDU session list.

4.	UE sends the QoE report with PDU session ID to NG-RAN node.

5.	NG-RAN node can remap the PDU session ID back to slice ID and attach it in the QoE report.

6.	NG-RAN node forwards the QoE report with slice ID to the MCE.

The procedure of mapping QoE report and slice performed by UE is as follows:

1.	OAM/CN transmits the QoE measurement configuration to NG-RAN node, including Slice Scope.

2.	NG-RAN node checks the Slice Scope with all of the ongoing PDU sessions, and sends QoE measurement configuration to UE with qualified PDU sessions, including Slice Scope.

3.	UE receives the QoE measurement configuration and sends it to the corresponding application layer according to the Slice Scope.

4.	UE sends the QoE report(s) including S-NSSAI to NG-RAN node.

5.	NG-RAN node forwards the QoE report including S-NSSAI to the MCE.

NOTE: Additional alternatives should not be precluded.

#### 6.9.2.3 	Reporting

The slice identification should be included in the QoE report.

Editor’s NOTE: The slice identification in the QoE report is FFS in WI stage.