3GPP TSG-RAN WG3 Meeting #111-e	R3-210865

E-meeting, 25 Jan – 5 Feb 2021

Title: 	General aspects and principles of the interface between CP and UP for eNB and ng-eNB

Source: 	Huawei

Agenda item:	23.2.1

Document Type:	pCR

1. Introduction

In RAN#86, a new WI “Enhanced eNB(s) architecture evolution” was approved [1]. The objectives of the WI are as follows:

The objective of this WI is to specify the CP-UP separation and the interface between the CP and UP for eNB and ng-eNB. The detailed objectives of the work item are: - Specification of the CP and UP interface (e.g. E1’) for eNB and ng-eNB - general principles, functions and procedures; - signaling transport - to transport the control information between CP and UP for eNB and ng-eNB based on the use of an appropriate TNL e.g. the SCTP/IP protocol stack; - application protocol - including stage-3 specification of elementary procedures and messages. NOTE: It should be clarified during the normative phase; - how to specify architecture impact and definition of the logical nodes on E-UTRAN case - whether to use E1’ or E1 for E-UTRAN and/or NG-RAN case - whether to reuse TS 36.425 or TS 38.425 for U-plane for E-UTRAN case - If deemed feasible, adoption of the E1 interface for the CP-UP split in eNBs and ng-eNBs

In this document, we intend to discuss the general aspects and principles of the interface between CP and UP for eNB and ng-eNB, and provide the related TP for stage 2 descriptions.

2. Discussion

- To take existing E1 as baseline

Similar as what we did for W1, we think we should take E1 as baseline for CP and UP for eNB and ng-eNB, since most of the work actually was already fully investigated during normative phase for E1, and we expect that major work for this WI should be a copy/paste from E1.

Proposal 1: RAN3 agree to take existing E1 as base line.

- To introduce a new protocol set or reuse existing protocol set

The WI scope indicates that the CP-UP separation interface for both eNB and ng-eNB will be specified. Maybe the first question is, whether there is a need to introduce a new set of protocols or just reuse the existing E1 protocol set.

Here we see both pros and cons. On one hand, to introduce a new protocol set is a straight way, and management wise easier to track the evolution of this feature; on the other hand, if we look into E1 spec, we could see that existing E1 covers the user plane establishment for both eNB (EN-DC case) and ng-eNB case (NG-RAN case), i.e. the existing E1 was already well structured and maybe just some small additions are needed to incorporate eNB and ng-eNB cases. Thus, RAN3 need to discuss whether to introduce a new protocol set or reuse existing protocol set.

Proposal 2: RAN3 need to decide whether to introduce a new protocol set or reuse existing protocol set.

- Terminology and principles

Since we have definitions for gNB-CU-CP and gNB-CU-UP, similarly we should introduce definitions for ng-eNB-CU-CP, ng-eNB-CU-UP, eNB-CP and eNB-UP, the following show the examples (assuming the existing E1 interface is reused):

ng-eNB-CU-Control Plane (ng-eNB-CU-CP): a logical node hosting the RRC and the control plane part of the PDCP protocol of the ng-eNB-CU for an ng-eNB. The ng-eNB-CU-CP terminates the E1 interface connected with the ng-eNB-CU-UP and the W1-C interface connected with the ng-eNB-DU.

eNB-Control Plane (eNB-CP): a logical node hosting the RRC/MAC/PHY and the control plane part of the PDCP protocol for an eNB. The eNB-CP terminates the E1 interface connected with the eNB-UP.

- Here as we could see that, we need to take the fact that there is no CU/DU split for legacy eNB, so we just have eNB-CP and eNB-UP. Hence the RAN3 need to discuss and agree related terminologies.

- Proposal 3: RAN3 discuss and agree on the introduction of new logical node name: ng-eNB-CU-CP, ng-eNB-CU-UP, eNB-CP and eNB-UP.

- Another scenario which has not been address in E1 is that, there is no CU/DU split for a base station but there is CP/UP separation for a base station, hence we will see a potential scenario where there are two logical RAN node, one is responsible for user plane handling, the other is for the rest functions, i.e. SDAP/PDCP for signalling bearer, RRC/RLC/MAC/PHY, for ng-eNB case, we may see ng-eNB-CP and ng-eNB-UP, this should be addressed, either we could ignore this, or we introduce new defintions.

- Proposal 3bis: RAN3 discuss whether to introduce new logical node name as ng-eNB-CP and ng-eNB-UP.

4. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1:	RAN3 agree to take existing E1 as base line.

Proposal 2:	RAN3 need to decide whether to introduce a new protocol set or reuse existing protocol set.

Proposal 3:	RAN3 discuss and agree on the introduction of new logical node name: ng-eNB-CU-CP, ng-eNB-CU-UP, eNB-CP and eNB-UP.

Proposal 3bis:	RAN3 discuss whether to introduce new logical node name as ng-eNB-CP and ng-eNB-UP.

5. Reference

- RP-193181, “New WID: Enhanced eNB(s) architecture evolution”, China Unicom.