3GPP TSG-RAN WG3 Meeting #111-e	R3-210867

E-meeting, 25 Jan – 5 Feb 2021

Title: 	Bearer context management function and procedures of the interface between CP and UP for eNB and ng-eNB

Source: 	Huawei

Agenda item:	23.2.3

Document Type:	pCR

1. Introduction

In RAN#86, a new WI “Enhanced eNB(s) architecture evolution” was approved [1]. The objectives of the WI are as follows:

The objective of this WI is to specify the CP-UP separation and the interface between the CP and UP for eNB and ng-eNB. The detailed objectives of the work item are: - Specification of the CP and UP interface (e.g. E1’) for eNB and ng-eNB - general principles, functions and procedures; - signaling transport - to transport the control information between CP and UP for eNB and ng-eNB based on the use of an appropriate TNL e.g. the SCTP/IP protocol stack; - application protocol - including stage-3 specification of elementary procedures and messages. NOTE: It should be clarified during the normative phase; - how to specify architecture impact and definition of the logical nodes on E-UTRAN case - whether to use E1’ or E1 for E-UTRAN and/or NG-RAN case - whether to reuse TS 36.425 or TS 38.425 for U-plane for E-UTRAN case - If deemed feasible, adoption of the E1 interface for the CP-UP split in eNBs and ng-eNBs

In this document, we intend to discuss the bearer context management function and procedures of the interface between CP and UP for eNB and ng-eNB, and provide the related TP for stage 2 descriptions.

2. Discussion

The following functions and procedures shall be supported by the CP-UP separation interface for ng-eNB/eNB.

-	Bearer context setup

The bearer context setup function allows to establish a bearer context in the CU-UP.

It is initiated by the CU-CP and accepted or rejected by the CU-UP based on admission control criteria, e.g., resource not available.

-	Bearer context release

The bearer context release function allows to release the bearer context previously established in the CU-UP.

It is used by the CU-CP to command the release of an UE-associated logical E1’ connection.

It is also used by the CU-UP to request the CU-CP to release an UE-associated logical E1’ connection.

-	Bearer context modification

The bearer context modification function allows to modify a bearer context in the CU-UP.

It is used by the CU-CP modify a bearer context in the CU-UP and accepted or rejected by the CU-UP.

It is also used by the CU-UP to modify a bearer context and inform the CU-CP.

- Proposal 1: The new interface shall support bearer context management function, which includes the Bearer Context Setup function, the Bearer Context Release function, and the Bearer Context Modification function.

In addition, it is essential to provide the following information during the bearer context setup or modification procedure:

-	QoS flow to DRB mapping configuration

The CU-CP decides the flow-to-DRB mapping and provides the generated SDAP and PDCP configuration to the CU-UP.

-	S-NSSAI

For each PDU session resource to be setup or modified, the CU-CP shall provide the S-NSSAI to the CU-UP in the bearer context setup procedure, and the CU-CP may provide the S-NSSAI to the CU-UP in the bearer context modification procedure.

- Proposal 2: The bearer context management function of the new interface should provide the QoS flow to DRB mapping configuration and the S-NSSAI.

4. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1:	The new interface shall support bearer context management function includes the Bearer Context Setup function, the Bearer Context Release function, and the Bearer Context Modification function.

Proposal 2:	The bearer context management of the new interface function should provide the QoS flow to DRB mapping configuration and the S-NSSAI.

5. Reference

- RP-193181, “New WID: Enhanced eNB(s) architecture evolution”, China Unicom.