3GPP TSG-RAN WG3 Meeting #111-e	R3-210868

E-meeting, 25 Jan – 5 Feb 2021

Title: 	Interface management function and procedures of the interface between CP and UP for eNB and ng-eNB

Source: 	Huawei

Agenda item:	23.2.3

Document Type:	pCR

1. Introduction

In RAN#86, a new WI “Enhanced eNB(s) architecture evolution” was approved [1]. The objectives of the WI are as follows:

The objective of this WI is to specify the CP-UP separation and the interface between the CP and UP for eNB and ng-eNB. The detailed objectives of the work item are: - Specification of the CP and UP interface (e.g. E1’) for eNB and ng-eNB - general principles, functions and procedures; - signaling transport - to transport the control information between CP and UP for eNB and ng-eNB based on the use of an appropriate TNL e.g. the SCTP/IP protocol stack; - application protocol - including stage-3 specification of elementary procedures and messages. NOTE: It should be clarified during the normative phase; - how to specify architecture impact and definition of the logical nodes on E-UTRAN case - whether to use E1’ or E1 for E-UTRAN and/or NG-RAN case - whether to reuse TS 36.425 or TS 38.425 for U-plane for E-UTRAN case - If deemed feasible, adoption of the E1 interface for the CP-UP split in eNBs and ng-eNBs

In this document, we intend to discuss the interface management function and procedures of the interface between CP and UP for eNB and ng-eNB, and provide the related TP for stage 2 descriptions and interface protocol architecture.

2. Discussion

Similar as for E1 interface, we think the following functions and procedures shall be supported by the CP-UP separation interface for ng-eNB/eNB.

-	Reset procedure

The reset function is used to initialize the peer entity after node setup and after a failure event occurred.

The reset function can be used by both the CU-CP and the CU-UP.

-	Error Indication procedure

The error indication function is used by the CU-UP to indicate to the CU-CP that an error has occurred. It can also be used by the CU-CP to indicate to the CU-UP that an error has occurred.

-	Interface Setup procedure

The interface setup function allows to exchange application level data needed for the CU-CP and CU-UP to interoperate correctly on the separation interface.

When the CU-CP is deployed distributed and the CU-UP is centralized (e.g., a single physical centralized UP entity implements several logical CU-UPs), it is reasonable to assume that the CU-UP resources are available first, and then the CU-CPs are deployed gradually. In this scenario, the CU-CP initiates the interface setup procedure.

In contrast, when the CU-UP is deployed distributed and the CU-CP is centralized, it is reasonable that the CU-UP initiates the interface setup procedure.

Therefore, it is proposed that the interface setup can be initiated by both the CU-CP and the CU-UP.

-	Configuration Update procedure

The configuration update function allows to update application level configuration data needed between the CU-CP and the CU-UP to interoperate correctly on the separation interface.

Like the interface setup function, the configuration update can be initiated by both the CU-CP and the CU-UP.

-	Status Indication procedure

The CU-UP status indication function allows to inform the status (e.g. overloaded, non-overloaded) over the separation interface.

- Protocol: The new interface management function includes the Reset function, the Error Indication function, the Interface Setup function, the Configuration Update function, and the Status Indication function.

In addition, the CU-UP may be operated by different operator or by different slices. Therefore, it is essential that the CU-UP should provide the following information to the CU-CP during the interface setup procedure or the configuration update procedure for future purposes:

-	ECGI

It is used by the CU-CP to relocate the CU-UP when the UE leaves the cells served by a particular CU-UP.

-	S-NSSAI

It is used by the CU-CP to configure a slice specific CU-UP.

-	PLMN-ID

It is used by the CU-CP to configure a PLMN specific CU-UP.

- Proposal 2: During the Interface Setup procedure or the Configuration Update procedure, the CU-UP should provide the supported ECGI, S-NSSAI and PLMN-ID.

4. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1:	The new interface management function includes the Reset function, the Error Indication function, the Interface Setup function, the Configuration Update function, and the Status Indication function.

Proposal 2:	During the Interface Setup procedure or the Configuration Update procedure, the CU-UP should provide the supported ECGI, S-NSSAI and PLMN-ID.

5. Reference

- RP-193181, “New WID: Enhanced eNB(s) architecture evolution”, China Unicom.