3GPP TSG-RAN WG3 #111-e							                                                R3-210901

25 January – 4 February 2021

Online

Agenda item:	15.2

Source:	CMCC

Title:	TP to 38.890 for NR QoE management

Document for:	Discussion

- Introduction

This contribution provides the text proposal to correlate RAN related measurement/information for NR QoE management in TR 38.890.

# 2	Text proposal for TR 38.890

--------------------------------------------------Start of the first change---------------------------------------------------

# 6	Potential NR QoE solutions and procedures

Editor’s NOTE: Description of potential NR QoE solutions and procedures, including but not limited to reuse Trace or MDT Functionality/Framework.

## 6.1 	Signalling-based procedures(FFS)

### 6.1.1	Signalling-based activation procedures

The procedure is used for activating the QoE measurement configured by OAM and triggered by CN shown in figure 6.1.1-1.

The CN initiates the activation of the QoE measurement configured by OAM, and sends the QoE measurement configuration to the NG-RAN node. UE AS layer sends the QoE measurement configuration to UE application layer.

UE application layer generates the QoE report and sends to the UE AS layer. UE AS layer correlates the radio related measurement/information, if any, and sends the QoE report with correlated radio related measurement/information to NG-RAN node. Then the NG-RAN node transmits the QoE report to the final destination configured (e.g. TCE/MCE).

Editor’s NOTE: the QoE report may also be visible at the NG-RAN node.

Management-based QoE measurement shall not overwrite a corresponding signalling-based existing configuration. (FFS)

Figure 6.1.1-1 Signalling-based NR QoE activation procedure

--------------------------------------------------End of the first change----------------------------------------------------

------------------------------------------------Start of the second change-------------------------------------------------

## 6.2 	Management-based procedures (FFS)

### 6.2.1	Management-based activation procedures

Editor’s NOTE: The section concerns activation of measurement collection job. It needs to be further checked if specific UEs can be targeted by the management based QoE measurement configuration.

The procedure is used for activating the QoE measurement configured and triggered by OAM shown in figure 6.2.1-1.

The OAM sends the QoE measurement configuration to NG-RAN node. NG-RAN finds multiple qualified UEs that meet the criteria (e.g. area scope, application layer capability, service type, etc.) or a single specific UE. NG-RAN node sends the QoE measurement configuration to the AS layer of the specific UE or each qualified UE. UE AS layer sends the QoE measurement configuration to UE application layer. When a session starts, the application layer in UE checks the criteria (e.g. cell list, service type, etc.), if the criteria are met, start QoE measurement and reporting.

FFS whether multiple QoE measurements could be configured for a UE at the same time.

UE application layer sends the QoE report to the UE AS layer. UE AS layer correlates the radio related measurement/information, if any, and sends the QoE report with correlated radio related measurement/information to NG-RAN node. Then the NG-RAN node transmits the QoE report to the final destination configured (e.g. the MCE).

Editor’s NOTE: the QoE report may also be visible at the NG-RAN node. (to be confirmed)

Management based QoE configuration should not override signalling based QoE configuration.(FFS)

Regarding Management-based configuration towards an individual specific UE, the following needs to be considered:

- The OAM is currently not capable of sending the QoE measurement configuration for a specific UE towards its serving gNB when the targeted UE becomes RRC connected. This is because OAM is not informed about when a specific UEs served by a certain gNB become RRC connected.

- There is currently no UE identifier available in OAM which can be used to identify an UE connected to a certain gNB.

- Although it is acknowledged that QoE measurement configuration towards an individual specific UE is already possible using signalling based configuration (via the CN), it should be noted that OAM can issue per-user policies for which the feedback in terms of user experience is required.

Figure 6.2.1-1 Management-based NR QoE activation procedure

------------------------------------------------End of the second change---------------------------------------------------