3GPP TSG-RAN WG3 #111-e	R3-210916

Jan 25nd – Feb 4nd, 2021 e-Meeting

Title:	TP to 38.300 BLCR for mobility between MBS supporting nodes

Source:	CMCC

Agenda item:	22.3.1

Document for:	Discussion

Introduction

This contribution provides message exchange between MBS supporting nodes in TS 38.300.

# 2 Text proposal for TS 38.300

--------------------------------------------------Start of the change---------------------------------------------------

- 16.x.5	Mobility

Editor’s Note: Mobility aspects to be covered here.

- 16.x.5.1	General

Mobility principles builds on existing functionality including functions described in section 9.2.

- 16.x.5.2	Multicast Mobility from MBS supporting cell to MBS supporting cell

During handover preparation phase, the source NG-RAN node transfers to the target NG-RAN node in the UE context information about the MBS sessions the UE has joined. For each Multicast session with ongoing user data transmission for which no MBS Session Resources exist at the target NG-RAN node, the target NG-RAN node triggers the setup of MBS user plane resources towards the 5GC. Which procedures to use is FFS.

The MBS data transmission/buffer status information is exchanged between source NG-RAN node and target NG-RAN node, which procedures to use is FFS.

During handover execution, the MBS configuration decided at target NG-RAN node is sent to the UE via the source NG-RAN node within an RRC container (FFS) as specified in TS 38.331 [12].

- 16.x.5.3	Multicast Mobility from/to MBS supporting cell to/from MBS non-supporting cell

Editor’s Note: FFS. Based on SA2 input. RAN impacts to be covered here.

- 16.x.5.4	Broadcast Mobility (FFS)

Editor’s Note: intends to cover information related to broadcast exchanged during mobility. FFS.

--------------------------------------------------End of the change-----------------------------------------------------