3GPP TSG-RAN WG3 Meeting #111-e	R3-211358 was R3-211340

E-meeting, 25 Jan – 5 Feb 2021

Title: 	TP to 38.890 on open issues of QoE configuration and reporting

Source: 	Huawei, Samsung, Ericsson

Agenda item:	15.2

Document Type:	other

1. Introduction

This TP tries to reflect the agreements achieve during online and offline discussions:

- Agree to support XR as new service type for QoE measurement

- Agree to supports for multiple QoE measurements for the same UE, detailed mechanisms on whether and how to deactivate one of them are up to RAN2

- In case of RAN overload, RAN could take some measures, detailed mechanisms are up to RAN2

- General rule for QoE measurement and its corresponding radio assisted measurement

  - The two might be configured together and reported together

Annex – TP

5	5G services, QoE metrics and UE KPI information

NR QoE supports the Application Layer Measurement Collection functionality.

This functionality enables the collection of application layer measurements from the UE. The supported service types include:

- Streaming services [3];

- MTSI services [4];

- VR [5];

- MBMS [6];

- XR

Support for additional service types is not precluded.

In addition to the QoE metrics, the radio related measurements and information to assist the NR QoE management functionality are considered.

////////////////////////////////////////unchanged omitted///////////////////////////////////////

6.2.1	Management-based activation procedures

The procedure is used for activating the QoE measurement configured and triggered by OAM shown in figure 6.2.1-1.

The OAM sends the QoE measurement configuration to NG-RAN node. NG-RAN finds multiple qualified UEs that meet the criteria (e.g. area scope, application layer capability, service type, etc.) or a single specific UE. NG-RAN node sends the QoE measurement configuration to the AS layer of the specific UE or each qualified UE. UE AS layer sends the QoE measurement configuration to UE application layer. When a session starts, the application layer in UE checks the criteria (e.g. cell list, service type, etc.), if the criteria are met, start QoE measurement and reporting.

Multiple QoE measurements could be configured for a UE at the same time. In case of multiple QoE measurements are configured at a UE, it can be discussed in the normative phase whether one measurement is configured per service type at the UE.

UE application layer sends the QoE report to the UE AS layer. UE AS layer sends the QoE report to NG-RAN node. Then the NG-RAN node transmits the QoE report to the final destination configured (e.g. the MCE).

////////////////////////////////////////unchanged omitted///////////////////////////////////////

6.5 	QoE measurement handling at RAN overload

In case of RAN overload in standalone connectivity, RAN can stop new QoE measurement configurations, release existing QoE measurement configurations and pause QoE measurement reporting. In case of RAN overload in dual connectivity, the details might be discussed in normative phase.

////////////////////////////////////////unchanged omitted///////////////////////////////////////

6.8 	Radio-related measurements and information for QoE

In order for the network to further evaluate and improve the QoE, RAN could also trigger radio-related measurements towards a certain UE, based on the QoE measurement configuration received from the OAM. For triggering the measurements an existing mechanism, e.g. MDT procedure can be used.

The radio-related QoE measurements are reported for all types of supported services, and they include MDT-like measurements and, potentially, additional measurements related to the radio interface. If new radio-related measurements, with respect to what is currently specified in MDT, are required for NR QoE management, these additional radio-related QoE measurements will be specified as a part of MDT measurements. Application-related QoE measurements are only collected when the application session is ongoing. If these radio-related measurements are used for assisting application-related QoE measurements, it is beneficial and efficient if measurement collection and reporting can start at the same time. If configured together e.g. using same trace reference and time aligned, e.g. based on time stamps, correlation of the results may be done by post processing.

Besides radio-related measurement results, radio-related information may also be reported. Radio-related information may be reported even when radio-related measurements are not triggered over the radio.

Both of the radio-related measurement results and radio-related information, if reported, should be aligned and correlated with the QoE report, using e.g. trace ID.