3GPP TSG-RAN3 Meeting #112-e	R3-211512

Electronic meeting, 17 - 27 May 2021

3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2104374

Electronic meeting, 2021-04-12 – 2021-04-20

Title:	LS on introducing extended DRX for RedCap UEs

Response to:	-

Release:	Release 17

Work Item:	NR_redcap-Core

Source:	RAN2

To:	SA2, CT1

Cc:	RAN3

Contact Person:

#### Name:	Tuomas Tirronen

E-mail Address:	tuomas dot tirronen at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN2 has started work on the support of reduced capability (RedCap) NR devices, where the related study item has been concluded in TR 38.875, v17.0.0 and work has been started according to WID in RP-210918. The WID has the following objectives on introduction of extended DRX:

- Specify support for the following Extended DRX enhancements for RedCap UEs [RAN2, RAN3, RAN4]: - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10.24 s, without using PTW and PH, and with common design (e.g. common set of eDRX values) between RRC Inactive and Idle - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10485.76 s; the details of mechanisms and feasibility regarding maximum length of the extended DRX cycles for RRC Inactive and Idle need to be checked by SA2, CT1 and/or RAN4. - RAN2 to decide which Node(s) configure eDRX in RRC_Idle and RRC_Inactive.

RAN2 has discussed support for extended DRX for RedCap UEs during RAN2#113bis-e and has reached the following agreements and would like to ask SA2 and CT1 for feedback, if any:

- RAN decides and configures eDRX via RRC for RRC_INACTIVE (FFS on the need and details of coordination with the CN) - At least for eDRX cycle, the configurations of the eDRX for RRC_IDLE and RRC_INACTIVE can be different (FFS for PTW, e.g. length and starting point, when eDRX cycles are longer than 10.24s) - RAN2 assumes that CN provides necessary assistance information on eDRX config. for RRC_IDLE to RAN (e.g. reusing eDRX config. defined in “CN Assistance Information for RRC INACTIVE IE” for E-UTRA/5GC). - eDRX feature, including the related parameters (i.e. PH, PTW. H-SFN) and corresponding paging operation defined for E-UTRA/5GC is used as baseline to enable eDRX >10.24sec for both RRC_IDLE and RRC_INACTIVE in NR/5GC.

RAN2 would like to ask whether it is feasible from SA2 and CT1 perspective to introduce extended DRX up to 10485.76 s in RRC_IDLE and RRC_INACTIVE and if feasible, to specify the necessary support.

RAN2 would like SA2 and CT1 to take the above aspects into consideration in their work and further consult with RAN2 if needed.

2. Actions:

To SA2 and CT1 groups:

ACTION: 	1) RAN2 respectfully requests SA2 and CT1 to evaluate whether it is feasible to specify extended DRX up to 10485.76 s for RRC_IDLE and RRC_INACTIVE and if found feasible, specify the necessary support.

2) RAN2 respectfully requests SA2 and CT1 to take above information into account and provide feedback, if any.

3. Date of Next RAN2 Meetings:

3GPP RAN2#114-e		19th – 27th May 2021				Electronic Meeting

3GPP RAN2#115-e		16th – 27th August 2021			Electronic Meeting