3GPP TSG-RAN WG3#112-e                                             	                     R3-211667

E-Meeting, 17 – 27 May 2021

Agenda item:	9.3.8.1

Source:	Nokia, Nokia Shanghai Bell

Title: 	Correction of NG handover

Document for:	Approval

# Discussion

NG handover

For NG handover we have the following IE in the Source NG-RAN to Target NG-RAN container IE in section 9.3.1.29:

| >>QoS Flow Information List |  | 1 |  |  | - |  |
| --- | --- | --- | --- | --- | --- | --- |
| >>>QoS Flow Information Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>>>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>>>DL Forwarding | O |  | 9.3.1.33 |  | - |  |
| >>>>UL Forwarding | O |  | 9.3.1.118 |  | YES | reject |
| >>DRBs to QoS Flows Mapping List | O |  | 9.3.1.34 |  | - |  |

And usage in section 8.4.1.2:

If the DL Forwarding IE is included for a given QoS flow in the PDU Session Resource Information Item IE within the Source NG-RAN node to Target NG-RAN node Transparent Container IE of the HANDOVER REQUIRED message and it is set to "DL forwarding proposed", it indicates that the source NG-RAN node proposes forwarding of downlink data for that QoS flow.

The forwarding decision is per QoS flow in the above statement.

However, when the flow-DRB mapping is sent another text states:

If the DRBs to QoS Flows Mapping List IE is included in the PDU Session Resource Information Item IE within the Source NG-RAN node to Target NG-RAN node Transparent Container IE of the HANDOVER REQUIRED message, it implicitly indicates that the source NG-RAN node proposes forwarding of downlink data for those DRBs

The sentence highlighted in yellow associate the sending of flow-DRB mapping to a forwarding proposal.

Observation 1: the highlighted sentence for NG handover introduces a dependency between sending the flow-DRB mapping and data forwarding proposal of associated QoS flows.

However, the DRBs to QoS Flows Mapping List IE includes itself a list of QoS flows as described in section 9.3.1.34:

Observation 2: The sentence highlighted in yellow for NG handovers introduces a redundant requirement for the data forwarding of a QoS flow.

For example, if:

- QFI1 is included without the DL Forwarding IE present (meaning data forwarding not proposed for QFI1)

- And QFI1 is included in the DRBs to QoS Flows Mapping List IE (meaning data forwarding proposed for QFI1)

Which one of these two IEs prevail?

Observation 3: the redundancy can lead to inconsistency and it is not clear which IE prevails.

Considering the issues reported in observations 1, 2 and 3, we propose to clarify the encoding of these two information elements.

Proposal 1: clarify the relationship between the DL Forwarding IE and the DRBs to QoS Flows Mapping List IE with the CR in [2].

# Conclusion and Proposal

This paper has described that NG handover has a statement including redundant and possibly contradicting data forwarding requirements for a same QoS flow. It makes the following proposal:

Proposal 1: clarify the relationship between the DL Forwarding IE and the DRBs to QoS Flows Mapping List IE with the CR in [2].

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- R3-211668, Correction of NG handover