3GPP TSG-RAN WG3 Meeting #112-e	R3-211898

17-27 May 2021

Agenda item:	20.2.5

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for BL CR for TS 38.413) Country Specific Routing for an RRC CONNECTED UE

Document for:	Discussion & decision

# 1	Introduction

This contribution analyzes the country specific routing for an RRC CONNECTED UE.

# 2	Discussion

SA2 TS23.502 ([1]) agreed following:

The Inter NG-RAN node N2 based handover procedure specified in clause 4.9.1.3 may also be used for intra-NG-RAN node handover.

NOTE: One use case for intra-NG-RAN handover to be performed by the Inter NG-RAN node N2 based handover procedure is when an NG-RAN node serves a satellite access system that covers more than one country. In such a situation, the UE might move from a "cell" in one country into a "cell" in another country, and the NG-RAN node may need to cause the AMF to change to an AMF serving the UE's new country.

An example for NTN is shown in below figure. A NTN cell (e.g. a GEO cell) may cover a large geographical area that may span more than one country. The operator from each country may use its own CN to serve its UEs. The neighboring operator may have roaming agreement to support a UE roams in a different country, but it is still in the same NTN cell. In this case, a NTN cell/gNB is shared among the operators. SA2 requirement mandates the gNB selects a correct AMF to serve the UE, e.g. based on the country that the UE is located. When a RRC CONNECTED UE moves across the country border (shown in Fig.1), the serving gNB initiates a N2-based Handover to change the AMF for the UE.

Figure 1: NTN deployment example when a shared gNB connects with different AMFs

In this N2-based handover, it is the same gNB/cell serving as the source gNB/cell and target gNB/cell. An example call flow is shown below (the procedures for CN nodes are skipped).

Figure 2: Example for inter-AMF HO

- Step 0: a RRC CONNECTED UE is in Country A, and it is served by the gNB and the source AMF.

- Step 1: the RRC CONNECTED UE moves to country B.

- Step 2: The UE may provide its location information to the gNB.

It is FFS (and maybe related to RAN2) on how the gNB track the UE’s location, and know the UE has moved into Country B.  In this example, it is assumed that the UE provides its location information to the gNB.

- Step 3: the gNB determines the UE has moved to country B. The gNB may be preconfigured with the country boarder information, in order to determine the country that the UE is located.

- Step 4: the gNB initiates the N2 handover preparation procedure.

- Step 5: source AMF initiates Namf_Communication_CreateUEContext Request to Target AMF.

- Step 6-7: Target AMF initiates the N2 Handover Resource Allocation procedure towards target gNB, which is also the source gNB.

- Step 8: Target AMF initiates Namf_Communication_CreateUEContext Response to Source AMF.

- Step 9: Source AMF sends a N2 Handover Command to source gNB.

- Step 10: source gNB send the HandoverCommand to the UE

- Step 11: The UE executes the Reconfiguration, e.g. reconfigure with the new security key associated with target AMF.

- Step 12: the UE sends the RRCReconfigurationComplete to the gNB.

- Step 13: the UE is still served by the same gNB/cell, but served by target AMF.

Since it is an existing UE served by the same “source” gNB/cell and “target” gNB/cell, there is no need to perform admission control, resource allocation, and radio parameter reconfiguration in the “target” gNB/cell. So it is necessary that “target” gNB shall know the HO is related to an existing UE served by the “source” gNB during the HO resource allocation procedure (Step 6).

Observation 1: during the N2-HO to change the AMF for a UE, the gNB shall know the HO is related to an existing UE.

It may require an identity to be sent from “source” gNB to “target” gNB to identify an existing UE. Similar issue was discussed in inter-system handover to 4G to enable node-internal data forwarding in case of shared en-gNB/gNBs. An UE Context Reference at Source IE was introduced in the Source eNB to Target eNB Transparent Container IE (TS36.413).

#### 9.2.1.144	UE Context Reference at Source

This IE uniquely identifies a UE association over an NG interface and the source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Source NG-RAN node | M |  | Global RAN Node ID 9.2.1.131 |  | – |  |
| RAN UE NGAP ID | M |  | 9.2.1.145 | Allocated at the source (Master-)NG-RAN node. | – |  |

Similar change is needed to support the N2-based HO for AMF change. When the “target” gNB receives this ID, “target” gNB can know the HO is for an existing UE served by the “source” gNB, thus skip the admission control, resource reservation, and radio parameter reconfiguration.

Proposal 1: To enable the “target’ gNB know the N2-HO is related to an existing UE, introduce a new ID in the source NG-RAN node to Target NG-RAN node transparent container.

A draft TP for TS38.413 BL CR can be found at the annex section.

# 3	Conclusions

In this contribution, we analyzed the impact to RAN3 to support country specific routing for an RRC CONNECTED UE. Our observation and proposals are:

Observation 1: during the N2-HO to change the AMF for a UE, the gNB shall know the HO is related to an existing UE.

Proposal 1: To enable the “target’ gNB know the N2-HO is related to an existing UE, introduce a new ID in the source NG-RAN node to Target NG-RAN node transparent container.

A draft TP for TS38.413 BL CR can be found at the annex section.

# References

- TS 23.502

- TS 36.413

- (BL CR for TS38.413) R3-211487, Support of NTN RAT identification and NTN RAT restrictions

Text proposal to TS38.413 BL CR

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

**  Unchanged part is skipped  **

If the Extended Connected Time IE is included in the HANDOVER REQUEST message, the NG-RAN node shall, if supported, use it as described in TS 23.501 [9].

If the target NG-RAN node receives the UE Context Reference at Source IE in the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE, it may use it to identify an existing UE.

#### 9.3.1.29	Source NG-RAN Node to Target NG-RAN Node Transparent Container

This IE is produced by the source NG-RAN node and is transmitted to the target NG-RAN node. For inter-system handovers to 5G, the IE is transmitted from the external handover source to the target NG-RAN node.

This IE is transparent to the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING | Includes the RRC HandoverPreparationInformation message as defined in TS 38.331 [18] if the target is a gNB. Includes the RRC HandoverPreparationInformation message as defined in TS 36.331 [21] if the target is an ng-eNB. | - |  |
| PDU Session Resource Information List |  | 0..1 |  | For intra-system handovers in NG-RAN. | - |  |
| >PDU Session Resource Information Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>QoS Flow Information List |  | 1 |  |  | - |  |
| >>>QoS Flow Information Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>>>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>>>DL Forwarding | O |  | 9.3.1.33 |  | - |  |
| >>>>UL Forwarding | O |  | 9.3.1.118 |  | YES | reject |
| >>DRBs to QoS Flows Mapping List | O |  | 9.3.1.34 |  | - |  |
| E-RAB Information List |  | 0..1 |  | For inter-system handovers to 5G. | - |  |
| >E-RAB Information Item |  | 1..<maxnoofE-RABs> |  |  | - |  |
| >>E-RAB ID | M |  | 9.3.2.3 |  | - |  |
| >>DL Forwarding | O |  | 9.3.1.33 |  | - |  |
| Target Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Index to RAT/Frequency Selection Priority | O |  | 9.3.1.61 |  | - |  |
| UE History Information | M |  | 9.3.1.95 |  | - |  |
| SgNB UE X2AP ID | O |  | 9.3.1.127 | Allocated at the Source en-gNB | - |  |
| UE History Information from UE | O |  | 9.3.1.166 |  | YES | ignore |
| UE Context Reference at Source | O |  | 9.3.1.xxx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofE-RABs | Maximum no. of E-RABs allowed towards one UE. Value is 256. |

#### 9.3.1.xxx	UE Context Reference at Source

This IE uniquely identifies a UE association over an NG interface and the source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Source NG-RAN node | M |  | Global gNB ID 9.3.1.6 |  | – |  |
| RAN UE NGAP ID | M |  | 9.3.3.2 | Allocated at the source NG-RAN node. | – |  |

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AlternativeQoSParaSetList,

id-BurstArrivalTimeDownlink,

id-Cause,

id-ContextatSource,

id-CNPacketDelayBudgetDL,

id-CNPacketDelayBudgetUL,

**  Unchanged part is skipped  **

ConfiguredTACIndication ::= ENUMERATED {

true,

...

}

ContextatSource ::= SEQUENCE {

sourceNG-RAN-node-ID				GlobalGNB-ID,

rAN-UE-NGAP-ID						RAN-UE-NGAP-ID,

iE-Extensions						ProtocolExtensionContainer { {ContextatSource-ExtIEs} }	OPTIONAL,

...

}

ContextatSource-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CoreNetworkAssistanceInformationForInactive ::= SEQUENCE {

uEIdentityIndexValue				UEIdentityIndexValue,

uESpecificDRX						PagingDRX																	OPTIONAL,

periodicRegistrationUpdateTimer		PeriodicRegistrationUpdateTimer,

mICOModeIndication					MICOModeIndication															OPTIONAL,

tAIListForInactive					TAIListForInactive,

expectedUEBehaviour					ExpectedUEBehaviour															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {CoreNetworkAssistanceInformationForInactive-ExtIEs} }	OPTIONAL,

...

}

**  Unchanged part is skipped  **

SONInformationRequest ::= ENUMERATED {

xn-TNL-configuration-info,

...

}

SourceNGRANNode-ToTargetNGRANNode-TransparentContainer ::= SEQUENCE {

rRCContainer							RRCContainer,

pDUSessionResourceInformationList		PDUSessionResourceInformationList													OPTIONAL,

e-RABInformationList					E-RABInformationList																OPTIONAL,

targetCell-ID							NGRAN-CGI,

indexToRFSP								IndexToRFSP																			OPTIONAL,

uEHistoryInformation					UEHistoryInformation,

iE-Extensions		ProtocolExtensionContainer { {SourceNGRANNode-ToTargetNGRANNode-TransparentContainer-ExtIEs} }	OPTIONAL,

...

}

SourceNGRANNode-ToTargetNGRANNode-TransparentContainer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SgNB-UE-X2AP-ID	CRITICALITY ignore	EXTENSION SgNB-UE-X2AP-ID 												PRESENCE optional		}|

{ ID id-UEHistoryInformationFromTheUE		CRITICALITY ignore	EXTENSION UEHistoryInformationFromTheUE		PRESENCE optional		}|

{ID id-ContextatSource						CRITICALITY ignore	EXTENSION ContextatSource							PRESENCE optional		},

...

}

SourceOfUEActivityBehaviourInformation ::= ENUMERATED {

subscription-information,

statistics,

...

}

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

**  Unchanged part is skipped  **

id-NB-IoT-PagingDRX									ProtocolIE-ID ::= 324

id-TraceCollectionEntityURI							ProtocolIE-ID ::= 325

id-EmergencyIndicator								ProtocolIE-ID ::= 326

id-ContextatSource									ProtocolIE-ID ::= 3xx

END