3GPP TSG-RAN WG3 Meeting #112-e	R3-212011

E-meeting, 17-28 May 2021

Title: 	(TP for SON BL CR for TS 38.300): Minimum activation time for energy saving)

Source: 	Huawei

Agenda item:	10.2.3

Document Type:	Other

1. Introduction

The following agreement was made at last meeting, for inter-system energy saving.

Minimum activation time to reduce ping-pong is beneficial; details (e.g. stage-2 or stage-3 are to be discussed).

The open issue is to continue discuss whether to specify the minimum activation time in stage 2 or stage 3.  In the SoD [1] of last meeting. It seems that specifying the activation time in stage 2 only is a compromised solution which can be agreed by all companies.

This contribution provides a TP to SON BLCR to TS 38.300 to reflect the compromised solution.

2. Reference

- R3-211173, SoD on Inter-System Inter-RAT Energy Saving.

Annex – TP for SON BL CR for TS 38.300

## 15.4	Support for Energy Saving

### 15.4.1	General

The aim of this function is to reduce operational expenses through energy savings.

The function allows, for example in a deployment where capacity boosters can be distinguished from cells providing basic coverage, to optimize energy consumption enabling the possibility for an E-UTRA or NR cell providing additional capacity via single or dual connectivity, to be switched off when its capacity is no longer needed and to be re-activated on a need basis.

### 15.4.2	Solution description

### 15.4.2.1	Intra-system energy saving

The solution builds upon the possibility for the NG-RAN node owning a capacity booster cell to autonomously decide to switch-off such cell to lower energy consumption (inactive state). The decision is typically based on cell load information, consistently with configured information. The switch-off decision may also be taken by O&M.

The NG-RAN node may initiate handover actions in order to off-load the cell being switched off and may indicate the reason for handover with an appropriate cause value to support the target node in taking subsequent actions, e.g. when selecting the target cell for subsequent handovers.

All neighbour NG-RAN nodes are informed by the NG-RAN node owning the concerned cell about the switch-off actions over the Xn interface, by means of the NG-RAN node Configuration Update procedure.

All informed nodes maintain the cell configuration data, e.g., neighbour relationship configuration, also when a certain cell is inactive. If basic coverage is ensured by NG-RAN node cells, NG-RAN node owning non-capacity boosting cells may request a re-activation over the Xn interface if capacity needs in such cells demand to do so. This is achieved via the Cell Activation procedure. During switch off time period of the boost cell, the NG-RAN node may prevent idle mode UEs from camping on this cell and may prevent incoming handovers to the same cell.

The NG-RAN node receiving a request should act accordingly. The switch-on decision may also be taken by O&M. All peer NG-RAN nodes are informed by the NG-RAN node owning the concerned cell about the re-activation by an indication on the Xn interface.

### 15.4.2.2	Inter-system energy saving

The solution builds upon the possibility for the NG-RAN node owning a capacity booster cell to autonomously decide to switch-off such cell to dormant state. The decision is typically based on cell load information, consistently with configured information. The switch-off decision may also be taken by O&M. The NG-RAN node indicates the switch-off action to the eNB over NG interface and S1 interface. The NG-RAN node could also indicates the switch-on action to the eNB over NG interface and S1 interface.

The eNB providing basic coverage may request a NG-RAN node’s cell re-activation based on its own cell load information or neighbour cell load information, the switch-on decision may also be taken by O&M. The eNB requests a NG-RAN node’s cell re-activation and receives the NG-RAN node’s cell re-activation reply from the NG-RAN node over the S1 interface and NG interface. The NG-RAN node should keep the re-activated cell in active state for a certain time by implementation to avoid ping-pong effect and to wait UEs in the eNB to complete measurement towards the re-activated cell.

### 15.4.3	O&M requirements

Operators should be able to configure the energy saving function.

The configured information should include:

-	The ability of an NG-RAN node to perform autonomous cell switch-off;

-	The ability of an NG-RAN node to request the re-activation of a configured list of inactive cells owned by a peer NG-RAN node.

O&M may also configure

-	policies used by the NG-RAN node for cell switch-off decision;

-	policies used by peer NG-RAN nodes for requesting the re-activation of an inactive cell.