3GPP TSG-RAN WG3 Meeting #112-e	R3-212021

(was R3-210872)

# E-meeting, 17-28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.473 | CR | 0007 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37.473 on MeasGapSharingConfig |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the MeasConfig configured to the UE, MeasGapSharingConfig is used to define the scheme for sharing gaps when UE is required to perform different types of gap-needed measurements, which was introduced by RAN2 for R15. In CU-DU architecture, gaps are generated by DU and sent to CU. However, in the current DU to CU RRC Information, MeasGapConfig is included whereas MeasGapSharingConfig is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add MeasGapSharingConfig into DU to CU RRC Information. Impact Analysis: This CR has isolated impact with the previous version of the specification (same release) This CR has no impact under functional point of view. The impact can be considered isolated because the change affects only the ng-eNB behaviour. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CU cannot determine gap sharing scheme, thus unable to send MeasGapSharingConfig to UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.58, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-210872 Rev1: R3-212021 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.58	DU to CU RRC Information

This IE contains the RRC Information that are sent from the ng-eNB-DU to the ng-eNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RadioResourceConfigDedicated | M |  | OCTET STRING | RadioResourceConfigDedicated, as defined in TS 36.331 [2]. |  |  |
| MeasGapConfig | O |  | OCTET STRING | MeasGapConfig as defined in TS 36.331 [2]. |  |  |
| Requested P-MaxFR1 | O |  | OCTET STRING | requestedP-MaxFR1, as defined in TS 36.331 [2]. |  |  |
| DRX Long Cycle Start Offset | O |  | INTEGER (0..10239) | Identical to the value of the drx-LongCycleStartOffset IE within the DRX-Config as defined in TS 36.331 [2]. |  |  |
| Selected BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [16]. | YES | ignore |
| Selected FeatureSetEntryIndex | O |  | OCTET STRING | FeatureSetEntryIndex, as defined in TS 38.331 [16]. | YES | ignore |
| Ph-InfoSCG | O |  | OCTET STRING | PH-TypeListSCG, as defined in TS 38.331[16]. | Yes | ignore |
| Requested BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [16]. This IE is used for the ng-eNB-DU to request a new Band Combination. | YES | ignore |
| Requested FeatureSetEntryIndex | O |  | OCTET STRING | FeatureSetEntryIndex, as defined in TS 38.331 [16]. This IE is used for the ng-eNB-DU to request a new Feature Set. | YES | ignore |
| DRX Config | O |  | OCTET STRING | DRX-Config, as defined in TS 36.331 [2]. | YES | ignore |
| MeasGapSharingConfig | O |  | OCTET STRING | MeasGapSharingConfig as defined in TS 38.331 [8]. | YES | ignore |

<Unchanged Text Omitted>

-- D

DLUPTNLInformation-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDLUPTNLInformation)) OF DLUPTNLInformation-ToBeSetup-Item

DLUPTNLInformation-ToBeSetup-Item ::= SEQUENCE {

dLUPTNLInformation	UPTransportLayerInformation	,

iE-Extensions	ProtocolExtensionContainer { { DLUPTNLInformation-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

DLUPTNLInformation-ToBeSetup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Activity-Item ::= SEQUENCE {

dRBID			DRBID,

dRB-Activity	DRB-Activity		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRB-Activity-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Activity-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Activity ::= ENUMERATED {active, not-active}

DRBID ::= INTEGER (1..32, ...)

DRBs-FailedToBeModified-Item	::= SEQUENCE {

dRBID		DRBID,

cause		Cause		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-FailedToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-FailedToBeModified-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-FailedToBeSetup-Item	::= SEQUENCE {

dRBID	DRBID,

cause	Cause	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-FailedToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-FailedToBeSetup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-FailedToBeSetupMod-Item	::= SEQUENCE {

dRBID		DRBID,

cause		Cause			OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-FailedToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-FailedToBeSetupMod-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Information	::=	SEQUENCE {

dRB-QoS		QoSFlowLevelQoSParameters,

sNSSAI		SNSSAI,

notificationControl		NotificationControl		OPTIONAL,

flows-Mapped-To-DRB-List	Flows-Mapped-To-DRB-List,

iE-Extensions	ProtocolExtensionContainer { { DRB-Information-ItemExtIEs } }	OPTIONAL

}

DRB-Information-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Modified-Item	::= SEQUENCE {

dRBID									DRBID,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

rLC-Status								RLC-Status								OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Modified-ItemExtIEs } }		OPTIONAL,

...

}

DRBs-Modified-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-ModifiedConf-Item	::= SEQUENCE {

dRBID							DRBID,

uLUPTNLInformation-ToBeSetup-List		ULUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ModifiedConf-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ModifiedConf-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Notify-Item ::= SEQUENCE {

dRBID			DRBID,

notification-Cause	Notification-Cause,

iE-Extensions	ProtocolExtensionContainer { { DRB-Notify-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Notify-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Required-ToBeModified-Item	::= SEQUENCE {

dRBID							DRBID,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

rLC-Status								RLC-Status							OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Required-ToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Required-ToBeModified-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Required-ToBeReleased-Item	::= SEQUENCE {

dRBID		DRBID,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Required-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Required-ToBeReleased-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Setup-Item ::= SEQUENCE {

dRBID							DRBID,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Setup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Setup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-SetupMod-Item	::= SEQUENCE {

dRBID							DRBID,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-SetupMod-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-SetupMod-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-ToBeModified-Item	::= SEQUENCE {

dRBID								DRBID,

qoSInformation						QoSInformation							OPTIONAL,

uLUPTNLInformation-ToBeSetup-List	ULUPTNLInformation-ToBeSetup-List,

dLPDCPSNLength						PDCPSNLength							OPTIONAL,

uLPDCPSNLength						PDCPSNLength							OPTIONAL,

bearerTypeChange					BearerTypeChange						OPTIONAL,

rLCMode								RLCMode									OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { DRBs-ToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeModified-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-ToBeReleased-Item	::= SEQUENCE {

dRBID	DRBID,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeReleased-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-ToBeSetup-Item ::= SEQUENCE	{

dRBID								DRBID,

qoSInformation						QoSInformation,

uLUPTNLInformation-ToBeSetup-List	ULUPTNLInformation-ToBeSetup-List,

rLCMode								RLCMode,

dLPDCPSNLength						PDCPSNLength,

uLPDCPSNLength						PDCPSNLength								OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeSetup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-ToBeSetupMod-Item	::= SEQUENCE {

dRBID									DRBID,

qoSInformation							QoSInformation,

uLUPTNLInformation-ToBeSetup-List		ULUPTNLInformation-ToBeSetup-List,

rLCMode									RLCMode,

dLPDCPSNLength							PDCPSNLength							OPTIONAL,

uLPDCPSNLength							PDCPSNLength							OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeSetupMod-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRXCycle	::= SEQUENCE {

longDRXCycleLength		LongDRXCycleLength,

shortDRXCycleLength		ShortDRXCycleLength		OPTIONAL,

shortDRXCycleTimer		ShortDRXCycleTimer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { DRXCycle-ExtIEs} }	OPTIONAL,

...

}

DRXCycle-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRX-Config ::= OCTET STRING

DRX-LongCycleStartOffset ::= INTEGER (0..10239)

DUtoCURRCContainer ::= OCTET STRING

DUtoCURRCInformation ::= SEQUENCE {

radioResourceConfigDedicated		RadioResourceConfigDedicated,

measGapConfig						MeasGapConfig					OPTIONAL,

requestedP-MaxFR1					OCTET STRING					OPTIONAL,

dRX-LongCycleStartOffset			DRX-LongCycleStartOffset		OPTIONAL,

selectedBandCombinationIndex		SelectedBandCombinationIndex	OPTIONAL,

selectedFeatureSetEntryIndex		SelectedFeatureSetEntryIndex	OPTIONAL,

ph-InfoSCG							Ph-InfoSCG						OPTIONAL,

requestedBandCombinationIndex		RequestedBandCombinationIndex	OPTIONAL,

requestedFeatureSetEntryIndex		RequestedFeatureSetEntryIndex	OPTIONAL,

dRX-Config							DRX-Config						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { DUtoCURRCInformation-ExtIEs} } OPTIONAL,

...

}

DUtoCURRCInformation-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

{ID id-MeasGapSharingConfig		CRITICALITY ignore	EXTENSION MeasGapSharingConfig		PRESENCE optional},

...

}

Dynamic5QIDescriptor	::= SEQUENCE {

qoSPriorityLevel			INTEGER (1..127),

packetDelayBudget			PacketDelayBudget,

packetErrorRate				PacketErrorRate,

fiveQI						INTEGER (0..255, ...)					OPTIONAL,

delayCritical					ENUMERATED {delay-critical, non-delay-critical}		OPTIONAL,

averagingWindow 			AveragingWindow										OPTIONAL,

maxDataBurstVolume			MaxDataBurstVolume									OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Dynamic5QIDescriptor-ExtIEs } } OPTIONAL,

...

}

Dynamic5QIDescriptor-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

<Unchanged Text Omitted>

-- M

MaxDataBurstVolume  ::= INTEGER (0..4095,...)

MaxPacketLossRate ::= INTEGER (0..1000)

MIB-message ::= OCTET STRING

MeasConfig ::= OCTET STRING

MeasGapConfig ::= OCTET STRING

MeasGapSharingConfig ::= OCTET STRING

MeasurementTimingConfiguration ::= OCTET STRING

MessageIdentifier ::= BIT STRING (SIZE (16))

<Unchanged Text Omitted>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

W1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) w1ap (3) version1 (1) w1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM W1AP-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

id-W1Setup									ProcedureCode ::= 1

id-ErrorIndication							ProcedureCode ::= 2

id-ngeNBDUConfigurationUpdate				ProcedureCode ::= 3

id-ngeNBCUConfigurationUpdate				ProcedureCode ::= 4

id-UEContextSetup							ProcedureCode ::= 5

id-UEContextRelease							ProcedureCode ::= 6

id-UEContextModification					ProcedureCode ::= 7

id-UEContextModificationRequired			ProcedureCode ::= 8

id-UEMobilityCommand						ProcedureCode ::= 9

id-UEContextReleaseRequest					ProcedureCode ::= 10

id-InitialULRRCMessageTransfer				ProcedureCode ::= 11

id-DLRRCMessageTransfer						ProcedureCode ::= 12

id-ULRRCMessageTransfer						ProcedureCode ::= 13

id-privateMessage							ProcedureCode ::= 14

id-UEInactivityNotification					ProcedureCode ::= 15

id-NGENBDUResourceCoordination				ProcedureCode ::= 16

id-Paging									ProcedureCode ::= 17

id-Notify									ProcedureCode ::= 18

id-WriteReplaceWarning						ProcedureCode ::= 19

id-PWSCancel								ProcedureCode ::= 20

id-PWSRestartIndication						ProcedureCode ::= 21

id-PWSFailureIndication						ProcedureCode ::= 22

id-NGENBDUStatusIndication 					ProcedureCode ::= 23

-- **************************************************************

--

-- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxEARFCN								INTEGER ::= 262143

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualW1ConnectionsToReset	INTEGER ::= 65536

maxCellinngeNBDU						INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofE-UTRANCellBands					INTEGER ::= 8

maxnoofSIBTypes							INTEGER ::= 32

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB							INTEGER ::= 256

maxnoofExtendedBPLMNs					INTEGER ::= 6

maxnoofUEIDs							INTEGER ::= 65536

maxBandsEutra							INTEGER ::= 256

maxnoofAdditionalSIBs					INTEGER ::= 63

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause													ProtocolIE-ID ::= 0

id-Cells-Failed-to-be-Activated-List						ProtocolIE-ID ::= 1

id-Cells-to-be-Activated-List							ProtocolIE-ID ::= 2

id-Cells-to-be-Deactivated-List							ProtocolIE-ID ::= 3

id-CriticalityDiagnostics								ProtocolIE-ID ::= 4

id-CUtoDURRCInformation									ProtocolIE-ID ::= 5

id-DRBs-FailedToBeModified-List							ProtocolIE-ID ::= 6

id-DRBs-FailedToBeSetup-List								ProtocolIE-ID ::= 7

id-DRBs-FailedToBeSetupMod-List							ProtocolIE-ID ::= 8

id-DRBs-ModifiedConf-List								ProtocolIE-ID ::= 9

id-DRBs-Modified-List									ProtocolIE-ID ::= 10

id-DRBs-Required-ToBeModified-List						ProtocolIE-ID ::= 11

id-DRBs-Required-ToBeReleased-List						ProtocolIE-ID ::= 12

id-DRBs-Setup-List										ProtocolIE-ID ::= 13

id-DRBs-SetupMod-List									ProtocolIE-ID ::= 14

id-DRBs-ToBeModified-List								ProtocolIE-ID ::= 15

id-DRBs-ToBeReleased-List								ProtocolIE-ID ::= 16

id-DRBs-ToBeSetup-List									ProtocolIE-ID ::= 17

id-DRBs-ToBeSetupMod-List								ProtocolIE-ID ::= 18

id-DRXCycle												ProtocolIE-ID ::= 19

id-DUtoCURRCInformation									ProtocolIE-ID ::= 20

id-ngeNB-CU-UE-W1AP-ID									ProtocolIE-ID ::= 21

id-ngeNB-DU-UE-W1AP-ID									ProtocolIE-ID ::= 22

id-ngeNB-DU-ID											ProtocolIE-ID ::= 23

id-ngeNB-DU-Served-Cells-List							ProtocolIE-ID ::= 24

id-oldngeNB-DU-UE-W1AP-ID								ProtocolIE-ID ::= 25

id-RAT-FrequencyPriorityInformation						ProtocolIE-ID ::= 26

id-ResetType											ProtocolIE-ID ::= 27

id-ResourceCoordinationTransferContainer				ProtocolIE-ID ::= 28

id-RRCContainer											ProtocolIE-ID ::= 29

id-SCell-ToBeRemoved-List								ProtocolIE-ID ::= 30

id-SCell-ToBeSetup-List									ProtocolIE-ID ::= 31

id-SCell-ToBeSetupMod-List								ProtocolIE-ID ::= 32

id-Served-Cells-To-Add-List								ProtocolIE-ID ::= 33

id-Served-Cells-To-Delete-List							ProtocolIE-ID ::= 34

id-Served-Cells-To-Modify-List							ProtocolIE-ID ::= 35

id-SpCell-ID											ProtocolIE-ID ::= 36

id-SRBID												ProtocolIE-ID ::= 37

id-SRBs-FailedToBeSetup-List							ProtocolIE-ID ::= 38

id-SRBs-FailedToBeSetupMod-List							ProtocolIE-ID ::= 39

id-SRBs-Required-ToBeReleased-List						ProtocolIE-ID ::= 40

id-SRBs-ToBeReleased-List								ProtocolIE-ID ::= 41

id-SRBs-ToBeSetup-List									ProtocolIE-ID ::= 42

id-SRBs-ToBeSetupMod-List								ProtocolIE-ID ::= 43

id-TransactionID										ProtocolIE-ID ::= 44

id-TransmissionActionIndicator							ProtocolIE-ID ::= 45

id-UE-associatedLogicalW1-ConnectionListResAck			ProtocolIE-ID ::= 46

id-SCell-FailedtoSetup-List								ProtocolIE-ID ::= 47

id-SCell-FailedtoSetupMod-List							ProtocolIE-ID ::= 48

id-RRCReconfigurationCompleteIndicator 					ProtocolIE-ID ::= 49

id-Cells-Status-List									ProtocolIE-ID ::= 50

id-Candidate-SpCell-List								ProtocolIE-ID ::= 51

id-Potential-SpCell-List								ProtocolIE-ID ::= 52

id-C-RNTI												ProtocolIE-ID ::= 53

id-InactivityMonitoringRequest							ProtocolIE-ID ::= 54

id-InactivityMonitoringResponse							ProtocolIE-ID ::= 55

id-DRB-Activity-List									ProtocolIE-ID ::= 56

id-EUTRA-NR-CellResourceCoordinationReq-Container 		ProtocolIE-ID ::= 57

id-EUTRA-NR-CellResourceCoordinationReqAck-Container	ProtocolIE-ID ::= 58

id-RequestType 											ProtocolIE-ID ::= 59

id-ServCellIndex										ProtocolIE-ID ::= 60

id-EUTRANCGI											ProtocolIE-ID ::= 61

id-PagingCell-List										ProtocolIE-ID ::= 62

id-PagingDRX											ProtocolIE-ID ::= 63

id-UEIdentityIndexValue									ProtocolIE-ID ::= 64

id-PagingIdentity										ProtocolIE-ID ::= 65

id-DUtoCURRCContainer									ProtocolIE-ID ::= 66

id-DRB-Notify-List										ProtocolIE-ID ::= 67

id-NotficationControl									ProtocolIE-ID ::= 68

id-PWSSystemInformation									ProtocolIE-ID ::= 69

id-RepetitionPeriod										ProtocolIE-ID ::= 70

id-NumberofBroadcastRequest								ProtocolIE-ID ::= 71

id-Cells-To-Be-Broadcast-List							ProtocolIE-ID ::= 72

id-Cells-Broadcast-Completed-List 						ProtocolIE-ID ::= 73

id-Broadcast-To-Be-Cancelled-List 						ProtocolIE-ID ::= 74

id-Cells-Broadcast-Cancelled-List 						ProtocolIE-ID ::= 75

id-E-UTRAN-CGI-List-For-Restart-List 					ProtocolIE-ID ::= 76

id-PWS-Failed-E-UTRAN-CGI-List 							ProtocolIE-ID ::= 77

id-ConfirmedUEID										ProtocolIE-ID ::= 78

id-Cancel-all-Warning-Messages-Indicator				ProtocolIE-ID ::= 79

id-NGENB-DU-UE-AMBR-UL									ProtocolIE-ID ::= 80

id-ServingPLMN											ProtocolIE-ID ::= 81

id-NGENBDUOverloadInformation							ProtocolIE-ID ::= 82

id-ResourceCoordinationTransferInformation				ProtocolIE-ID ::= 83

id-Associated-SCell-List								ProtocolIE-ID ::= 84

id-SRBs-Setup-List										ProtocolIE-ID ::= 85

id-SRBs-SetupMod-List									ProtocolIE-ID ::= 86

id-SRBs-Modified-List									ProtocolIE-ID ::= 87

id-IgnoreResourceCoordinationRequestContainer			ProtocolIE-ID ::= 88

id-NotificationInformation								ProtocolIE-ID ::= 89

id-AdditionalSIBMessageList								ProtocolIE-ID ::= 90

id-MeasGapSharingConfig									ProtocolIE-ID ::= xx1

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>