3GPP TSG-RAN WG3 Meeting #112-e	R3-212213

E-meeting, 17-28 May 2021

Title: 	Discussion on E-CID LTE measurements

Source: 	Huawei

Agenda item:	8.1

Document Type:	Other

# 1. Introduction

A LS has been received from RAN2 and has the following description [1]:

During RAN2#113-e, RAN2 discussed the support for gNB reporting E-UTRA measurements for UL E-CID positioning in Rel-15. RAN2 kindly requests RAN3 to confirm whether gNB can report E-UTRA measurement to the LMF for UL E-CID positioning in Rel-15.

This paper discusses the E-CID measurements in Rel-15.

# 2. Discussion

In the E-CID Measurement Initiation procedure, the following information can be reported from the NG-RAN to LMF:

- E-CID Measurement Result - Cell Portion ID - Other-RAT Measurement Result - WLAN Measurement Result

The “E-CID Measurement Result” refers to the measurements results at the serving cell of the UE while “Other-RAT Measurement Result” includes the measurement results of the other RAT cells, which could be neighbouring cells or second cells in DC scenarios. When a UE served by an NR cell would like to report the E-UTRAN measurement to the LMF, the E-UTRA measurement needs to be included in the “Other-RAT Measurement Result”.

Observation 1: When a UE served by an NR cell would like to report the E-UTRAN measurement to the LMF, the E-UTRA measurement needs to be included in the “Other-RAT Measurement Result”.

According to the TS 38.455 v15.3.0 [2], the “Other-RAT Measurement Result” IE includes the following information:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Other-RAT Measured Results |  | 1.. <maxnoMeas> |  |  |
| >CHOICE Other-RAT Measured Results Value | M |  |  |  |
| >>Result GERAN | M | 1..<maxGERANMeas> |  |  |
| >>> sub IE omitted.. |  |  |  |  |
| >>Result UTRAN |  | 1..<maxUTRANMeas> |  |  |
| >>> sub IE omitted.. |  |  |  |  |

Thus, the other-RAT measurements only include GERAN and UTRAN measurements. This means the serving gNB cannot report other-RAT measurements except for the GERAN and UTRAN cells.

Observation 2: the other-RAT measurements in Rel-15 E-CID positioning only include GERAN and UTRAN measurements, but not EUTRAN measurements.

In RAN3#100 meeting, the paper R3-182768 had the following proposal [3]:

“Proposal 3: In NRPPa, extend both the Inter-RAT Measurement Quantities Item IE in the E-CID MEASUREMENT INITIATION REQUEST message, and the CHOICE structure in the Inter-RAT Measurement Result IE, to include E-UTRAN measurement results.”

This proposal aims to let the serving gNB to report measurement results of neighbouring E-UTRAN cells to the LMF. However, this paper was not agreed.

R3-182768 Inter-RAT Measurements for E-CID Type: pCR For: (not specified) 38.455 v0.6.0 Source: Ericsson Decision: The document was noted.

Therefore, the RAN3 could have the common understanding that in Rel-15 the serving gNB cannot report E-UTRAN measurement results to the LMF for E-CID positioning.

Observation 3: The proposal for gNB to report E-UTRAN measurements to LMF was not agreed in Rel-15.

Proposal 1: Send the reply LS to RAN2 to confirm that the gNB cannot report E-UTRA measurements to the LMF for UL E-CID positioning in Rel-15.

# 3. Conclusion

Based on the discussion in this paper, we propose the following:

Observation 1: When a UE served by an NR cell would like to report the E-UTRAN measurement to the LMF, the E-UTRA measurement needs to be included in the “Other-RAT Measurement Result”.

Observation 2: the other-RAT measurements in Rel-15 E-CID positioning only include GERAN and UTRAN measurements, but not EUTRAN measurements.

Observation 3: The proposal for gNB to report E-UTRAN measurements to LMF was not agreed in Rel-15.

Proposal 1: Send the reply LS to RAN2 to confirm that the gNB cannot report E-UTRA measurements to the LMF for UL E-CID positioning in Rel-15.

# 4. Reference

- R2-2102128 LS on E-CID LTE measurement.

- 3GPP TS 38.455 V15.3.0, NR Positioning Protocol A (NRPPa).

- R3-182768, Inter-RAT Measurements for E-CID.