3GPP TSG-RAN WG3 Meeting #112-e	R3-211261

# Online, May 17th – May 27th 2021

Agenda Item:	10.2.2

Source:	Ericsson

Title:	(TP for SON BL CR for TS 38.423): CCO

Document for:	Discussion, Decision

# 1	Introduction

This paper describes a sample TP for SON Baseline CR for TS 38.423 for CCO, according to the proposals in [1].

# Reference

- R3-212262 (TP for SON BL CR for TS 38.300) CCO

# Sample TP for XnAP (38.423) for CCO

/////////////////////////////////////// Change Start ///////////////////////////////////////////////

### 8.4.2	NG-RAN node Configuration Update

#### 8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending the NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the NG-RAN NODE CONFIGURATION UPDATE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells To Add IE and in the Served NR Cells To Modify IE.

If Supplementary Uplink is configured at the NG-RAN node2, the NG-RAN node2 shall include in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells IE if any.

If the TAI Support List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message, the receiving node shall replace the previously provided TAI Support List IE by the received TAI Support List IE.

If the Cell Assistance Information NR IE is present, the NG-RAN node2 shall, if supported, use it to generate the Served NR Cells IE and include the list in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

If the Cell Assistance Information E-UTRA IE is present, the NG-RAN node2 shall, if supported, use it to generate the Served E-UTRA Cells IE and include the list in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

If the Partial List Indicator NR IE is included in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message and set to "partial" the NG-RAN node1 shall, if supported, assume that the Served NR Cells IE in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message includes a partial list of NR cells.

If the Partial List Indicator E-UTRA IE is included in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message and set to "partial" the NG-RAN node1 shall, if supported, assume that the Served E-UTRA Cells IE in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message includes a partial list of NR cells.

If the Cell and Capacity Assistance Information NR IE is present in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message from the candidate NG-RAN node2, the NG-RAN node1 shall, if supported, store the collected information to be used for future NG-RAN node interface management.

If the Cell and Capacity Assistance Information E-UTRA IE is present in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message from the candidate NG-RAN node2, the NG-RAN node1 shall, if supported, store the collected information to be used for future NG-RAN node interface management.

Upon reception of the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall update the information for NG-RAN node1 as follows:

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the NG-RAN NODE CONFIGURATION UPDATE message and the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

If the TNL Configuration Info IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall take this IE into account for IPSec establishment.

If the TNL Configuration Info IE is contained in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 shall take this IE into account for IPSec establishment.

If the CSI-RS Transmission Indication IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall take this IE into account for neighbour cell’s CSI-RS measurement.

The NG-RAN NODE CONFIGURATION UPDATE message may contain for each cell served by NG-RAN node1 NPN related broadcast information. The NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message may contain for each cell served by NG-RAN node2 NPN related broadcast information.

If the NG-RAN Coverage Modification List IE is present in the NG-RAN NODE CONFIGURATION UPDATE, the NG-RAN node2 may use the information in the Cell Coverage State IE to identify the cell deployment configuration and for configuring the mobility towards the cell(s) indicated by the NG-RAN CGI IE, as described in TS 38.300 [8]. If the NG-RAN Coverage Modification List IE is present, the NG-RAN node2 may use the information in the SSB Coverage State IE to identify the SSB beam deployment configuration enabled by NG-RAN node1.

If the NG-RAN Deployment Status Indicator IE is present in the NG-RAN Coverage Modification List IE and the Replacing Cell List IE is also present, the NG-RAN node2 shall, if supported, consider the deployment configuration of the cells to be modified as the next planned configuration as indicated by the NG-RAN Replacing Info IE and shall remove any planned configuration stored for this cell.

If the NG-RAN Deployment Status Indicator IE is present in the NG-RAN Coverage Modification List IE and the Replacing SSB Beam List IE is also present, the NG-RAN node2 shall, if supported, consider the deployment configuration of the SSB beam to be modified as activated and replace any previous configuration for the cells and for the SSB beams indicated in the NG-RAN Coverage Modification List IE.

/////////////////////////////////////// Next Change ///////////////////////////////////////////////

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.2.3.1 |  |  |  | YES |  | reject |
| TAI Support List | O |  |  | 9.2.3.20 |  | List of supported TAs and associated characteristics. |  | GLOBAL |  | reject |
| CHOICE Initiating NodeType | M |  |  |  |  |  |  | YES |  | ignore |
| >gNB |  |  |  |  |  |  |  |  |  |  |
| >>Served Cells To Update NR | O |  |  | 9.2.2.15 |  |  |  | YES |  | ignore |
| >>Cell Assistance Information NR | O |  |  | 9.2.2.17 |  |  |  | YES |  | ignore |
| >ng-eNB |  |  |  |  |  |  |  |  |  |  |
| >>Served Cells to Update E-UTRA | O |  |  | 9.2.2.16 |  |  |  | YES |  | ignore |
| >>Cell Assistance Information NR | O |  |  | 9.2.2.17 |  |  |  | YES |  | ignore |
| TNLA To Add List |  |  | 0..1 |  |  |  | YES |  | ignore |  |
| >TNLA To Add Item |  |  | 1..<maxnoofTNLAssociations> |  |  |  | – |  |  |  |
| >>TNLA Transport Layer Information | M |  |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |  |  |
| >> TNL Association Usage | O |  |  |  | 9.2.3.84 |  | – |  |  |  |
| TNLA To Update List |  |  | 0..1 |  |  |  | YES |  | ignore |  |
| >TNLA To Update Item |  |  | 1..<maxnoofTNLAssociations> |  |  |  | – |  |  |  |
| >>TNLA Transport Layer Information | M |  |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |  |  |
| >> TNL Association Usage | O |  |  |  | 9.2.3.84 |  | – |  |  |  |
| TNLA To Remove List |  |  | 0..1 |  |  |  | YES |  | ignore |  |
| >TNLA To Remove Item |  |  | 1..<maxnoofTNLAssociations> |  |  |  | – |  |  |  |
| >>TNLA Transport Layer Information | M |  |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |  |  |
| Global NG-RAN Node ID | O |  |  |  | 9.2.2.3 |  | YES |  | reject |  |
| AMF Region Information To Add | O |  |  |  | AMF Region Information 9.2.3.83 | List of all added AMF Regions to which the NG-RAN node belongs. | YES |  | reject |  |
| AMF Region Information To Delete | O |  |  |  | AMF Region Information 9.2.3.83 | List of all deleted AMF Regions to which the NG-RAN node belongs. | YES |  | reject |  |
| Interface Instance Indication | O |  |  |  | 9.2.2.39 |  | YES |  | reject |  |
| TNL Configuration Info | O |  |  |  | 9.2.3.96 |  | YES |  | Ignore |  |
| NG-RAN Coverage Modification List |  |  | 0 .. < maxnoofCellsinNG-RANnode > |  |  | List of NR cells and SSB beams with modified coverage | GLOBAL |  | reject |  |
| > NG-RAN CGI | M |  |  |  | 9.2.3.25 | Cell Global Identifier of the cell to be modified | - |  | - |  |
| > Cell Coverage State | M |  |  |  | INTEGER (0..15, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell | - |  | - |  |
| > SSB Beam List |  |  | 0 .. < maxnoofSSBBeams> |  |  |  | - |  | - |  |
| >> SSB Beam Index | M |  |  |  | INTEGER (0..63) | Identify an SSB area of an NR cell | - |  | - |  |
| >> SSB Beam Coverage State | M |  |  |  | INTEGER (0..15, …) | Value '0' indicates that the SSB beam is inactive. Other values Indicates that the SSB beam is active and also indicates the coverage configuration of the concerned SSB beam | - |  | - |  |
| > NG-RAN Deployment Status Indicator | O |  |  |  | ENUMERATED (pre-change-notification, ...) | Indicates that the Cell Coverage State and, if present, the SSB Beam Coverage State, is planned to be used at the next reconfiguration | - |  | - |  |
| > NG-RAN Replacing Info | C-ifNGRANDeploymentStatusIndicatorPresent |  |  |  |  |  | - |  | - |  |
| >> Replacing Cells List |  |  | 0 .. < maxnoofCellsinNG-RANnode > |  |  |  | - |  | - |  |
| >>> NG-RAN CGI |  |  |  |  | 9.2.3.25 | Cell Global Identifier of a cell that may replace all or part of the coverage of the cell to be modified | - |  | - |  |
| >>> Replacing SSB Beams List |  |  | 0 .. < maxnoofSSBBeams> |  |  |  | - |  | - |  |
| >>>> SSB Beam Index |  |  |  |  | INTEGER (0..63) | Identify an SSB area of an NR cell | - |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the NG RAN nodes. Value is 32. |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |
| maxnoofSSBBeams | Maximum no. of SSB beams in a NR cell. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifNGRANDeploymentStatusIndicatorPresent | This IE shall be present if the NG-RAN Deployment Status Indicator IE is present. |

/////////////////////////////////////// End of Changes ///////////////////////////////////////////////