3GPP TSG-RAN3 Meeting # 112-e	   R3-212338

# 17th - 28th  May 2021

Agenda Item:	8.1

Source:	Ericsson

Title: 	Discussion on the RAN2 LS on E-CID LTE measurement in Rel-15

Document for:	Discussion

# 1	Introduction

RAN3 has received a liaison statement from RAN2 in [1], asking whether gNB can report E-UTRA measurement to the LMF for E-CID positioning in Rel-15. This discussion paper provides our view on the question raised by RAN2 and a proposition for the reply LS.

# 2	Discussion and background

The answer to RAN2’s question lies in the Release-15 version of the NRPPa specification [2]. In the E-CID related procedures such as the E-CID MEASUREMENT INITIATION REQUEST message, the LMF can ask the NG-RAN node to provide the cell ID and E-UTRA measurements, if available, such as the E-UTRA Angle of arrival, Timing advances and the E-UTRA RSRP & RSRQ measurements. The details of these measurements can be found in the E-CID Measurement Result IE in section 9.2.5 of [2]. One can observe that the NG-RAN node can report its access node position and that the LMF can further ask for other RATs (GERAN, UTRAN) and WLAN measurements.

Observation 1 : In release 15 NRPPa, LMF can get E-UTRA related measurements form the NG-RAN node.

However, it must be noted that the NG-RAN node mentioned hereabove means ng-eNB and not gNB.

In Release 15, gNB was still being specified with the CU-DU split. NRPPa meanwhile was ported directly from LPPa specification [3], taking the existing signalling between eNB and E-SMLC as baseline for the later-to-be-specified Rel-16 NR positioning. Therefore, in Rel-15, only E-UTRA cells could be considered, and the two positioning methods available in NRPPa are the ones that have been ported from LPPa based on E-UTRA measurements from LTE cells, which are: OTDOA and LTE E-CID.

If we also consider the positioning architecture figure below, one can observe that only the higher NG-RAN node block was considered, meaning that Rel-15 NRPPa only supports Option 5 architecture, i.e. ng-eNB connected to 5GC. The NRPPa support of the block below with gNB as NG-RAN node was later specified during Rel-16 with the NR positioning methods.

Figure 1 : Overall Positioning architecture as defined in TS 38.305. In Rel-15 NRPPa, only the ng-eNB block was supported

Observation 2 : In Release-15, the NRPPa protocol only supports the exchange of the positioning information and measurements between the ng-eNB and the LMF.

Based on the two observations above, it is proposed to reply to RAN2 with the following:

“RAN3 thanks RAN2 for their LS on Release-15 UL E-CID and would like to inform RAN2 that in Release-15 specification, NRPPa only supports the exchange of E-UTRA positioning information and measurements between an ng-eNB node and LMF. This includes E-UTRA measurements from ng-eNB for E-CID positioning.”

A draft LS reply is provided in [4].

Proposal 1 : Reply RAN2 with a clarification that in Rel-15 NRPPa, no gNB is in the scope; NRPPa only supports exchange between ng-eNB and LMF.

# Conclusion

In this paper, we have discussed the question raised by RAN2.

Observation 1 : In Release 15 NRPPa, LMF can get E-UTRA related measurements form the NG-RAN node.

Observation 2 : In Release-15, the NRPPa protocol only supports the exchange of the positioning information and measurements between the ng-eNB and the LMF.

Proposal 1 : Reply RAN2 with a clarification that in Rel-15 NRPPa, no gNB is in the scope; NRPPa only supports exchange between ng-eNB and LMF.

# Reference

- R2-2102128, LS on E-CID LTE measurement in Rel-15

- TS 38.455, NRPPa protocol V15.3.0

- TS 36.455, LPPa protocol V16.1.0

- R3-212339, Reply LS on E-CID LTE measurement in Rel-15