3GPP TSG-RAN3 Meeting #112-e	R3-212401

# E-meeting, 17th May - 28th May, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0613 | Rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS38.413 on including survival time in TSC Assistance Information enhancements. |
| --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2021-05-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To include the survival time in the TSC Assistance Information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce the Survival Time IE into the TSC Assistance Information IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The survival time can’t be supported in the NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.131, 9.3.1.1xx, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 38.423 CR 0625 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the 1st change

9.3.1.131	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.132 |  |
| Burst Arrival Time | O |  | 9.3.1.133 |  |
| Survival Time | O |  | 9.3.1.1xx |  |

***************************************************************************************************************************

Skip unchanged part

***************************************************************************************************************************

9.3.1.1xx	Survival Time

This IE indicates the Survial Time as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0..1920000, …) | Survival time expressed in units of 1 us. |

End of the 1st change

Start of the 2nd change

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

***************************************************************************************************************************

Skip unchanged part

***************************************************************************************************************************

id-UserLocationInformationW-AGF,

id-SurvivalTIme	,

maxnoofAllowedAreas,

***************************************************************************************************************************

Skip unchanged part

***************************************************************************************************************************

SupportedTAItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication	PRESENCE optional	}|

{ID id-RAT-Information				CRITICALITY reject	EXTENSION RAT-Information			PRESENCE optional	},

...

}

SurvivalTime ::= INTEGER (0..1920000, ...)

SuspendIndicator ::= ENUMERATED {

true,

...

}

***************************************************************************************************************************

Skip unchanged part

***************************************************************************************************************************

TSCAssistanceInformation ::= SEQUENCE {

periodicity				Periodicity,

burstArrivalTime		BurstArrivalTime												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TSCAssistanceInformation-ExtIEs} }	OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-SurvivalTime			CRITICALITY ignore	EXTENSION SurvivalTIme		PRESENCE optional },

...

}

TSCTrafficCharacteristics ::= SEQUENCE {

tSCAssistanceInformationDL		TSCAssistanceInformation								OPTIONAL,

tSCAssistanceInformationUL		TSCAssistanceInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TSCTrafficCharacteristics-ExtIEs} }	OPTIONAL,

...

}

End of the 2nd change

Start of the 3rd change

9.4.7	Constant Definitions

***************************************************************************************************************************

Skip unchanged part

***************************************************************************************************************************

id-ExtendedUEIdentityIndexValue							ProtocolIE-ID ::= 280

id-SurvivalTIme											ProtocolIE-ID ::= xxx

END

-- ASN1STOP

End of the 3rd change