3GPP TSG-RAN WG3 Meeting #112-e	R3-212425

E-meeting, 17-28 May 2021

Title: 	(TP to TS 38.413 BL CRs) Multicast Session Management

Source: 	Huawei, CBN

Agenda item:	22.2.2

Document Type:	other

1. Introduction

- As discussed in [1], the following proposals are achieved:

Proposal 1: SMF provides the MBS session and flow related info to gNB via PDU Session Resource Modify Request Transfer IE and PDU Session Resource Setup Request Transfer IE.

Proposal 2: In the PDU Session Resource Modify Request Transfer IE and PDU Session Resource Setup Request Transfer IE, add MBS QoS Flow Identifier IE and MBS Session ID IE in the same level of existing QoS Flow Identifier, reuse the existing QoS Flow Identifier to indicate the mapped unicast QoS Flow Identifier, FFS on the need of a new MBS QoS Flow Level QoS Parameters IE.

Proposal 3: Introduce a MBS supported IE in the PDU Session Resource Modify Response Transfer IE and PDU Session Resource Setup Response Transfer IE.

Proposal 4: Introduce non UE associated class1 NGAP: Session activation/deactivation procedures for Multicast Session Management.

Proposal 5: Introduce non UE associated Class 2 NGAP: Group Paging procedure to support group paging from CN to RAN.

Proposal 6: introduce non UE associated class1 NGAP: Multicast Distribution Setup procedure, triggered by NG-RAN node.

- in case the NG-RAN node decides or is configured to use unicast transport for multicast distribution sessions, it includes the DL GTP-U tunnel info to the core network in the Multicast Distribution Setup Request.

- in case the DL GTP-U tunnel info is not included in the Multicast Distribution Setup Request, the core network shall provide IP multicast DL tunnel info to the NG-RAN node, to enable multicast transport of the multicast distribution.

Proposal 7: introduce non UE associated class1 NGAP: Multicast Distribution Release procedure, triggered by NG-RAN node or 5GC.

- In this paper, we provide the text proposal to BL CR of TS 38.413 in Annex A.

5. Reference

- R3-212424 (TP to TS 38.410 BL CRs) Multicast Session Management, Huawei

Annex A. Text Proposal to BL CR of TS 38.413

-----------------Start of the Changes-------------------

8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Multicast Session Activation | MULTICAT SESSION ACTIVATION REQUEST | MULTICAT SESSION ACTIVATION RESPONSE | MULTICAT SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAT SESSION DEACTIVATION REQUEST | MULTICAT SESSION DEACTIVATION RESPONSE |  |
| Multicast Distribution Setup | MULTICAST DISTRIBUTION SETUP REQUEST | MULTICAST DISTRIBUTION SETUP RESPONSE | MULTICAST DISTRIBUTION SETUP FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE REQUEST | MULTICAST DISTRIBUTION RELEASE RESPONSE |  |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Group Paging | GROUP PAGING |

-----------------Start of the next Change-------------------

8.xx	MBS Session Management Procedures

8.xx.a	Multicast Session Activation

8.xx.a.1	General

The purpose of the Multicast Session Activation procedure is to request a NG-RAN node to activate the MBS resources of one MBS session and the corresponding MBS QoS flows.

The procedure uses non-UE-associated signalling.

8.xx.a.2	Successful Operation

Figure 8.xx.a.2-1: Multicast Session Activation, successful operation

An AMF initiates the procedure by sending a MULTICAST SESSION ACTIVATION REQUEST message to a NG-RAN node.

Upon receipt of this message, the NG-RAN node should activate the MBS resources indicated in the MULTICAST SESSION ACTIVATION REQUEST message and shall indicate in the MULTICAST SESSION ACTIVATION RESPONSE message for which MBS Session the request was fulfilled.

8.xx.a.3	Unsuccessful Operation

Figure 8.xx.a.3-1: Multicast Session Activation, unsuccessful operation

If the NG-RAN node cannot activate the MBS resources indicated in the MULTICAST SESSION ACTIVATION REQUEST message, it shall respond with a MULTICAST SESSION ACTIVATION FAILURE message with an appropriate cause value.

8.xx.a.4	Abnormal Conditions

Not applicable.

8.xx.b	Multicast Session Deactivation

8.xx.b.1	General

The purpose of the Multicast Session Deactivation procedure is to request a NG-RAN node to deactivate the MBS resources of one MBS session and the corresponding MBS QoS flows.

The procedure uses non-UE-associated signalling.

8.xx.b.2	Successful Operation

Figure 8.xx.b.2-1: Multicast Session Deactivation, successful operation

An AMF initiates the procedure by sending a MULTICAST SESSION DEACTIVATION REQUEST message to a NG-RAN node.

Upon receipt of this message, the NG-RAN node should deactivate the MBS resources indicated in the MULTICAST SESSION DEACTIVATION REQUEST message and shall indicate in the MULTICAST SESSION DEACTIVATION RESPONSE message for which MBS Session the request was fulfilled.

8.xx.b.3	Unsuccessful Operation

Not applicable.

8.xx.b.4	Abnormal Conditions

Not applicable.

8.xx.c	Multicast Distribution Setup

8.xx.c.1	General

The purpose of the Multicast Distribution Setup procedure is to assign NG-U resources for one MBS session and the corresponding MBS QoS flows. The procedure uses non-UE-associated signalling.

8.xx.c.2	Successful Operation

Figure 8.xx.c.2-1: Multicast Distribution Setup: successful operation

The NG-RAN node initiates the procedure by sending a MULTICAST DISTRIBUTION SETUP REQUEST message to the AMF. The AMF responds with a MULTICAST DISTRIBUTION SETUP RESPONSE message.

If the DL Transport Layer Information IE is included in the Multicast Distribution Setup Request Transfer IE in the MULTICAST DISTRIBUTION SETUP REQUEST message, the SMF shall consider that the included information as the downlink termination point for the included associated QoS flows for this MBS session.

If the DL Transport Layer Information IE is not included in the Multicast Distribution Setup Request Transfer IE in the MULTICAST DISTRIBUTION SETUP REQUEST message, the SMF shall interpret that the IP multicast is used for this MBS Session and provide IP Multicast Address Information IE in the Multicast Distribution Setup Response Transfer IE in the MULTICAST DISTRIBUTION SETUP RESPONSE message.

8.xx.c.3	Unsuccessful Operation

Figure 8.xx.c.3-1: Multicast Distribution Setup: unsuccessful operation.

In case the Multicast Distribution cannot be setup successfully, the AMF shall response with the MULTICAST DISTRIBUTION SETUP FAILURE message to the NG-RAN node with an appropriate cause value in the Cause IE.

8.xx.c.4	Abnormal Conditions

Not applicable.

8.xx.d	Multicast Distribution Release

8.xx.d.1	General

The purpose of the Multicast Distribution Release procedure is to enable the release of already established NG-U resources for a given MBS Session. The procedure uses non-UE-associated signalling.

8.xx.d.2	Successful Operation

Figure 8.xx.d.2-1: Multicast Distribution Release initiated by the NG-RAN node

Figure 8.xx.d.2-2: Multicast Distribution Release initiated by the AMF

The NG-RAN node or the AMF initiates the procedure by sending a MULTICAST DISTRIBUTION RELEASE REQUEST message.

Upon receipt of the MULTICAST DISTRIBUTION RELEASE REQUEST message, the receiving node shall send the MULTICAST DISTRIBUTION RELEASE RESPONSE message after successfully remove the corresponding NG-U resource for the MSB Session.

8.xx.d.3		Unsuccessful Operation

Not applicable.

8.xx.d.4	Abnormal Conditions

Not applicable.

-----------------Start of the next Change-------------------

8.yy	Group Paging Procedures

8.yy.1	Group Paging

8.yy.1.1	General

The purpose of the Group Paging procedure is to enable the AMF to page the UEs jointed a MBS Session in the specific NG-RAN node.

8.yy.1.2	Successful Operation

Figure 8.yy.1.2-1: Paging

The AMF initiates the Group Paging procedure by sending the GROUPING PAGING message to the NG-RAN node.

8.5.1.3	Abnormal Conditions

Void.

-----------------Start of the next Change-------------------

9.2.x	MBS Session Management Messages

9.2.x.a1	MULTICAST SESSION ACTIVATION REQUEST

This message is sent by the AMF to a NG-RAN node to request previously deactivated MBS resources to be re-activated.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| FFS |  |  |  |  |  |  |

9.2.x.a2	MULTICAST SESSION ACTIVATION RESPONSE

This message is sent by the NG-RAN node to the AMF to indicate that the MBS resources previously deactivated have been activated.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| FFS |  |  |  |  |  |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.x.a3	MULTICAST SESSION ACTIVATION FAILURE

This message is sent by the NG-RAN node to the AMF to indicate multicast session activation failure.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| FFS |  |  |  |  |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.x.b1	MULTICAST SESSION DEACTIVATION REQUEST

This message is sent by the AMF to a NG-RAN node to request to deactivate the MBS resources of a MBS Session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| FFS |  |  |  |  |  |  |

9.2.x.b2	MULTICAST SESSION DEACTIVATION RESPONSE

This message is sent by the NG-RAN node to the AMF to indicate that the MBS resources have been deactivated.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| FFS |  |  |  |  |  |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.x.c1	MULTICAST DISTRIBUTION SETUP REQUEST

This message is sent by the NG-RAN node to request the setup of the NG-U Transport for a MBS Session.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| FFS |  |  |  |  |  |  |
| Multicast Distribution Setup Request Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Setup Request Transfer IE specified in subclause 9.3.4.a. | YES | reject |

9.2.x.c2	MULTICAST DISTRIBUTION SETUP RESPONSE

This message is sent by the AMF to confirm the setup of the NG-U Transport for a MBS Session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| FFS |  |  |  |  |  |  |
| Multicast Distribution Setup Response Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Setup Response Transfer IE specified in subclause 9.3.4.b. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.x.c3	MULTICAST DISTRIBUTION SETUP FAILURE

This message is sent by the AMF to indicate that the setup of the NG-U Transport for a MBS Session was unsuccessful.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| FFS |  |  |  |  |  |  |
| Multicast Distribution Setup Unsuccessful Transfer | M |  | OCTET STRING | Containing the PDU Session Resource Setup Unsuccessful Transfer IE specified in subclause 9.3.4.c. |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

9.2.x.d1	MULTICAST DISTRIBUTION RELEASE REQUEST

This message is sent by the AMF and the NG-RAN node to request the release of the NG-U Transport for a MBS Session.

Direction: AMF  NG-RAN node and NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| FFS |  |  |  |  |  |  |
| Multicast Distribution Release Request Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Release Request Transfer IE specified in subclause 9.3.4.d. | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

9.2.x.d2	MULTICAST DISTRIBUTION RELEASE RESPONSE

This message is sent by the NG-RAN node and the AMF to confirm the release of the NG-U Transport for a MBS Session.

Direction: NG-RAN node  AMF and AMF  NG-RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| FFS |  |  |  |  |  |  |
| Multicast Distribution Release Response Transfer | M |  | OCTET STRING | Containing the Multicast Distribution Release Response Transfer IE specified in subclause 9.3.4.e. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

9.2.y	Group Paging Messages

9.2.y.1	GROUP PAGING

This message is sent by the AMF and is used to page the UEs jointed a MBS session in one or several tracking areas.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| FFS |  |  |  |  |  |  |

-----------------Start of the next Change-------------------

9.3.1.51	QoS Flow Identifier

This IE identifies a QoS flow within a PDU Session or a MBS QoS flow within a MBS Session. The definition and use of the QoS Flow Identifier is specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flow Identifier | M |  | INTEGER (0..63, …) |  |

-----------------Start of the next Change-------------------

9.3.1.x	MBS Session ID

This IE identifies a MBS Session. The definition and use of the MBS Session ID is specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | FFS |  |

9.3.1.y	MBS Supported

This IE indicates MBS is supported

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Supported | M |  | ENUMERATED (true, …) |  |

-----------------Start of the next Change-------------------

9.3.4.1	PDU Session Resource Setup Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | 9.3.1.102 | This IE shall be present when at least one Non-GBR QoS flow is being setup and is ignored otherwise. | YES | reject |
| UL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs. | YES | reject |
| Additional UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s), for delivery of UL PDUs for split PDU session. | YES | reject |
| Data Forwarding Not Possible | O |  | 9.3.1.63 | This IE may be present in case of HANDOVER REQUEST message and is ignored otherwise. | YES | reject |
| PDU Session Type | M |  | 9.3.1.52 |  | YES | reject |
| Security Indication | O |  | 9.3.1.27 |  | YES | reject |
| Network Instance | O |  | 9.3.1.113 | This IE is ignored if the Common Network Instance IE is included. | YES | reject |
| QoS Flow Setup Request List |  | 1 |  |  | YES | reject |
| >QoS Flow Setup Request Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  | - |  |
| >>E-RAB ID | O |  | 9.3.2.3 |  | - |  |
| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| >>Redundant QoS Flow Indicator | O |  | 9.3.1.134 | This IE indicates whether this QoS flow is requested for the redundant transmission. | YES | ignore |
| >>MBS QoS Flow Identifier | O |  | 9.3.1.51 | Indicates the MBS QoS Flow Identifier | YES | ignore |
| >>MBS Session ID | O |  | 9.3.1.x |  | YES | ignore |
| Common Network Instance | O |  | 9.3.1.120 |  | YES | ignore |
| Direct Forwarding Path Availability | O |  | 9.3.1.64 | This IE may be present in case of inter-system handover and is ignored otherwise. | YES | ignore |
| //skip the unchanged part |  |  |  |  |  |  |

-----------------Start of the next Change-------------------

9.3.4.2	PDU Session Resource Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL QoS Flow per TNL Information | M |  | QoS Flow per TNL Information 9.3.2.8 | NG-RAN node endpoint of the NG-U transport bearer for delivery of DL PDUs, together with associated QoS flows. | - |  |
| Additional DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of DL PDUs for split PDU session, together with associated QoS flows and corresponding to the Additional UL NG-U UP TNL Information IE in the PDU Session Resource Setup Request Transfer IE. | - |  |
| Security Result | O |  | 9.3.1.59 |  | - |  |
| QoS Flow Failed to Setup List | O |  | QoS Flow List with Cause 9.3.1.13 |  | - |  |
| Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information 9.3.2.8 | NG-RAN node endpoint of the NG-U transport bearer(s) for delivery of DL PDUs of the indicated Redundant QoS Flow(s) and corresponding to the Redundant UL NG-U UP TNL Information IE in the PDU Session Resource Setup Request Transfer IE. | YES | ignore |
| Additional Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of redundant DL PDUs for split PDU session, together with associated QoS flows and corresponding to the Additional Redundant UL NG-U UP TNL Information IE in the PDU Session Resource Setup Request Transfer IE. | YES | ignore |
| Used RSN Information | O |  | Redundant PDU Session Information 9.3.1.136 |  | YES | ignore |
| Global RAN Node ID of Secondary NG-RAN Node | O |  | Global RAN Node ID 9.3.1.5 |  | YES | ignore |
| MBS Supported | O |  | 9.3.1.y |  | YES | ignore |

-----------------Start of the next Change-------------------

9.3.4.3	PDU Session Resource Modify Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | 9.3.1.102 |  | YES | reject |
| UL NG-U UP TNL Modify List |  | 0..1 |  |  | YES | reject |
| >UL NG-U UP TNL Modify Item |  | 1..<maxnoofMultiConnectivity> |  |  | - |  |
| //skip the unchanged part |  |  |  |  |  |  |
| Network Instance | O |  | 9.3.1.113 | This IE is ignored if the Common Network Instance IE is included. | YES | reject |
| QoS Flow Add or Modify Request List |  | 0..1 |  |  | YES | reject |
| >QoS Flow Add or Modify Request Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>QoS Flow Level QoS Parameters | O |  | 9.3.1.12 |  | - |  |
| >>E-RAB ID | O |  | 9.3.2.3 |  | - |  |
| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| >>Redundant QoS Flow Indicator | O |  | 9.3.1.134 | This IE indicates whether this QoS flow is requested for the redundant transmission. | YES | ignore |
| >>MBS QoS Flow Identifier | O |  | 9.3.1.51 | Indicates the MBS QoS Flow Identifier | YES | ignore |
| >>MBS Session ID | O |  | 9.3.1.x |  | YES | ignore |
| QoS Flow to Release List | O |  | QoS Flow List with Cause 9.3.1.13 |  | YES | reject |
| //skip the unchanged part |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofMultiConnectivity | Maximum no. of connectivity allowed for a UE. Value is 4. The current version of the specification supports up to 2 connectivity. |

-----------------Start of the next Change-------------------

9.3.4.4	PDU Session Resource Modify Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. | - |  |
| UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the 5GC node. | - |  |
| //skip the unchanged part |  |  |  |  |  |  |
| Redundant DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs for the redundant transmission. | YES | ignore |
| Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the 5GC node for the redundant transmission. | YES | ignore |
| Additional Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of redundant DL PDUs for split PDU session, together with associated QoS flows. | YES | ignore |
| Additional Redundant NG-U UP TNL Information | O |  | UP Transport Layer Information Pair List 9.3.2.11 | NG-RAN node endpoint of the NG-U transport bearer for delivery of redundant DL PDUs corresponding to the modified UPF endpoint(s) received in the UL NG-U UP TNL Modify List IE of the PDU Session Resource Modify Request Transfer IE in case of PDU session split. | YES | ignore |
| MBS Supported | O |  | 9.3.1.y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

-----------------Start of the next Change-------------------

9.3.4.a	Multicast Distribution Setup Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| DL Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. | YES | reject |

9.3.4.b	Multicast Distribution Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| IP Multicast Address Information |  | 0..1 |  |  | YES | reject |
| >IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  | - | - |
| >IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  | - | - |
| >GTP DL TEID | M |  | GTP-TEID 9.3.2.5 |  | - | - |

9.3.4.c	Multicast Distribution Setup Unsuccessful Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

9.3.4.d	Multicast Distribution Release Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |
| Cause | M |  | 9.3.1.2 |  |

9.3.4.e	Multicast Distribution Release Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.x |  |

-----------------End of the Changes-------------------