3GPP TSG-RAN WG3 Meeting #112-e	R3-212464

- 17-28 May 2021

- Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0627 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS38.423 on Resource Status Reporting for RAN Sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, China Telecom, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For RAN sharing scenario, the load reporting among neighbouring shared gNBs has to be per PLMN granularity for MLB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resource Status Request message and Resource Status Update message is enhanced per-PLMN basis. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact on the Resource Status Reporting over Xn interface. This CR has impact on the ASN.1 with backward compatible manner. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of Resource Status Reporting can not work properly in the case of RAN sharing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.10.2; 9.1.3.18; 9.1.3.22; 9.3.5; 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of  the first Change >>>>>>>>>>>>>>>>>>>>

8.4.10	Resource Status Reporting Initiation

8.4.10.1	General

This procedure is used by an NG-RAN node to request the reporting of load measurements to another NG-RAN node.

The procedure uses non UE-associated signalling.

8.4.10.2	Successful Operation

Figure 8.4.10.2-1: Resource Status Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the RESOURCE STATUS REQUEST message to NG-RAN node2 to start a measurement, stop a measurement or add cells to report for a measurement. Upon receipt, NG-RAN node2:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the RESOURCE STATUS REQUEST message and the Report Characteristics IE indicates cell specific measurements, the Cell To Report List IE shall be included.

If Registration Request IE is set to "add" in the RESOURCE STATUS REQUEST message, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all requested resource status information, it shall initiate the measurement as requested by NG-RAN node1 and respond with the RESOURCE STATUS RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects NG-RAN node2 shall perform measurements on. For each cell, NG-RAN node2 shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1". If NG-RAN node2 is a gNB and if the cell for which Radio Resource Status IE is requested to be reported supports more than one SSB, the Radio Resource Status IE for such cell shall include the SSB Area Radio Resource Status Item IE for all SSB areas supported by the cell. If the SSB To Report List IE is included for a cell, the Radio Resource Status IE for such cell shall include the requested SSB Area Radio Resource Status List IE;

-	the TNL Capacity Indicator IE, if the second bit, "TNL Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1". The received TNL Capacity Indicator IE represents the lowest TNL capacity available for the cell.

-	the Composite Available Capacity Group IE, if the third bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1". If the Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. If NG-RAN node2 is a gNB and if the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one SSB, the Composite Available Capacity Group IE for such cell shall include the SSB Area Capacity Value List for all SSB areas supported by the cell, providing the SSB area capacity with respect to the Cell Capacity Class Value. If the SSB To Report List IE is included for a cell, the Composite Available Capacity Group IE for such cell shall include the requested SSB Area Capacity Value List IE.

If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Slice Available Capacity IE for such cell shall include the requested Slice Available Capacity Value Downlink IE and Slice Available Capacity Value Uplink IE, providing the slice capacity with respect to the Cell Capacity Class Value.

- 	the Number of Active UEs IE, if the fourth bit, "Number of Active UEs" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1";

-	the RRC Connections IE, if the fifth bit, "RRC Connections" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

If the Reporting Periodicity IE in the RESOURCE STATUS REQUEST is present, this indicates the periodicity for the reporting of periodic measurements. the NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

If the PLMN To Report List IE is included in the Cell To Report List IE, NG-RAN node 2 shall, if supported, report the results of the admitted measurements in RESOURCE STATUS UPDATE message per PLMN granularity for the concerned cell.

<<<<<<<<<<<<<<<<<<<<< End of the First Change >>>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of the Second Change >>>>>>>>>>>>>>>>>>>>

9.1.3.18	RESOURCE STATUS REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested measurement according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the NG-RAN node2 is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit =Number of Active UEs, Fifth Bit =RRC connections. Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>PLMN To Report List |  | 0..1 |  | PLMN list to which the request applies. |  |  |
| >>>PLMN To Report Item |  | 1 .. < maxnoofBPLMNs> |  |  |  |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 |  |  |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | – |  |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB-Index | M |  | INTEGER (0..,63..) |  | – |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | – |  |
| >>>Slice To Report Item |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.3.1.14 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | S-NSSAI 9.3.1.38 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of PRB Periodic, TNL Capacity Ind Periodic, Composite Available Capacity Periodic. Also used as the averaging window length for all measurement object if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |

<<<<<<<<<<<<<<<<<<<< End of the Second Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of the Third Change >>>>>>>>>>>>>>>>>>>>

9.1.3.21	RESOURCE STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>TNL Capacity Indicator | O |  | 9.2.2.49 |  | – |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.51 |  | – |  |
| >>Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| >>Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >> RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>Requested PLMN Identity Info List NR |  | 0..<maxnoofBPLMNs> |  |  | YES | ignore |
| >>>Request PLMNs |  | 1..<maxnoofBPLMNs> |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| >>>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>>TNL Capacity Indicator | O |  | 9.2.2.49 |  | – |  |
| >>>Composite Available Capacity Group | O |  | 9.2.2.51 |  | – |  |
| >>>Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| >>>Number of Active UEs | O |  | 9.2.2.62 |  | – |  |
| >>>RRC Connections | O |  | 9.2.2.56 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |

<<<<<<<<<<<<<<<<<<<< End of the Third Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of the Fourth Change >>>>>>>>>>>>>>>>>>>>

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

id-DLCarrierList,

id-EndpointIPAddressAndPort,

id-ExtendedTAISliceSupportList,

id-FiveGCMobilityRestrictionListContainer,

id-SecondarydataForwardingInfoFromTarget-List,

id-LastE-UTRANPLMNIdentity,

id-IntendedTDD-DL-ULConfiguration-NR,

id-MaxIPrate-DL,

id-SecurityResult,

id-OldQoSFlowMap-ULendmarkerexpected,

id-PDUSessionCommonNetworkInstance,

id-BPLMN-ID-Info-EUTRA,

id-BPLMN-ID-Info-NR,

id-DRBsNotAdmittedSetupModifyList,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-ULForwardingProposal,

id-DRB-IDs-takenintouse,

id-SplitSessionIndicator,

id-NonGBRResources-Offered,

id-MDT-Configuration,

id-TraceCollectionEntityURI,

id-NPN-Broadcast-Information,

id-NPNPagingAssistanceInformation,

id-NPNMobilityInformation,

id-NPN-Support,

id-LTEUESidelinkAggregateMaximumBitRate,

id-NRUESidelinkAggregateMaximumBitRate,

id-ExtendedRATRestrictionInformation,

id-QoSMonitoringRequest,

id-QoSMonitoringDisabled,

id-QosMonitoringReportingFrequency,

id-DAPSRequestInfo,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-NBIoT-UL-DL-AlignmentOffset,

id-TDDULDLConfigurationCommonNR,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRCellPRACHConfig,

id-Redundant-UL-NG-U-TNLatUPF,

id-Redundant-DL-NG-U-TNLatNG-RAN,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-Additional-Redundant-UL-NG-U-TNLatUPF-List,

id-RedundantCommonNetworkInstance,

id-TSCTrafficCharacteristics,

id-RedundantQoSFlowIndicator,

id-Additional-PDCP-Duplication-TNL-List,

id-RedundantPDUSessionInformation,

id-UsedRSNInformation,

id-RLCDuplicationInformation,

id-CSI-RSTransmissionIndication,

id-UERadioCapabilityID,

id-secondary-SN-UL-PDCP-UP-TNLInfo,

id-pdcpDuplicationConfiguration,

id-duplicationActivation,

id-NPRACHConfiguration,

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-SFN-Offset,

id-Requested-PLMN-ID-Info-NR,

<<<<<<<<<<<<<<<<<<<< End of the Fourth Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of the Fifth Change >>>>>>>>>>>>>>>>>>>>

CellMeasurementResult ::= SEQUENCE (SIZE(1..maxnoofCellsinNG-RANnode)) OF CellMeasurementResult-Item

CellMeasurementResult-Item	::= SEQUENCE {

cell-ID								GlobalNG-RANCell-ID,

radioResourceStatus					RadioResourceStatus              OPTIONAL,

tNLCapacityIndicator					TNLCapacityIndicator          	 OPTIONAL,

compositeAvailableCapacityGroup  	CompositeAvailableCapacityGroup  OPTIONAL,

sliceAvailableCapacity          	 	SliceAvailableCapacity           OPTIONAL,

numberofActiveUEs                	NumberofActiveUEs                OPTIONAL,

rRCConnections                   	RRCConnections                   OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CellMeasurementResult-Item-ExtIEs} }	OPTIONAL,

...

}

CellMeasurementResult-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-Requested-PLMN-ID-Info-NR				CRITICALITY ignore	EXTENSION Requested-PLMN-ID-Info-NR						PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< End of the Fifth Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of the Sixth Change >>>>>>>>>>>>>>>>>>>>

Requested-PLMN-ID-Info-NR ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF Requested-PLMN-ID-Info-NR-Item

Requested-PLMN-ID-Info-NR-Item ::= SEQUENCE {

requestedPLMNs					BroadcastPLMNs,

radioResourceStatus					RadioResourceStatus              OPTIONAL,

tNLCapacityIndicator					TNLCapacityIndicator          	 OPTIONAL,

compositeAvailableCapacityGroup  	CompositeAvailableCapacityGroup  OPTIONAL,

sliceAvailableCapacity          	 	SliceAvailableCapacity           OPTIONAL,

numberofActiveUEs                	NumberofActiveUEs                OPTIONAL,

rRCConnections                   	RRCConnections                   OPTIONAL,

iE-Extension					ProtocolExtensionContainer { {Requested-PLMN-ID-Info-NR-Item-ExtIEs} } OPTIONAL,

...

}

Requested-PLMN-ID-Info-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of the Sixth Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of the Seventh Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-ActivatedServedCells										ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation							ProtocolIE-ID ::= 1

id-admittedSplitSRB											ProtocolIE-ID ::= 2

id-admittedSplitSRBrelease									ProtocolIE-ID ::= 3

id-AMF-Region-Information									ProtocolIE-ID ::= 4

id-AssistanceDataForRANPaging								ProtocolIE-ID ::= 5

id-BearersSubjectToCounterCheck								ProtocolIE-ID ::= 6

id-Cause												    ProtocolIE-ID ::= 7

id-cellAssistanceInfo-NR									ProtocolIE-ID ::= 8

id-ConfigurationUpdateInitiatingNodeChoice					ProtocolIE-ID ::= 9

id-CriticalityDiagnostics								    ProtocolIE-ID ::= 10

id-XnUAddressInfoperPDUSession-List						    ProtocolIE-ID ::= 11

id-DRBsSubjectToStatusTransfer-List						    ProtocolIE-ID ::= 12

id-ExpectedUEBehaviour										ProtocolIE-ID ::= 13

id-GlobalNG-RAN-node-ID										ProtocolIE-ID ::= 14

id-GUAMI													ProtocolIE-ID ::= 15

id-indexToRatFrequSelectionPriority							ProtocolIE-ID ::= 16

id-initiatingNodeType-ResourceCoordRequest				    ProtocolIE-ID ::= 17

id-List-of-served-cells-E-UTRA							    ProtocolIE-ID ::= 18

id-List-of-served-cells-NR									ProtocolIE-ID ::= 19

id-LocationReportingInformation								ProtocolIE-ID ::= 20

id-MAC-I													ProtocolIE-ID ::= 21

id-MaskedIMEISV												ProtocolIE-ID ::= 22

id-M-NG-RANnodeUEXnAPID										ProtocolIE-ID ::= 23

id-MN-to-SN-Container										ProtocolIE-ID ::= 24

id-MobilityRestrictionList									ProtocolIE-ID ::= 25

id-new-NG-RAN-Cell-Identity									ProtocolIE-ID ::= 26

id-newNG-RANnodeUEXnAPID									ProtocolIE-ID ::= 27

id-UEReportRRCTransfer										ProtocolIE-ID ::= 28

id-oldNG-RANnodeUEXnAPID									ProtocolIE-ID ::= 29

id-OldtoNewNG-RANnodeResumeContainer						ProtocolIE-ID ::= 30

id-PagingDRX												ProtocolIE-ID ::= 31

id-PCellID												    ProtocolIE-ID ::= 32

id-PDCPChangeIndication										ProtocolIE-ID ::= 33

id-PDUSessionAdmittedAddedAddReqAck							ProtocolIE-ID ::= 34

id-PDUSessionAdmittedModSNModConfirm						ProtocolIE-ID ::= 35

id-PDUSessionAdmitted-SNModResponse							ProtocolIE-ID ::= 36

id-PDUSessionNotAdmittedAddReqAck							ProtocolIE-ID ::= 37

id-PDUSessionNotAdmitted-SNModResponse						ProtocolIE-ID ::= 38

id-PDUSessionReleasedList-RelConf							ProtocolIE-ID ::= 39

id-PDUSessionReleasedSNModConfirm							ProtocolIE-ID ::= 40

id-PDUSessionResourcesActivityNotifyList					ProtocolIE-ID ::= 41

id-PDUSessionResourcesAdmitted-List							ProtocolIE-ID ::= 42

id-PDUSessionResourcesNotAdmitted-List					    ProtocolIE-ID ::= 43

id-PDUSessionResourcesNotifyList						    ProtocolIE-ID ::= 44

id-PDUSession-SNChangeConfirm-List							ProtocolIE-ID ::= 45

id-PDUSession-SNChangeRequired-List							ProtocolIE-ID ::= 46

id-PDUSessionToBeAddedAddReq								ProtocolIE-ID ::= 47

id-PDUSessionToBeModifiedSNModRequired						ProtocolIE-ID ::= 48

id-PDUSessionToBeReleasedList-RelRqd						ProtocolIE-ID ::= 49

id-PDUSessionToBeReleased-RelReq							ProtocolIE-ID ::= 50

id-PDUSessionToBeReleasedSNModRequired						ProtocolIE-ID ::= 51

id-RANPagingArea											ProtocolIE-ID ::= 52

id-PagingPriority										    ProtocolIE-ID ::= 53

id-requestedSplitSRB										ProtocolIE-ID ::= 54

id-requestedSplitSRBrelease									ProtocolIE-ID ::= 55

id-ResetRequestTypeInfo									    ProtocolIE-ID ::= 56

id-ResetResponseTypeInfo									ProtocolIE-ID ::= 57

id-RespondingNodeTypeConfigUpdateAck						ProtocolIE-ID ::= 58

id-respondingNodeType-ResourceCoordResponse					ProtocolIE-ID ::= 59

id-ResponseInfo-ReconfCompl									ProtocolIE-ID ::= 60

id-RRCConfigIndication										ProtocolIE-ID ::= 61

id-RRCResumeCause											ProtocolIE-ID ::= 62

id-SCGConfigurationQuery									ProtocolIE-ID ::= 63

id-selectedPLMN												ProtocolIE-ID ::= 64

id-ServedCellsToActivate									ProtocolIE-ID ::= 65

id-servedCellsToUpdate-E-UTRA							    ProtocolIE-ID ::= 66

id-ServedCellsToUpdateInitiatingNodeChoice				    ProtocolIE-ID ::= 67

id-servedCellsToUpdate-NR									ProtocolIE-ID ::= 68

id-s-ng-RANnode-SecurityKey									ProtocolIE-ID ::= 69

id-S-NG-RANnodeUE-AMBR										ProtocolIE-ID ::= 70

id-S-NG-RANnodeUEXnAPID										ProtocolIE-ID ::= 71

id-SN-to-MN-Container										ProtocolIE-ID ::= 72

id-sourceNG-RANnodeUEXnAPID								    ProtocolIE-ID ::= 73

id-SplitSRB-RRCTransfer									    ProtocolIE-ID ::= 74

id-TAISupport-list										    ProtocolIE-ID ::= 75

id-TimeToWait												ProtocolIE-ID ::= 76

id-Target2SourceNG-RANnodeTranspContainer					ProtocolIE-ID ::= 77

id-targetCellGlobalID									    ProtocolIE-ID ::= 78

id-targetNG-RANnodeUEXnAPID									ProtocolIE-ID ::= 79

id-target-S-NG-RANnodeID									ProtocolIE-ID ::= 80

id-TraceActivation											ProtocolIE-ID ::= 81

id-UEContextID										   	    ProtocolIE-ID ::= 82

id-UEContextInfoHORequest									ProtocolIE-ID ::= 83

id-UEContextInfoRetrUECtxtResp								ProtocolIE-ID ::= 84

id-UEContextInfo-SNModRequest								ProtocolIE-ID ::= 85

id-UEContextKeptIndicator									ProtocolIE-ID ::= 86

id-UEContextRefAtSN-HORequest								ProtocolIE-ID ::= 87

id-UEHistoryInformation										ProtocolIE-ID ::= 88

id-UEIdentityIndexValue										ProtocolIE-ID ::= 89

id-UERANPagingIdentity										ProtocolIE-ID ::= 90

id-UESecurityCapabilities									ProtocolIE-ID ::= 91

id-UserPlaneTrafficActivityReport							ProtocolIE-ID ::= 92

id-XnRemovalThreshold 										ProtocolIE-ID ::= 93

id-DesiredActNotificationLevel							    ProtocolIE-ID ::= 94

id-AvailableDRBIDs 											ProtocolIE-ID ::= 95

id-AdditionalDRBIDs 									    ProtocolIE-ID ::= 96

id-SpareDRBIDs 											    ProtocolIE-ID ::= 97

id-RequiredNumberOfDRBIDs 									ProtocolIE-ID ::= 98

id-TNLA-To-Add-List											ProtocolIE-ID ::= 99

id-TNLA-To-Update-List									    ProtocolIE-ID ::= 100

id-TNLA-To-Remove-List										ProtocolIE-ID ::= 101

id-TNLA-Setup-List											ProtocolIE-ID ::= 102

id-TNLA-Failed-To-Setup-List								ProtocolIE-ID ::= 103

id-PDUSessionToBeReleased-RelReqAck							ProtocolIE-ID ::= 104

id-S-NG-RANnodeMaxIPDataRate-UL								ProtocolIE-ID ::= 105

id-PDUSessionResourceSecondaryRATUsageList					ProtocolIE-ID ::= 107

id-Additional-UL-NG-U-TNLatUPF-List							ProtocolIE-ID ::= 108

id-SecondarydataForwardingInfoFromTarget-List				ProtocolIE-ID ::= 109

id-LocationInformationSNReporting							ProtocolIE-ID ::= 110

id-LocationInformationSN									ProtocolIE-ID ::= 111

id-LastE-UTRANPLMNIdentity								    ProtocolIE-ID ::= 112

id-S-NG-RANnodeMaxIPDataRate-DL								ProtocolIE-ID ::= 113

id-MaxIPrate-DL												ProtocolIE-ID ::= 114

id-SecurityResult											ProtocolIE-ID ::= 115

id-S-NSSAI													ProtocolIE-ID ::= 116

id-MR-DC-ResourceCoordinationInfo							ProtocolIE-ID ::= 117

id-AMF-Region-Information-To-Add							ProtocolIE-ID ::= 118

id-AMF-Region-Information-To-Delete							ProtocolIE-ID ::= 119

id-OldQoSFlowMap-ULendmarkerexpected						ProtocolIE-ID ::= 120

id-RANPagingFailure											ProtocolIE-ID ::= 121

id-UERadioCapabilityForPaging								ProtocolIE-ID ::= 122

id-PDUSessionDataForwarding-SNModResponse					ProtocolIE-ID ::= 123

id-DRBsNotAdmittedSetupModifyList							ProtocolIE-ID ::= 124

id-Secondary-MN-Xn-U-TNLInfoatM								ProtocolIE-ID ::= 125

id-NE-DC-TDM-Pattern										ProtocolIE-ID ::= 126

id-PDUSessionCommonNetworkInstance							ProtocolIE-ID ::= 127

id-BPLMN-ID-Info-EUTRA										ProtocolIE-ID ::= 128

id-BPLMN-ID-Info-NR											ProtocolIE-ID ::= 129

id-InterfaceInstanceIndication							    ProtocolIE-ID ::= 130

id-S-NG-RANnode-Addition-Trigger-Ind					    ProtocolIE-ID ::= 131

id-DefaultDRB-Allowed										ProtocolIE-ID ::= 132

id-DRB-IDs-takenintouse										ProtocolIE-ID ::= 133

id-SplitSessionIndicator									ProtocolIE-ID ::= 134

id-CNTypeRestrictionsForEquivalent							ProtocolIE-ID ::= 135

id-CNTypeRestrictionsForServing								ProtocolIE-ID ::= 136

id-DRBs-transferred-to-MN									ProtocolIE-ID ::= 137

id-ULForwardingProposal										ProtocolIE-ID ::= 138

id-EndpointIPAddressAndPort 								ProtocolIE-ID ::= 139

id-IntendedTDD-DL-ULConfiguration-NR						ProtocolIE-ID ::= 140

id-TNLConfigurationInfo										ProtocolIE-ID ::= 141

id-PartialListIndicator-NR									ProtocolIE-ID ::= 142

id-MessageOversizeNotification								ProtocolIE-ID ::= 143

id-CellAndCapacityAssistanceInfo-NR							ProtocolIE-ID ::= 144

id-NG-RANTraceID											ProtocolIE-ID ::= 145

id-NonGBRResources-Offered									ProtocolIE-ID ::= 146

id-FastMCGRecoveryRRCTransfer-SN-to-MN					    ProtocolIE-ID ::= 147

id-RequestedFastMCGRecoveryViaSRB3							ProtocolIE-ID ::= 148

id-AvailableFastMCGRecoveryViaSRB3						    ProtocolIE-ID ::= 149

id-RequestedFastMCGRecoveryViaSRB3Release				    ProtocolIE-ID ::= 150

id-ReleaseFastMCGRecoveryViaSRB3						    ProtocolIE-ID ::= 151

id-FastMCGRecoveryRRCTransfer-MN-to-SN						ProtocolIE-ID ::= 152

id-ExtendedRATRestrictionInformation						ProtocolIE-ID ::= 153

id-QoSMonitoringRequest										ProtocolIE-ID ::= 154

id-FiveGCMobilityRestrictionListContainer					ProtocolIE-ID ::= 155

id-PartialListIndicator-EUTRA								ProtocolIE-ID ::= 156

id-CellAndCapacityAssistanceInfo-EUTRA						ProtocolIE-ID ::= 157

id-CHOinformation-Req										ProtocolIE-ID ::= 158

id-CHOinformation-Ack										ProtocolIE-ID ::= 159

id-targetCellsToCancel										ProtocolIE-ID ::= 160

id-requestedTargetCellGlobalID								ProtocolIE-ID ::= 161

id-procedureStage											ProtocolIE-ID ::= 162

id-DAPSRequestInfo											ProtocolIE-ID ::= 163

id-DAPSResponseInfo-List									ProtocolIE-ID ::= 164

id-CHO-MRDC-Indicator										ProtocolIE-ID ::= 165

id-OffsetOfNbiotChannelNumberToDL-EARFCN					ProtocolIE-ID ::= 166

id-OffsetOfNbiotChannelNumberToUL-EARFCN					ProtocolIE-ID ::= 167

id-NBIoT-UL-DL-AlignmentOffset								ProtocolIE-ID ::= 168

id-LTEV2XServicesAuthorized									ProtocolIE-ID ::= 169

id-NRV2XServicesAuthorized									ProtocolIE-ID ::= 170

id-LTEUESidelinkAggregateMaximumBitRate						ProtocolIE-ID ::= 171

id-NRUESidelinkAggregateMaximumBitRate					    ProtocolIE-ID ::= 172

id-PC5QoSParameters											ProtocolIE-ID ::= 173

id-AlternativeQoSParaSetList								ProtocolIE-ID ::= 174

id-CurrentQoSParaSetIndex									ProtocolIE-ID ::= 175

id-MobilityInformation 										ProtocolIE-ID ::= 176

id-InitiatingCondition-FailureIndication				    ProtocolIE-ID ::= 177

id-UEHistoryInformationFromTheUE							ProtocolIE-ID ::= 178

id-HandoverReportType										ProtocolIE-ID ::= 179

id-HandoverCause											ProtocolIE-ID ::= 180

id-SourceCellCGI										    ProtocolIE-ID ::= 181

id-TargetCellCGI											ProtocolIE-ID ::= 182

id-ReEstablishmentCellCGI									ProtocolIE-ID ::= 183

id-TargetCellinEUTRAN									    ProtocolIE-ID ::= 184

id-SourceCellCRNTI										    ProtocolIE-ID ::= 185

id-UERLFReportContainer										ProtocolIE-ID ::= 186

id-NGRAN-Node1-Measurement-ID								ProtocolIE-ID ::= 187

id-NGRAN-Node2-Measurement-ID								ProtocolIE-ID ::= 188

id-RegistrationRequest										ProtocolIE-ID ::= 189

id-ReportCharacteristics									ProtocolIE-ID ::= 190

id-CellToReport											    ProtocolIE-ID ::= 191

id-ReportingPeriodicity										ProtocolIE-ID ::= 192

id-CellMeasurementResult									ProtocolIE-ID ::= 193

id-NG-RANnode1CellID										ProtocolIE-ID ::= 194

id-NG-RANnode2CellID										ProtocolIE-ID ::= 195

id-NG-RANnode1MobilityParameters							ProtocolIE-ID ::= 196

id-NG-RANnode2ProposedMobilityParameters				    ProtocolIE-ID ::= 197

id-MobilityParametersModificationRange						ProtocolIE-ID ::= 198

id-TDDULDLConfigurationCommonNR								ProtocolIE-ID ::= 199

id-CarrierList												ProtocolIE-ID ::= 200

id-ULCarrierList											ProtocolIE-ID ::= 201

id-FrequencyShift7p5khz									    ProtocolIE-ID ::= 202

id-SSB-PositionsInBurst									    ProtocolIE-ID ::= 203

id-NRCellPRACHConfig										ProtocolIE-ID ::= 204

id-RACHReportInformation									ProtocolIE-ID ::= 205

id-IABNodeIndication										ProtocolIE-ID ::= 206

id-Redundant-UL-NG-U-TNLatUPF								ProtocolIE-ID ::= 207

id-CNPacketDelayBudgetDownlink								ProtocolIE-ID ::= 208

id-CNPacketDelayBudgetUplink								ProtocolIE-ID ::= 209

id-Additional-Redundant-UL-NG-U-TNLatUPF-List			    ProtocolIE-ID ::= 210

id-RedundantCommonNetworkInstance							ProtocolIE-ID ::= 211

id-TSCTrafficCharacteristics								ProtocolIE-ID ::= 212

id-RedundantQoSFlowIndicator								ProtocolIE-ID ::= 213

id-Redundant-DL-NG-U-TNLatNG-RAN							ProtocolIE-ID ::= 214

id-ExtendedPacketDelayBudget								ProtocolIE-ID ::= 215

id-Additional-PDCP-Duplication-TNL-List						ProtocolIE-ID ::= 216

id-RedundantPDUSessionInformation							ProtocolIE-ID ::= 217

id-UsedRSNInformation										ProtocolIE-ID ::= 218

id-RLCDuplicationInformation								ProtocolIE-ID ::= 219

id-NPN-Broadcast-Information								ProtocolIE-ID ::= 220

id-NPNPagingAssistanceInformation					        ProtocolIE-ID ::= 221

id-NPNMobilityInformation									ProtocolIE-ID ::= 222

id-NPN-Support												ProtocolIE-ID ::= 223

id-MDT-Configuration									    ProtocolIE-ID ::= 224

id-MDTPLMNList												ProtocolIE-ID ::= 225

id-TraceCollectionEntityURI									ProtocolIE-ID ::= 226

id-UERadioCapabilityID										ProtocolIE-ID ::= 227

id-CSI-RSTransmissionIndication								ProtocolIE-ID ::= 228

id-SNTriggered  											ProtocolIE-ID ::= 229

id-DLCarrierList											ProtocolIE-ID ::= 230

id-ExtendedTAISliceSupportList								ProtocolIE-ID ::= 231

id-cellAssistanceInfo-EUTRA									ProtocolIE-ID ::= 232

id-ConfiguredTACIndication									ProtocolIE-ID ::= 233

id-secondary-SN-UL-PDCP-UP-TNLInfo							ProtocolIE-ID ::= 234

id-pdcpDuplicationConfiguration							    ProtocolIE-ID ::= 235

id-duplicationActivation									ProtocolIE-ID ::= 236

id-NPRACHConfiguration										ProtocolIE-ID ::= 237

id-QosMonitoringReportingFrequency						    ProtocolIE-ID ::= 238

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated		    ProtocolIE-ID ::= 239

id-DL-scheduling-PDCCH-CCE-usage							ProtocolIE-ID ::= 240

id-UL-scheduling-PDCCH-CCE-usage							ProtocolIE-ID ::= 241

id-SFN-Offset												ProtocolIE-ID ::= 242

id-QoSMonitoringDisabled								    ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue							    ProtocolIE-ID ::= 244

id-Requested-PLMN-ID-Info-NR								 ProtocolIE-ID ::= xxx

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of the Seventh Change >>>>>>>>>>>>>>>>>>>>