3GPP TSG-RAN WG3 Meeting #112-e	R3-212591

E-meeting, 17 – 27 May, 2021

Agenda item:	15.2.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On per-slice QoE measurement

Document for:	Discussion and Decision

# 1	Introduction

Three different solutions for per-slice QoE measurement are described in TR 38.890, and the TR also indicates: "For management-based QoE, Slice Scope can be clarified in normative stage."

We here briefly provide some discussion on the potential impact of management-based QoE slice cope on the solution to be selected.

# 2	Discussion

On our side we believe that the aspect of roaming should additionally be further checked in normative phase:

The slice scope may consist of list of slice ids and slice id ranges. In case of management-based QMC activation the slice scope information corresponds to the scope of slices allowed by the CN. The gNB will select UEs having PDU sessions with slice ID within the slice scope information received from OAM. The slice scope information is also forwarded by the gNB to the UE, and the UE will use this information to select the appropriate application client session(s) for QoE measurements.

In case of signaling-based QMC activation, which may originate either in the H-PLMN or the V-PLMN (in case of roaming), the slice scope information corresponds to the scope of slices allowed by the CN or the scope of UE subscribed slices. The received slice scope information, within the QMC configuration information, may therefore be transparently forwarded by the gNB to the UE (not interpretable in the gNB).

In this case, mapping of slice scope information received for QMC configuration should take place in the UE. If the received slice scope information corresponds to slices to which the UE is subscribed, the UE maps the slice scope onto the corresponding allowed slice(s).

Proposal: The per-slice QoE measurement solution should support roaming users for which the slice scope may relate to HPLMN slices in case of signalling based activation.

# 3	Conclusion

Proposal: The per-slice QoE measurement solution should support roaming users for which the slice scope may relate to HPLMN slices in case of signalling based activation.