3GPP TSG-RAN WG3 #112-e	R3-212948

Online, May 17 – 27, 2021                                                                    (revision of R3-212656)

Agenda Item:	9.3.8.1

Source:	Qualcomm (moderator)

Title:	CB # 107_Rel-16Corr_IAB

Document for:	Discussion

# Introduction

CB: # 107_Rel-16Corr_IAB - Discuss if agreeable; revise as needed (QC - moderator) Summary of offline disc R3-212656

These four CRs seem rather straightforward. The moderator believes that we only need one Phase with deadline of Tuesday, May 25, 12:00:00 UTC.

# For the Chairman’s Notes

Propose the following:

Proposal 1: R3-212173 is agreed.

Proposal 2: Revision of R3-212410 in R3-212798 is agreed.

Proposal 3: R3-212411 is agreed.

Proposal 4: Revision of R3-212412 in R3-212799 is agreed.

# Discussion

## 3.1	R3-212173 Miscellaneous corrections on IAB in TS 38.473 (ZTE)

Summary of change:

- Add “ or a BH RLC channel.” in the field description of RLC mode IE to support configuring RLC mode for a BH RLC channel.

- Replace GRB with GBR in the following sentence.

NOTE:	For a BH RLC channel, the listed mandatory IEs and the GRB QoS Flow Information IE are applicable, where GBR QoS Flow Information IE may be present if BH RLC channel conveys the traffic belonging to a GRB QoS Flow.

Q1: Do you agree with this CR and/or have any comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| QCOM | Yes |  |
| Ericsson | OK |  |
| Samsung | Yes |  |
| ZTE | Yes |  |
| Huawei | Yes |  |

Summary:

Proposal 1: R3-212173 is agreed.

## 3.2	R3-212410  Correction on BH RLC CH configured for BAP control PDU to TS 38.473 (Huawei, CATT, Qualcomm , Lenovo, Motorola Mobility,Nokia, Nokia Shanghai Bell)

In section 8.3.4.2:

- Delete the description of “Otherwise, the gNB-DU shall consider that the configured BH RLC channel cannot be used to transmit BAP Control PDU” from the paragraph related to the receiving operation about the BAP Control PDU Channel IE in the BH RLC Channel to be Modified List IE .

Q2: Do you agree with this CR and/or have any comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| QCOM | Yes | The important point is that the IAB-node can select any BH RLC CH for BAP Control PDUs in case none is configured. |
| Ericsson | Hm… | The text says “if supported”. What does the “if supported” refer to? We understand the intention, but perhaps the whole paragraph needs rewriting since it does not reflect the QC interpretation above. Proposed edit: “If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Modified List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30]. Otherwise, if the BAP Control PDU Channel IE is not present, any available BH RLC channel can be used to transmit BAP Control PDUs.” |
| Samsung | Yes |  |
| ZTE | Yes |  |
| Huawei | Yes | The text added by Ericsson has been captured in BAP spec, so we are not sure we need to repeat it again in the F1AP spec. But we have no strong view, if companies agree, we can revise the CR to add the suggested text from Ericsson. |

Summary: Indeed, Ericsson’s addition is already in BAP spec.

Proposal 2: Revision of R3-212410 in R3-212798 is agreed.

## 3.3	R3-212411  Correction on gNB-DU Resource Configuration to TS 38.473 (Huawei, China Unicom)

In section 8.10.2.2:

- Delete the description of “If the Child-Node List IE is included in the GNB-DU RESOURCE CONFIGURATION message, the gNB-DU shall store the information therein for the child node(s) indicated by the gNB-CU UE F1AP ID IE and gNB-DU UE F1AP ID IE, in the cells(s) indicated by the NR CGI IE in the Child-Node Cells List IE.”.

- Add the description of “indicated by the gNB-CU UE F1AP ID IE and gNB-DU UE F1AP ID IE” after “for each child-node”, and add the description of “by the NR CGI IE” after “for each cell served by this child node indicated”.

Change the refered clause in TS38.213 to the correct one, i.e. the clause 14

Q3: Do you agree with this CR and/or have any comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| QCOM | Yes |  |
| Ericsson | OK |  |
| Samsung | Yes |  |
| ZTE | Yes |  |
| Huawei | Yes |  |

Summary:

Proposal 3: R3-212411 is agreed.

## 3.4	R3-212412  Correction on UL BH information configuration for DRBs  support CA based duplication to TS 38.473 (Huawei, Qualcomm, Lenovo, Motorola Mobility, Nokia, Nokia Shanghai Bell)

In Section 8.3.1.2, add “or the Additional PDCP Duplication TNL List IE” between “UL UP TNL Information to be setup List IE” and “for a DRB”.

In Section 8.3.4.2, add “or the Additional PDCP Duplication TNL List IE” between “UL UP TNL Information to be setup List IE” and “for a DRB”.

In Section 8.3.5.2, add “If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].”

In Section 9.2.2.1, add the “BH Information” IE to the “Additional PDCP Duplication TNL Items” of the “UE CONTEXT SETUP REQUEST” message.

In Section 9.2.2.7, add the “BH Information” IE to the “Additional PDCP Duplication TNL Items” of the “UE CONTEXT MODIFICATION REQUEST” message.

In Section 9.2.2.11, add the “BH Information” IE to the “UL UP TNL Information to Be Setup Item IEs” and the “Additional PDCP Duplication TNL Items” of the “UE CONTEXT MODIFICATION CONFIRM” messages.

In section 9.4.5, add the “BHInfo” in the “AdditionalPDCPDuplicationTNL-ItemExtIEs”

Q4: Do you agree with this CR and/or have any comments?

| Company | Yes/No | Comments |
| --- | --- | --- |
| QCOM | Yes |  |
| Ericsson | OK | A small remark: it seems like some rows in the tabulars are added without change marks activated. |
| Samsung | Yes |  |
| ZTE | Yes |  |
| Huawei | Yes | Would Ericsson indicate the change marks are missed in which rows? It will be much appreciated. |

Summary: Based on Ericsson’s remark, CR has been revised in R3-212799.

Proposal 4: Revision of R3-212412 in R3-212799 is agreed.

# References

| R3-212173 | Miscellaneous corrections on IAB in TS 38.473 (ZTE) | CR0763r, TS 38.473 v16.5.0, Rel-16, Cat. F |
| --- | --- | --- |
| R3-212410 | Correction on BH RLC CH configured for BAP control PDU (Huawei, CATT, Qualcomm , Lenovo, Motorola Mobility,Nokia, Nokia Shanghai Bell) | CR0770r, TS 38.473 v16.5.0, Rel-16, Cat. F |
| R3-212411 | Correction on gNB-DU Resource Configuration (Huawei, China Unicom) | CR0771r, TS 38.473 v16.5.0, Rel-16, Cat. F |
| R3-212412 | Correction on UL BH information configuration for DRBs support CA based duplication (Huawei, Qualcomm, Lenovo, Motorola Mobility, Nokia, Nokia Shanghai Bell) | CR0772r, TS 38.473 v16.5.0, Rel-16, Cat. F |