3GPP TSG-RAN WG3 #112-e	R3-212639

Online, 17 – 27 May, 2021

Agenda Item:	15.2.1.2

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion on CB #: NRQoE3-RANConfig

Document for:	Approval

Introduction

CB: # NRQoE3-RANConfig

- Detail infor of QoE configurations? E.g., Service Type, Container for application layer measurement configuration, QOE reference ID, Area Scope of QMC defined either at a cell/TA/TAI/PLMN level, Slice Scope...

- How to handle multiple simultaneous QoE configuration? Send an LS to RAN2 to define ad-hoc UE capabilities for supporting multiple QoE measurements?

- How to support per slice QoE measurements? Impact on configuration and report? Send LS to other groups with clarification questions, if any? Support roaming users for which the slice scope may relate to HPLMN slices in case of signalling based activation?

- How to support MR-DC case? QoE measurement configuration transfer is NOT supported from MN to SN?

- How to support QoE measurement handling at RAN overload ? Including a QoE paused indication?

- Capture agreements as TP for BL CRs, if agreeable

- List open issues for next meeting in the summary

(HW - moderator)

Summary of offline disc R3-212639

For the Chairman’s Notes

Propose the following:

Agreements:

Wait for the conclusion of CB#NRQoE2 to decide whether to reuse existing “Trace Activation IE” or introduce a new one .

Agree to introduce a new IE "QoE Reference", FFS whether it is per service type or not. (could be reflected this in an LS to SA5 which is part of CB#NRQoE2)

Agree to introduce a new IE "Measurement Collection Entity IP Address", FFS whether it is per service type or per "QoE Reference ID".

Agree to include slice info as explicit IE in the configuration message over NG, FFS whether it should be also included inside the transparent configuration container; FFS whether slice info should be signalled as an explicit IE in the configuration message and in the report message over radio interface.

Agree there is no need to transfer QoE measurement configuration from MN to SN.

Agree to introduce the following additional new IEs:

- a list of UE Application layer measurement configuration IE for each service type.

- inside each UE Application layer measurement configuration IE:

- Container.

- a numerated IE indicating service type (Streaming services, MTSI services, VR, MBMS, XR).

- Area scope (a list of cells/TA/TAI/PLMN).

- Slice scope (FFS a list of S-NSSAI).

Additional IEs are FFS

Agree to send an LS to CT1/SA4/SA5 on the mapping between service types and slice, to check if application layer is aware of the mapping between service types and slice or not (to further check if SA2 should be involved), LS is R3-212873

Agree to send LS to RAN2 on RAN3 agreements on QoE measurement configuration, to inform our agreements

Agree to revise R3-212520 in R3-212815 as BL CR to 38.413

To be continued:

Further details on how slice info should be reflected in the configuration info and report message.

Stage 3 details, e.g. whether Measurement Collection Entity IP Address and QoE reference ID is per service type or not, slice scope, etc.

Whether and how to support of roaming UEs.

Whether a pause indication is needed, as guidance from OAM/CN to RAN for handling in case of RAN overload.

Whether a prioritization mechanism of different service types or slices is needed for the RAN to pause or release ongoing QoE measurements in case of RAN overload.

Whether a prioritization mechanism is needed for UE to send to RAN pending QoE reports when RAN overload is solved.

Whether to introduce any or all of these criteria, including one or more time-based, one or more threshold-based and one or more event-based, as conditions for triggering/stopping the QoE measurement.

Discussion [if needed]

To reuse and update the existing Trace Activation IE (like we did in LTE), or to create a new IE NR QoE activation

This is overlapped with the CB: # NRQoE2-Activation_Deactivation, main reason is that this issue is a basic one and is the starting point for configuration, so companies just need to provide your views.

Q1: To reuse existing IE or to create a new IE?

| Company | To Reuse or to create a new one | Comment |
| --- | --- | --- |
| Samsung | Reuse Trace Activation IE | As we commented in CB QoE2, using Trace activation IE had already be agreed in the TR38.890 and also agreed by RAN2. |
| Huawei | Reuse | It is a simpler way with less spec impacts. |
| ZTE | Create a new IE | We expect the 5GC/OAM to be able to configure QOE measurements multiple times for the change of UE services. However, In legacy trace procedure, only one Trace Session per Trace Reference for a UE is allowed, limiting the ability of the RAN/OAM to flexibly configure QoE. As the discussion in our paper R3-212434, we propose to decouple the QoE function with the Trace Function. This is discussed in CB: # NRQoE2-Activation_Deactivation, but not related to detail configuration. |
| Nokia | Reuse | Similar to our comment in CB QoE2. |
| Qualcomm | Based on CB # NRQoE2 | Let’s use CB # NRQoE2 for this decision so as to have uniform agreement across CBs. |
| LG | Reuse |  |
| China Telecom |  | Agree with Qualcomm. The conclusion in CB#NRQOE2 can be input for this CB |
| Lenovo, Motorola Mobility | Based on CB # NRQoE2 |  |
| China Unicom | Reuse Trace Activation IE and also create a new IE | We support the QoE configuration can be included in the trace procedure, this procedure can used as a baseline, and a separate QoE configuration procedure can also be introduce, it can be used when there is an ongoing trace procedure |
| CATT | Reuse | Similar to our comment in CB QoE2. |
| CMCC | Reuse as the WA | Prefer to discuss in CB QOE2. |
| Ericsson | QoE CB#2 | Both the reuse and a new IE should be considered, due to the limitations imposed by the current Trace framework. |

If to reuse, is a new IE like “QoE Reference ID” needed or not

In LTE, there is only one ID “E-UTRAN Trace ID” which is shared by all the trace related tasks, include interface trace, MDT and QMC, which implies that if all the three tasks are configured at the same, there is only one ID to refer; in other words, we could also use different messages to configure different tasks with different Trace ID respectively.

Q2: is a new IE like “QoE Reference ID” needed or not if reusing existing IE?

| Company | A new IE needed or not? | Comment |
| --- | --- | --- |
| Samsung | New IE needed | To support multiple QMC |
| Huawei | Either way is ok | If a new IE “QoE Reference ID” is introduced, then there is no need to share Trace ID with e.g. MDT when MDT is also configured. |
| ZTE | QoE Reference ID | Since multiple simultaneous QoE measurement configurations shall be supported, we need to use a QoE reference ID to identify the specfic QoE configuration/job. |
| Nokia | May not be needed | Allocate TR/TRSR per QMC session would enable support of multiple QMC (needs coordination with SA5) |
| Qualcomm | QoE Reference ID | Needed to support multiple QMC |
| LG | New IE needed | To support multiple QMC. |
| China Telecom | QoE Reference ID | QoE Reference ID can be used to identify each QOE measurement configuration |
| Lenovo, Motorola Mobility | QoE Reference ID | To support multiple QMC. |
| China Unicom | A new IE needed | QoE reference ID is need, since multiple QoE measurement configuration need to be supported. |
| CATT | Depend on SA5 design | We should ask SA5 to design the rule Both trace reference and QoE reference are provided by OAM SA5 designed. If the trace include several kinds trace include MDT, QoE etc. at same time, the SA5 should design the reference ID for them. Either uses one or each one for each trace. If multiple QoE is configured, the multiple QoE reference should be designed. In this situation, how to handle the trace reference in the message should be studied further. |
| CMCC | QoE reference ID | To support multiple QMC. |
| Ericsson | A new one |  |

If a new IE like “QoE Reference ID” is needed in the same message, should it be service type specific if multi-service type is configured, or shared by multi-service types in one message?

Here the main scenario is that when multi-service type or multi-QoE measurement (even two QoE measurement for the same service type towards the same UE) is configured in one message at the same time, do we need to introduce different “QoE Reference ID” for each service, or it is shared by multi-service?

Q3: If “QoE Reference ID” is needed, should it be service type specific, or shared by multi-service?

| Company | “QoE Reference ID” is service type specific, or shared by multi-service, in one message | Comment |
| --- | --- | --- |
| Samsung | Not sure | We noticed that companies have different views, we think it’s not clear for RAN3 that whether one QoE reference is associated with one service type or multiple service types, since this Multiple QMC is the request from SA5 and QoE reference is defined by SA5, shall we ask them the relationship between the two, and then make decisions? |
| Huawei | Service type specific | There might be a case that two QoE measurements are requested at the same time towards a UE but service type are the same, but it should be a rare case. |
| ZTE | service type specific seems reasonable | We expect different “QoE Reference ID” is used for different QoE measurement. But it is uncertain whether we can configure more than one QoE measurement for the same service or only one QoE measurement is allowed for one service. To us, for one UE, it seems reasonable that there is only one QoE measurement for the same service. |
| Nokia | Service type specific | Different services are typically handled by different applications, so independent activation and deactivation would be the nominal case in our understanding. |
| Qualcomm | Per every entry of the QoE configuration list | A “QoE Reference ID” should be added per every entry of the QoE configuration list. This would keep it generic in case multiple QoE measurements can be configured for the same service type. Related open issues: - Whether UE can be configured with multiple QoE measurements for the same service type One use case is per-slice QoE (UE can be configured with different QoE configurations for different slices but the same service type). Although slice ID could be a distinguishing identifier here, but then does OAM signal the same “QoE Reference ID” for both slices? (ok to check this with SA5). RAN3 signaling should not be affected by SA5 response if we define a “QoE Reference ID” per entry of the list. - What should be the format of the “QoE Reference ID”? - RAN2 is discussing whether to use SA4 defined QoE Reference as ID or to use a simpler identifier. RAN3 should wait for RAN2 decision and follow the same An example of the QoE configuration list would be: [RAN2 RefID X, serviceType 1, App layer container with QoERefID 1, RAN2 RefID Y, serviceType 1, App layer container with QoERefID 2, RAN2 RefID Z, serviceType 2, App layer container with QoERefID 3] |
| LG | Not sure | It seems that one QoE measurement is allowed for one service type. But, we should check that this relation is correct to SA5. |
| China Telecom | Service type specific |  |
| Lenovo, Motorola Mobility | Service type specific | per service type should be allowed. However, as QC and Huawei said, it could be possible to have different QMC configuration for a same service type. It would be better wait for a moment before making a decision. |
| China Unicom | Per every QoE measurement | Support QoE reference ID configured based on service type, since different service type may have different QoE configuration. And the same service type may have different QoE configurations with unique QoE reference ID. |
| CATT | Not sure for one service or multiple service But should per configuration file | We should check with SA5 for this question. In SA5 spec the QoE reference is along with the QoE configuration file and service type. |
| CMCC | Service type specific | Our understanding is QoE ref ID can be set per service type per slice. |
| Ericsson | QoE reference ID per service type | We think that there can be up to one measurement configuration per service type. |

If to reuse, is a new IE like “Measurement Collection Entity IP Address” needed or not

In LTE, there is only one ID “Trace Collection Entity IP Address” which is shared by all the trace related tasks, include interface trace, MDT and QMC, which implies that if all the three tasks are configured at the same, there is only one collection IP address to refer; in other words, we could also use different messages to configure different tasks with different collection IP address respectively.

Q4: is a new IE like “Measurement Collection Entity IP Address” needed or not if reusing existing IE?

| Company | A new IE needed or not? | Comment |
| --- | --- | --- |
| Samsung | May be needed |  |
| Huawei | Either way is OK | If a new IP address IE is introduced, then MDT and QoE measurement are configured at the same time, they could be reported to different TCE/MCE identity. |
| ZTE | YES, MCE address is needed | The destination of the data collected by QoE function is not always TCE, which could be other collection entity such as MCE. In the TR38.890 and TS28.405, MCE entity is also used for QoE measurement collection. |
| Nokia | No strong view | SA5 should be consulted to check if streaming is also possible. |
| Qualcomm | OK | Okay to add a new IE for MCE in case of different collection entities. |
| LG | No strong view | Share view with Nokia. |
| China Telecom | No strong view | Agree with Nokia |
| Lenovo, Motorola Mobility | OK | Same view with ZTE and QC., MCE needs to be considered. |
| China Unicom | Yes | For the TCE/MCE deployment more flexible. |
| CATT | No strong view | Depend on the system design. If more than one consumer requests the QoE report, the multiple IP/TCE is needed. |
| CMCC | Check with SA5 | Till now we haven’t checked with SA5 on the relationship between TCE and MCE, which needs to be clarified also for alignment of MDT and QoE. |
| Ericsson | Yes to a separate MCE address | We cannot assume that TCE and MCE are one. And even if they are one, we cannot assume that the same IP address is to be used. |

If a new IE like “Measurement Collection Entity IP Address” is needed, in the same message, should it be service type specific if multi-service type is configured, or shared by multi-service types in one message

Here the main scenario is that when multi-service type or multi-QoE measurement is configured in one message at the same time, do we need to introduce different “Measurement Collection Entity IP Address” for each service, or it is shared by multi-service, or it is configured per QoE measurement?

Q5: If  “Measurement Collection Entity IP Address” is needed, should it be service type specific, or shared by multi-service?

| Company | “Measurement Collection Entity IP Address” is service type specific, or shared by multi-service. | Comment |
| --- | --- | --- |
| Samsung | QoE reference specific | In 28.405, it’s said that QoE reference is used to identify the QoE measurement collection job in the traffic nodes and in the measurement collection centre. So we think this entity IP address should be QoE reference specific. |
| Huawei | Either way is ok | Service type specific seems to be more flexible. |
| ZTE | Check with SA4? | From RAN3 point of view, one single Measurement Collection Entity seems enough. |
| Nokia | Service Type | It should be per QoE configuration |
| Qualcomm | Shared by multi-service types | Share the view with ZTE. Single MCE for all service types seem enough. MCE can do post processing to separate per-service type metrics if needed. Even in MDT, we don’t have different TCE IP address for different MDT types. |
| LG | Service type specific |  |
| China Telecom | Service Type |  |
| Lenovo, Motorola Mobility | Shared by multi-service types | It seems single IP address for MCE is sufficient. It is up to MCE to process the service type information. |
| China Unicom | Shared by multi-service types | Agree with ZTE and Qualcomm |
| CATT | QoE reference specific | Agree with Samsung |
| CMCC | Shared by multi-service types | Agree with ZTE. |
| Ericsson | MCE IP address per QoE configuration | We cannot assume that the same entity will be the recipient of QoE reports for all services. Even if this is agreed, nothing prevents us with indicating the same IP address in all instances of MCE IP Address IE. Example: a signalling based QoE job could be ordered by a customer care center due to that a customer has made a complaint. A management based QoE job can be for calculating a KPI. A QoE job (management or signalling based) can be for assurance of a service (automation of keeping a certain level of quality for a service). |

Configuration and report message if QoE measurement with Slice is requested

Here the issues are mainly about what should be configured over NG, Uu for QMC with slice and what should be included in the RRC message for QoE measurement report over Uu. According to the discussion papers, the main questions are:

- The first one is, if application layer is aware of the mapping between service types and slice. Assuming the slice info (a slice ID or a list of slice IDs?) is configured in parallel with the container, should it also be added into the container, for the configuration message over NG and Uu? (An LS maybe needed to send to SA4)

- Then for QoE measurement report, should slice ID be included as an explicit IE in the RRC message, or just leave it to application layer to include slice ID info the QoE measurement report container?

Companies please share your views in the following table and indicate your preferred configuration over NG and Uu.

Q6: if application layer is aware of the mapping between service types and slice or not?

| Company | Comments and views |
| --- | --- |
| Samsung | Q1: Firstly, the application is aware of the mapping between service type and slice, in TS 27.007, the S-NSSAI associated to the PDU session can be obtained by Application layer through AT command (10.1.23 PDP context read dynamic parameters +CGCONTRDP). The same as service type, slice scope should be finally checked in UE application layer to decide whether to start QoE measurement, no matter it’s transmitted in or out of the container. If we learn from “service type” IE in LTE QMC, slice ID can be out of the container, but finally it should be transmitted to the application layer. Q2: slice ID should be included in QoE report, but regarding it’s inside or outside of the report container, we have no strong view. Maybe it’s better to let application to do it. |
| Huawei | We think it is better to check with CT1, it is out of RAN3 scope |
| ZTE | We can check it with SA4. |
| Nokia | SA2 and/or SA4 should be asked |
| Qualcomm | Similar understanding as Samsung. No need to involve UE AS if application can do the slice scope check. Okay to check this with SA4. |
| LG | Need to check with SA4 and/or CT1. |
| China Telecom | Need to check with SA4/CT1 |
| Lenovo, Motorola Mobility | Check with SA4. |
| China Unicom | Need to check with SA4 and/or CT1. |
| CATT | Need to check with SA4 and/or CT1. |
| CMCC | 同上(Same as above). |
| Ericsson | Of course not, the slices are AS. The application has no idea about slices. |

Q7: should slice info also be added into the container, i.e. as part of the transparent container?

Moderator: here I assume slice info is anyway configured in parallel with the container (outside of the container), please companies give your views if think otherwise, and also provide your view on a slice ID or a list of slice IDs

| Company | Comments and views |
| --- | --- |
| Samsung | If it’s anyway configured in parallel with container, it can also be transmitted to the application layer, just the same as service type. |
| Huawei | Not needed. Since slice info is anyway a wireless conception, even application layer is aware of such info, it doesn’t bring additional benefits to application layer to include slice info when application layer configures the QoE measurements. |
| ZTE | The information in container shall be checked with SA4 |
| Nokia | RAN2 is discussing this topic. This has direct UE impact, so RAN3 should wait for the RAN2 outcome. |
| Qualcomm | Firstly, whether to add slice info outside the container (i.e. in RRC) is also dependent on Q6. If application is aware of slice-service mapping, there is no need to include slice info outside the container. Here are the steps we propose for QoE configuration: - Slice info is included by OAM both outside and inside the container and signaled to NG-RAN. - NG-RAN does the slice scope check and forwards the QoE configuration to UE with slice info included only inside the container - UE APP checks service-slice mapping and starts QoE measurements only if the mapping is correct. |
| LG | Wait for RAN2 outcome. |
| China Telecom | Wait for RAN2 outcome |
| Lenovo, Motorola Mobility | Similar view with Qualcomm. |
| China Unicom | Slice don’t need to be included in the transparent container. |
| CATT | Before answer this question, we should make one question clear. If the function of slice ID is equal to the Area scope. i.e. if the UE is not in this slice, the QoE will not be triggered. If Area scope liked slice function support, the slice ID/list slice ID should be outer container. So the RAN can select the qualified UE. If slice ID just used as post processing filter, we agree QC comments, if the application layer have this slice information, the slice id is in container is ok |
| CMCC | We haven’t discussed whether to transmit slice ID/slice scope outside the configuration container over Uu, but slice information will anyway need to be informed to UE APP layer. |
| Ericsson | No, the S-NSSAI (or a list thereof, to be discussed) should only be outside the container since the application layer is unaware of slices. In our view, the things should work as follows: To activate per-slice QoE measurements, NG-RAN receives S-NSSAI (FFS if one S-NSSAI or a list of) included in the QoE configuration. The received S-NSSAI are used to check if the QoE configuration can be sent to a UE, based on the S-NSSAIs associated to the PDU Sessions established for the UE. The S-NSSAI (FFS if one S-NSSAI or a list of) is included in the QoE configuration sent to the UE and in the corresponding QoE report(s) received from the UE. |

Q8: should slice ID be included as an explicit IE in the QoE measurement report RRC message, or just leave it to application layer to include slice ID info the container?

| Company | Comments and views |
| --- | --- |
| Samsung | Both are OK for us. No strong view |
| Huawei | We think slice ID info should be an explicit IE in the QoE measurement report RRC message. |
| ZTE | We prefer the slice ID is explicitly included in the RRC QoE report, so the RAN can be aware of the corresponding slice information of QoE report, and can pause QoE reporting of certain slice under overload. |
| Nokia | RAN2 is discussing this topic. This has direct UE impact, so RAN3 should wait for the RAN2 outcome. |
| Qualcomm | NG-RAN has the knowledge of the mapping between QoE Reference ID and slice ID from the QoE configuration received via OAM. NG-RAN can use this to pause QoE reporting of certain slices it needed. Don’t see why the slice ID has to be signaled in RRC QoE report when NG-RAN already would have the knowledge. This would be consistent with our proposal in Q7 (i.e. not include slice ID in RRC) |
| LG | Wait for RAN2 outcome. |
| China Telecom | Wait for RAN2 outcome |
| Lenovo, Motorola Mobility | Similar view with Qulcomm. |
| China Unicom | We prefer the slice ID is explicitly included in the RRC QoE report |
| CATT | Prefer slice ID in the report container. Looks it is not useful for RAN. Even you plan to pause the report, it just associated the configuration is ok |
| CMCC | For legacy QoE measurement, include slice info in the container is enough in our understanding. Since RAN3 is the leading group for per-slice QoE discussion, we prefer to have related discussions in RAN3. |
| Ericsson | Explicitly in the RRC message carrying the report, outside the container. |

There is another issue, which is about the support of roaming UE for which the slice scope may relate to HPLMN slices in case of signalling based activation. Please companies share your views.

Q9: Support of Roaming UEs

| Company | Comments and views |
| --- | --- |
| Samsung | If the QoE configuration is from the Home network, does home network really need or is home network allowed to collect QoE in the visited network slice? |
| Huawei | Not sure the intention, firstly do we need to configure home PLMN in the area scope when UE is camping on the visiting PLMN? And then, similar comment as SS. |
| ZTE | It is out of RAN3 scope. |
| Nokia | We believe this e.g. can have relevance for multi-PLMN operators where slices are configured per PLMN id (but still under same operator). Also this would also have relevance if RAN visible QoE metrics becomes needed for the NG-RAN to provide the required service level (QoE level). If companies prefer a solution which architecturally excludes support of roaming UEs, so that introduction of support for roaming UEs (e.g. in future releases) would be cumbersome or impossible, such choice should be checked with SA4 and probably also SA2. |
| China Unicom | Share the view with ZTE, Huawei and Samsung |
| CATT | We don’t think it is needed. The slice in VPLMN is mapped. |
| CMCC | Similar question as SS. Check with SA2 may be needed. |
| Ericsson | Not needed. Is this really an issue for the first meeting? Support for different services is a basic issue and should have been treated in this CB. |

MR-DC support

Here the main question is: whether QoE measurement configuration should be transferred from MN to SN or not?

Q10: whether QoE measurement configuration should be transferred from MN to SN or not?

| Company | Yes or No | Comment |
| --- | --- | --- |
| Samsung | No |  |
| Huawei | Not needed | Anyway, to UE application layer, whether DC is configured or not is invisible. |
| ZTE |  | We can discuss this topic later |
| Nokia | No | Not in Rel-17. |
| Qualcomm | No | No. If we decide to include QoE configuration inside the Trace Activation IE (section 3.1), then we need to clarify that QoE configuration should not be transferred to SN in S-NODE ADDITION REQUEST in this release or if included SN should ignore this IE etc. |
| LG | No |  |
| China Telecom | Can be discussed in next release | From our point of view, to support of QOE collection in MR-DC can be discussed in next release. |
| Lenovo, Motorola Mobility | No |  |
| China Unicom |  | Not in Rel-17. |
| CATT |  | Not in Rel-17 scope |
| CMCC | Not in R17 |  |
| Ericsson | Can be discussed in Rel-18. |  |

Overload handling

From the discussion papers, we could see that there are several proposals, the main intention is whether any guidance from OAM/CN is needed, including

- an explicit pause indication?

- the priorities of the QoE measurements of different service types or slices for the RAN to pause or release ongoing QoE measurements in case of RAN overload?

- prioritization mechanism for UE to send to RAN pending QoE reports when RAN overload is solved?

Please companies share your views to the three sub-questions.

Q11: over NG/ the interaction between RAN and OAM, a pause indication and/or priorities for RAN to pause or release, priorities for UE to send to RAN pending QoE reports when RAN overload is solved?

| Company | Comments and views |
| --- | --- |
| Samsung | At least we should have pause indication, priorities mechanism may need further discussion. |
| Huawei | For explicit pause indication, we think it is not needed, since it is RAN behavior whether to pause or not; For prioritization list as guidance for pause, it might be good for high layer to give some guidance; For prioritization mechanism for UE, we think it is up to UE implementation, since such reports should be sent anyway. |
| ZTE | The NG-RAN node may initiate the procedure to pause/resume some QoE measurement reporting by sending a list of QoE reference ID with explicit pause/resume indication to UE. We think that setting priorities is unnecessary and the RAN node could control which service reporting is to be suspended or resumed |
| Nokia | The overload handling should remain inside RAN. |
| Qualcomm | Share the view with Samsung. Pause indication is NG-RAN dependent and need not be signaled by OAM. And prioritization list can be implementation specific at NG-RAN as well i.e. NG-RAN can choose to pause QMC for video service first and keep QMC for voice service running. |
| LG | For an explicit pause indication and prioritization mechanism for UE, we have similar understanding as Huawei. |
| China Telecom | We support to introduce a new pause/resume indication. |
| Lenovo, Motorola Mobility | The explicit pause indication seems fine. FFS on others. |
| China Unicom | Explicit pause indication is not needed, since it is RAN behavior. Support priority configuration over NG/ the interaction between RAN and OAM, and RAN can used to pause or resume the QoE reports. |
| CATT | In 28.405 about the indication as below. Looks needed, should check with SA5 Step 1 (M) The RAN node sends a request to temporarily stop the reporting to the UEs that has started the QoE information collection. An indication about the temporary stop is sent to the management system. Step 2 (M) When the UE receives the request from the RAN node to temporarily stop reporting, the UE access stratum informs the application that reporting has temporarily been stopped. The application continues any ongoing recording and stores the recorded information until a restart request is received. Step 3 (M) When the RAN overload situation is ended, the RAN node sends a request to restart the reporting to the UEs that has temporarily stopped the QoE information reporting. An indication about the restart is sent to the management system. Step 4 (M) When the UE receives the request from the RAN node, the UE access stratum informs the application to restart the QoE information reporting. Ends when The management system has received the indication that a recording session has been restarted. Support priority configuration over NG/ the interaction between RAN and OAM, and RAN can used to pause or resume the QoE reports. |
| CMCC | Pause indication is needed over Uu, but not NG. |
| Samsung2 | Just to clarify the proposal, our initial proposal for overload handling is we need pause indication during mobility, the impact will be on Xn/NG for mobility related messages. If the QoE reporting is paused in the source node because of overload, the reporting configuration (i.e. pause indication) should be transmitted to the target node which may have no overload situation, so that the target node can resume the QoE reporting. We would like to know companies’ opinions on this proposal . |
| Ericsson | RAN has full autonomy wrt handling of pause/resume/release at overload, and no pause/resume/release indication over NG or from OAM is necessary. As for prioritization of pending report delivery after overload, RAN3 should discuss the possibility that the OAM indicates the prioritization among different service types. The pending reports of high prio services would be delivered first. |

Other parameters?

As could be seen from discussion papers, I could see consensus on the introduction of the following IE:

- Container

- UE Application layer measurement configuration IE for each service type

- a numerated IE indicating service type (Streaming services, MTSI services, VR, MBMS, XR)

- Area scope, could be a list of cells/TA/TAI/PLMN

- Slice scope, could be a list of S-NSSAI

Q12: Any comments to the four parameters above, e.g. yes or no, additional comments.

| Company | Comment |
| --- | --- |
| Samsung | A little confused, what’s the difference between “container” and “UE Application layer measurement configuration IE for each service type”? We think the container, service type, area scope and slice scope should be inside of the UE Application layer measurement configuration IE. And to support multiple QMC, a list of UE Application layer measurement configuration IE should be included in Trace Activation IE. |
| Huawei | We think all these parameters listed above are needed. To Samsung, agree, UE Application layer measurement configuration IE is an upper level IE, and there is a list of this IE. |
| ZTE | OK for above parameters. |
| Nokia | OK |
| Qualcomm | OK for all |
| LG | OK |
| China Telecom | Ok |
| Lenovo, Motorola Mobility | ok |
| China Unicom | Agree with Samsung, “We think the container, service type, area scope and slice scope should be inside of the UE Application layer measurement configuration IE. “ We think QoE reference ID should be introduced for each QoE measurement. |
| CATT | OK |
| CMCC | OK |
| Ericsson | The content should be the following, for each item of the list of QoE measurement configurations: - One container, transparent to RAN, containing application layer measurement configuration. - Outside the container, one MCE IP address. - Outside the container, one indication of the service type (e.g. MTSI or streaming). - Outside the container, one QoE reference, e.g. to be used for alignment of QoE reports and radio measurements. - Outside the container, S-NSSAI (FFS if one S-NSSAI or a list of) to be used as filtering criteria in case of per-slice QoE measurements. - Outside the container, one or more time-based criteria, to be used for triggering and/or stopping QoE measurements. - Outside the container, one or more threshold-based criteria, to be used for triggering and/or stopping QoE measurements. - Outside the container, one or more filtering criteria, including: (1) a list of cells; (2) a list of Tracking Areas; (3) a list of PLMNs. - Outside the container, one or more event-based criteria, to be used for triggering and/or stopping QoE measurements |

In addition, I could also see some proposals on introducing criteria from OAM/CN to RAN a set of criteria, including one or more time-based criteria, one or more threshold-based criteria and one or more event-based criteria, to be used for triggering and/or stopping QoE measurements. Please companies share your views whether any or all of these criteria are needed or not.

Q13. If any or all of these criteria, one or more time-based, one or more threshold-based, one or more event-based, are needed or not.

| Company | Comment |
| --- | --- |
| Samsung | It’s good to have some conditions for triggering/stopping the QoE measurement, it’s more flexible to collect QoE in different scenarios |
| Huawei | Not sure. Actually in the QMC container, application layer configured some criteria for the UE to conduct this QoE measurement task, from UE AS layer, it should just transfer to application layer when QMC is received. |
| ZTE | Not needed, it shall be done by implementation. |
| Qualcomm | Not needed, QMC can be deactivated by OAM whenever needed if any of these criteria is met. No need to signal this to NG-RAN or UE. |
| China Telecom | Not needed. QMC can trigger or stop QoE measurement based on its policy. There is no need to signal this criteria to NG-RAN. |
| Lenovo, Motorola Mobility | Seems not needed. these criteria can be transparent to RAN. |
| China Unicom | Not sure the benefits to support these criteria. Need for the clarification. |
| CATT | Not sure, should further study. These information looks inside configuration file |
| CMCC | Could be further studied. Maybe some MDT trigger is needed for alignment of MDT and QoE. |
| Ericsson | We think that the triggering criteria are essential – perhaps the most important motivation for QoE monitoring is to measure QoE during certain network or radio events. Besides, the TR 38.890 states the following in Clause 6.3: QoE measurement triggering and stopping can be realized using time-based and/or threshold-based criteria, configured by the OAM. Time-based QoE measurement triggering and stopping in NR is achieved by reusing mechanisms specified in LTE for the start and stop of QoE measurements. Threshold-based QoE measurement triggering and stopping allows to start and stop QoE measurement when given thresholds are passed. |

If to introduce a new IE NR QoE activation, what else needed

Here we should note that all the discussions and conclusions (except 3.1) above are also applied here for this issue, e.g. one QoE reference ID or not, one TCE address or not, slice configuration, etc.

Q14. If to introduce a new IE NR QoE activation, what else needed, in addition to the agreements reached above?

| Company | Comment |
| --- | --- |
| Samsung | I may not understand the question. Why we need a new IE NR QoE activation? What’s the issue? |
| Huawei | No. To SS, this question is based on the assumption that if a new NR QoE activation is introduced (since some companies proposed), then what else are needed, in addition to the agreements reached from Q2~13) |
| ZTE | We expect the 5GC/OAM to be able to configure QOE measurements multiple times for the change of UE services. However, in legacy trace procedure, only one Trace Session per Trace Reference for a UE is allowed, limiting the ability of the RAN/OAM to flexibly configure QoE. As discussed in our paper R3-212434, we propose to decouple the QoE function with the Trace Function. This is discussed in CB: # NRQoE2-Activation_Deactivation, but not related to detail configuration. For the detail configuration: In NGAP, QoE Activation IE could be included in INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST , HANDOVER REQUIRED and HANDOVER REQUEST message; In XnAP, QoE Activation IE could be included in HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE. QoE Activation IE includes the QoE configuration, in order to support multiple QoE function. The QoE configuration includes : 1: List of UE Application layer measurement configuration 2: MCE Address (new IE, shared by multi-service) 3: MDT Trace ID ( used for the correlation between MDT and QoE Measurement, see our paper R3-212449) Where each UE Application layer measurement configuration IE in the list further contains a container and other parameters out of the container: 1: QoE Reference ID (new IE, service type specific) 2: Area scope (could be a list of cells/TA/TAI/PLMN) 3: Service type (Streaming services, MTSI services, VR, MBMS, XR) 4: Slice scope (could be a list of S-NSSAI) 5: Container for application layer measurement configuration In the case of overload, the NG-RAN node may initiate the procedure to pause/resume some QoE measurement reporting by sending a list of QoE reference ID with explicit pause/resume indication to UE. |
| Qualcomm | We can look into this once there is a conclusion from CB_NRQoE2 |
| China Telecom | The conclusion in CB#NRQOE2 can be input for this CB |
| China Unicom | We support separate QoE configuration IE. |
| CATT | The RAN visible QoE may be considered later |
| CMCC | Wait for CB QoE2. |
| Ericsson | 1) The items listed in our answer to Q12. 2) Anything stemming from the conclusions of CB#2, especially wrt the limitations imposed by the Trace framework. 3) In our view, the support for multiple QoE measurements cannot be presumed for all UEs. This is because e.g. memory requirements to perform concurrent QoE measurements or buffering of QoE reporting might be supported by some UEs and not others. Therefore, we propose to introduce specific, ad-hoc UE capabilities. Such UE capabilities can, e.g. indicate a maximum number of concurrent QoE measurements that a UE can perform, or if QoE reports and MDT reports are supported simultaneously. RAN2 should be consulted to define the appropriate signalling. Proposal 2: Send an LS to RAN2 to define ad-hoc UE capabilities for supporting multiple QoE measurements. 4) We did not discuss which services are to be supported in NR QoE |

Send LS to other groups

[Moderator] I think pending on the final agreements we will reach, we may need send LS at least to RAN2, SA4 and SA5, we could leave this discussion to the phase II.

Conclusion, Recommendations [if needed]

If needed

References

R3-211732	NR QoE configuration and reporting	Qualcomm Incorporated

R3-211733	TP for 38.413 and 38.423) NR QoE configuration and reporting	Qualcomm Incorporated

R3-211734	Per slice QoE measurements	Qualcomm Incorporated

R3-211836	Discussion on NR QoE configuration details	CATT

R3-211837	Discussion on per-slice QoE measurement	CATT

R3-211986	QoE Configuration and Reporting	Ericsson

R3-212324	Principles for configuration of NR QoE measurements	Nokia, Nokia Shanghai Bell

R3-212381	CR TS 38.413 QoE Configuration and Reporting - Signalling Design	Ericsson

R3-212435	Discussion on NR QoE Configuration	ZTE Corporation, China Telecom

R3-212440	TP for TS 38.300) Introduce NR QoE	ZTE Corporation, China Telecom

R3-212441	Discussion on configuration details in NR standalone mode	LG Electronics

R3-212442	TP for TS 38.413) NR QOE configuration	ZTE Corporation, China Telecom

R3-212443	TP for TS 38.423) NR QOE configuration	ZTE Corporation, China Telecom

R3-212498	Per-slice QoE measurement	CMCC

R3-212518	Further analysis on spec impacts of the potential solutions to QoE measurement per slice	Huawei

R3-212521	Further analysis on spec impacts on RAN intervention of QoE measurement	Huawei

R3-212591	On per-slice QoE measurement	Nokia, Nokia Shanghai Bell

R3-211978	RAN3 impacts for supporting QoE Measurement Collection in NR	Samsung

R3-211979	Support of NR QoE Measurement Collection	Samsung

R3-211980	Support of NR QoE Measurement Collection	Samsung

R3-211835	Discussion on NR QoE configuration procedures	CATT

R3-211985	Procedures for Configuration, Activation and Deactivation of QMC	Ericsson

R3-212322	Baseline and stage 2 aspects for NR QoE	Nokia, Nokia Shanghai Bell

R3-212323	Introduction of QoE Measurement Collection for NR	Nokia, Nokia Shanghai Bell

R3-212434	Consideration on NR QoE activation procedure	ZTE Corporation, China Telecom, China Unicom

R3-212519	Further analysis on spec impacts on configuration and reporting	Huawei

R3-212520	CR to 38.413 on Introduction of QoE measurement	Huawei