3GPP TSG-RAN WG3 Meeting #112-e	R3-212824

E-meeting, 17-28 May 2021

Title:	LS on Area scope configuration and Frequency band info in MDT configuration

Response to:

Release:	Rel-17

Work Item:	NR_ENDC_SON_MDT_enh-Core

Source:	RAN3

To:	RAN2

Cc:

Contact person:	ZHANG Hongzhuo

zhanghongzhuo(at)huawei(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 discussed the configuration of Area Scope of Neighbour Cells for logged MDT and the NR Frequency Band in the Area Scope of Neighbour Cells. RAN3 would like to check if there is alignment between TS 38.413 and TS 38.331.

- Area Scope of Neighbour Cells for logged MDT

The Area Scope of Neighbour Cells was introduced for signalling based logged MDT in Rel-16 as an IE that does not depend on the presence of the Area Scope of MDT, e.g. see TS38.413. However, RAN3 observed that the Area Scope of Neighbour Cells cannot be configured to the UE if the Area Scope of MDT is configured as PLMN wide at NGAP level. The reason is that in TS 38.331, the areaConfiguration-r16 is optional, and the interFreqTargetList-r16 is encoded inside the areaConfiguration-r16.If the Area Scope of MDT is configured as PLMN wide, the IE AreaConfiguration-r16 would not be configured to the UE which leads to the fact that the interFreqTargetList-r16 cannot be configured in this case.

RAN3 would like RAN2 to check whether this is an erroneous implementation in TS 38.331.

- Frequency band info

In TS 38.413, the NR Frequency Info in Area Scope of Neighbour Cells supports NR Frequency Band List configuration. While in TS 38.331, there isn’t any NR Frequency Band configuration in the InterFreqTargetInfo configured to the UE.

RAN3 would like RAN2 to feedback whether NR Frequency Band needs to be supported for the Area Scope of Neighbour Cells.

2	Actions

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to provide feedback on the topics mentioned above.

3	Dates of next RAN3 meetings

RAN3 #113-e	16-26 Aug 2021	Electronic Meeting