3GPP TSG-RAN WG3 Meeting #113-e	R3-213178

Online, 16-26 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0179 | rev | 2 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BL CR to TS 38.401 on support of eIAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  |  | Date: |  | 2021-08-16 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Support the inter-donor IAB topology update for Rel-17 eIAB |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RAN3#112e Add the procedure for the IAB inter-donor migration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support the inter-donor IAB topology update for Rel-17 eIAB |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1; 8.xx (new); 8.xx.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | Rev -: - Add the procedure for the IAB inter-donor migration, according to agreements in RAN3-112e meeting. Rev 1: R3-212971 - Rewording step 13 as ” The F1-C and F1-U are switched to the target path, details are FFS” - Add an Editor note after step 13, to indicate that whether to align step 13 with the corresponding R16 text. - Add “may” in step 15, and remove the FFS part. Rev 2: R3-213178 - Submit to the RAN3-112e meeting, rebase the v16.6.0 of the latest spec. |
| --- | --- |

START OF CHANGE

3	Definitions and abbreviations

3.1		Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Boundary IAB-node: an IAB-node, who’s IAB-DU terminates the F1 interface at an IAB-donor-CU different than its parent gNB-DU.

Conditional Handover: as defined in TS 38.300 [2].

Conditional PSCell Change: as defined in TS 37.340 [12].

DAPS Handover: as defined in TS 38.300 [2].

en-gNB: as defined in TS 37.340 [12].

Early Data Forwarding: as defined in TS 38.300 [2].

gNB: as defined in TS 38.300 [2].

gNB Central Unit (gNB-CU): a logical node hosting RRC, SDAP and PDCP protocols of the gNB or RRC and PDCP protocols of the en-gNB that controls the operation of one or more gNB-DUs. The gNB-CU terminates the F1 interface connected with the gNB-DU.

gNB Distributed Unit (gNB-DU): a logical node hosting RLC, MAC and PHY layers of the gNB or en-gNB, and its operation is partly controlled by gNB-CU. One gNB-DU supports one or multiple cells. One cell is supported by only one gNB-DU. The gNB-DU terminates the F1 interface connected with the gNB-CU.

gNB-CU-Control Plane (gNB-CU-CP): a logical node hosting the RRC and the control plane part of the PDCP protocol of the gNB-CU for an en-gNB or a gNB. The gNB-CU-CP terminates the E1 interface connected with the gNB-CU-UP and the F1-C interface connected with the gNB-DU.

gNB-CU-User Plane (gNB-CU-UP): a logical node hosting the user plane part of the PDCP protocol of the gNB-CU for an en-gNB, and the user plane part of the PDCP protocol and the SDAP protocol of the gNB-CU for a gNB. The gNB-CU-UP terminates the E1 interface connected with the gNB-CU-CP and the F1-U interface connected with the gNB-DU.

IAB-node: as defined in TS 38.300 [2].

IAB-donor: as defined in TS 38.300 [2].

IAB-donor-CU: the gNB-CU of an IAB-donor, terminating the F1 interface towards IAB-nodes and IAB-donor-DU.

IAB-donor-DU: the gNB-DU of an IAB-donor, hosting the IAB BAP sublayer (as defined in TS 38.340 [22]), providing wireless backhaul to IAB-nodes.

IAB-DU: as defined in TS 38.300 [2].

IAB-MT: as defined in TS 38.300 [2].

ng-eNB: as defined in TS 38.300 [2].

ng-eNB Central Unit (ng-eNB-CU): as defined in TS 37.470 [21].

ng-eNB Distributed Unit (ng-eNB-DU): as defined in TS 37.470 [21].

NG-RAN node: as defined in TS 38.300 [2].

PDU Session Resource: This term is used for specification of NG, Xn, and E1 interfaces. It denotes NG-RAN interface and radio resources provided to support a PDU Session.

Public Network Integrated NPN: as defined in TS 23.501 [3].

Stand-alone Non-Public Network: as defined in TS 23.501 [3].

NEXT  CHANGE

8.xx	IAB Inter-gNB-CU Topology Adaptation

### 8.xx.1  IAB inter-CU topology adaptation procedure

During the inter-CU topology adaptation for single-connected IAB-node, the IAB-MT switches connection from an old parent node to a new parent node, where the old and the new parent nodes are served by different IAB-donor-CUs. Without loss of generality, the old parent node can be referred to as source parent node, and the new parent node can be referred to as target parent node.

Figure 8.xx.1-1 shows an example of the topology adaptation procedure where the migrating IAB-MT is migrated from one IAB-donor-CU to another IAB-donor-CU. In case the IAB-DU of the migrating IAB-node retains its F1 connection with the first IAB-donor-CU (i.e. the source IAB-donor-CU) after the migrating IAB-MT connects to the second IAB-donor-CU (i.e. the target IAB-donor-CU), this procedure renders the migrating IAB-node as a boundary IAB-node.

Figure 8.xx.1-1: IAB inter-CU topology adaptation procedure

- The source IAB-donor-CU sends a HANDOVER REQUEST message to the target IAB-donor-CU over the Xn interface.

- The target IAB-donor-CU sends a UE CONTEXT SETUP REQUEST message to the target parent node IAB-DU to create the UE context for the migrating IAB-MT and set up one or more bearers. These bearers can be used by the migrating IAB-MT for its own signalling, and, optionally, data traffic.

- The target parent node IAB-DU responds to the target IAB-donor-CU with a UE CONTEXT SETUP RESPONSE message.

- The target IAB-donor-CU performs admission control and provides the new RRC configuration as part of the HANDOVER REQUEST ACKNOWLEDGE message.

- The source IAB-donor-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source parent node IAB-DU, which includes the received RRCReconfiguration message from the target IAB-donor-CU.

- The source parent node IAB-DU forwards the received RRCReconfiguration message to the migrating IAB-MT.

- The source parent node IAB-DU responds to the source IAB-donor-CU with the UE CONTEXT MODIFICATION RESPONSE message.

- A random access procedure is performed at the target parent node IAB-DU.

- The migrating IAB-MT responds to the target parent node IAB-DU with an RRCReconfigurationComplete message.

- The target parent node IAB-DU sends an UL RRC MESSAGE TRANSFER message to the target IAB-donor-CU to convey the received RRCReconfigurationComplete message.

- The target IAB-donor-CU triggers path switch procedure for the migrating IAB-MT, if needed.

- The target IAB-donor-CU configures BH RLC channels and BAP-sublayer routing entries on the target path between the target parent IAB-node and target IAB-donor-DU as well as DL mappings on the target IAB-donor-DU for the migrating IAB-node’s target path.

Editor’s Note: Whether the step 12 can be performed at earlier stage is FFS.

Editor’s Note: Before step 12, the signalling exchange between the source IAB-donor-CU and the target IAB-donor-CU is needed to help the target IAB-donor-CU configure the BAP routing and BH RLC CH mapping at the target path, and help the target IAB-donor-CU perform the configuration for traffic transmission via the target path. FFS on details.

- The F1-C and F1-U are switched to the target path, details are FFS.

Editors’ Note: Whether the step 13 should align to the corresponding Rel-16 text?

Editors’ Note: The routing in the target path is FFS.

- The target IAB-donor-CU sends UE CONTEXT RELEASE message to the source IAB-donor-CU.

- The source IAB-donor-CU may release BH RLC channels and BAP-sublayer routing entries on the source path between source parent IAB-node and source IAB-donor-DU.

END OF CHANGE