3GPP TSG-RAN WG3 Meeting #113-e	R3-213218

E-meeting, 16-27 Aug 2021

Title: 	(TP for SON BL CR for TS 38.401): Stage 2 update for RACH Optimization

Source: 	Huawei

Agenda item:	10.2.1.7

Document Type:	Other

1. Introduction

This contribution contains the TP for SON BLCR for TS 38.401 as discussed in R3-213217.

Annex – TP for SON BLCR for TS 38.401

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

7.5	RACH Optimisation Function

The RACH Optimization Function in non-split gNB case is specified in TS 38.300 [2].

In case of split gNB architecture, RACH configuration conflict detection and resolution function is located at the gNB-DU. To perform RACH optimisation at gNB-DU, gNB-CU sends the RACH report reported by the UE to gNB-DU via F1AP signalling. The gNB-DU signals the PRACH configuration per-cell to gNB-CU. The gNB-CU may forward a limited set of neighbour cell’s PRACH configurations received from neighbour gNBs and other gNB-DUs to the gNB-DU to resolve the configuration conflict.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>