3GPP TSG-RAN WG3 Meeting #113-e	R3-213223

E-meeting, 16-27 Aug 2021

Title: 	L2 measurement in MR-DC

Source: 	Huawei

Agenda item:	10.4

Document Type:	discussion

1. Introduction

In the last meeting, RAN3 discussed the M6 measurement in MR-DC and has the following FFS:

Enable sending the following measurements from the CU-UP to the TCE.

1) Number of PDCP PDUs sent via MN or SN within a measurement period, when PDCP duplication is enabled.

2) Number of PDCP PDUs sent over MN within a measurement period, when the PDCP duplication is not enabled.

3) Number of PDCP PDUs sent over SN within a measurement period, when the PDCP duplication is not enabled.

Other solutions are not precluded

To be continued on this basis...

In this contribution, we will provide the analysis for these FFS.

2. Discussion

In our understanding, RAN2 and RAN3 introduce the M6 measurement for different purposes. We think RAN3 need to analyse the M6 measurement in MR-DC case by case.

2.1 QoS monitoring and L2 measurement

For the QoS monitoring, the gNB provides the delay measurement result to the CN via the user plane. Therefore the gNB need combine the whole parts of RAN delay measurement results to get one combined result to the CN.

For the L2 measurement, we also think the gNB need to get one combined result.

Observation 1: For the QoS monitoring and L2 measurement, it is the NG-RAN to combine the whole parts of delay measurements.

And the delay measurement is performed per DRB. In MR-DC, there are several bearer types, i.e. MN terminated MCG/SCG/Split bearer and SN terminated MG/SCG/Split bearer. In R16, RAN2 and RAN3 has supported the delay measurement of MN terminated MCG bearer and SN terminated SCG bearer. In the following, we will provide the analysis for the delay measurement of other bearers.

2.1.1 QoS monitoring

For the QoS monitoring, the AMF sends the QoS monitoring configuration per QoS flow to the MN.

For the MN terminated SCG bearer, the MN can send the QoS monitoring configuration per QoS flow to the SN via the QoS Flow Level QoS Parameters within the PDU Session Resource Setup Info – MN terminated. Therefore the SN can know whether and which DRB need to perform the delay measurement.

In the meeting of RAN2#113b-e, RAN2 has the following agreements:

Agreements:

1	For MN terminated SCG bearer and SN terminated MCG bearer, the terminated node, e.g., MN in case of MN terminated SCG bearer,configures the configuration to UE.

Therefore for the MN terminated SCG bearer, the MN sends the D1 measurement configuration to the UE and receives the D1 measurement results from the UE. For the delay measurement performed in SN, the SN can report them to the MN like the measurement report in CU-DU. Therefore we think there is no RAN3 impact.

For the SN terminated MCG bearer, the MN can send the QoS monitoring configuration per QoS flow to the SN via the QoS Flow Level QoS Parameters within the PDU Session Resource Setup Info – SN terminated. Therefore the SN can know whether and which DRB need to perform the delay measurement. And also the SN informs the QoS flows of each DRB in the QoS Flows Mapped To DRB List within the PDU Session Resource Setup Response Info – SN terminated. Therefore the MN also can know whether and which DRB need to perform the delay measurement. For the delay measurement performed in MN, the MN can report them to the SN like the measurement report in CU-DU. Therefore we think there is no RAN3 impact.

Proposal 1: For QoS monitoring of MN terminated SCG and SN terminated MCG bearer, there is no RAN3 impact.

For the MN terminated split bearer and SN terminated split bearer, RAN2 has the following agreements:

7	For QoS monitoring related delay reporting to CN, the minimum value between two legs is defined as the total delay measurement M6 over MCG/SCG for split bearers WITH PDCP duplication.

Agreement:

For QoS monitoring related delay reporting to CN, ‘weighted average (consider the number of packets) over MN and SN’ is used to calculate the total delay measurement M6 over MCG/SCG for split bearers WITHOUT PDCP duplication.

RAN2 is still discussing how to send the measurement configuration to the UE.

In our understanding, the node hosting the PDCP entity know whether the PDCP duplication is used and know the number of packets of each legs. Therefore the current agreement does not have impact on RAN3.

Proposal 2: The current agreements of QoS monitoring in RAN2 does not have impact on RAN3.

2.1.2 L2 measurement

In CU-DU, RAN3 does not define the signalling interaction for L2 measurement. We think RAN3 can use the same principle to the delay measurement for MR-DC. Therefore we think RAN3 does not need to specify the interaction for L2 measurement configuration. For the measurement result reporting, we think it is same to the reporting of QoS monitoring.

Observation 2: In CU-DU, RAN3 does not specify the signalling interaction for L2 measurement configuration.

Proposal 3: For the L2 measurement, there is no RAN3 impact

2.1 MDT measurement

In the TS 37.320, the following principles of immediate MDT in EN-DC is specified in R16.

#### 5.4.1.3 Immediate MDT for MR-DC Immediate MDT is supported for EN-DC scenario. In signalling based immediate MDT, MME provides MDT configuration for both MN and SN towards MN including multi RAT SN configuration, specifically E-UTRA and NR MDT configuration. MN then forwards the NR MDT configuration towards SN (EN-DC scenario, SN is always NR). In management-based immediate MDT, OAM provides the MDT configuration to both MN and SN independently. For both MN and SN, Management based MDT should not overwrite signalling based MDT. For immediate MDT configuration, MN and SN can independently configure and receive measurement from the UE.

The MDT for MR-DC was discussed in RAN2 #113b meeting and achieved the following agreements:

All the immediate MDT configurations and reporting in EN-DC scenario (i.e. section 5.4.1.3 Immediate MDT for MR-DC in TS 37.320) are also applicable for (NG) EN-DC, NE-DC and NR-DC.

Therefore we think the MN and SN will perform the delay measurement independently. We think there is no RAN3 impact.

Observation 3: For the management based immediate MDT, RAN2 has agreed that the MN and SN performs the MDT measurement independently.

Proposal 4: For the management based immediate MDT configuration, there is no RAN3 impact.

According to the agreement of RAN2, we think RAN2 only agrees to combine the results between the two legs for the QoS monitoring, but for MDT.

7	For QoS monitoring related delay reporting to CN, the minimum value between two legs is defined as the total delay measurement M6 over MCG/SCG for split bearers WITH PDCP duplication.

Agreement:

For QoS monitoring related delay reporting to CN, ‘weighted average (consider the number of packets) over MN and SN’ is used to calculate the total delay measurement M6 over MCG/SCG for split bearers WITHOUT PDCP duplication.

In the last meeting, some companies propose that the CU-CP send the following information to the TCE to associate the TCE to calculate the whole RAN part delay for the split bearer.

1) Number of PDCP PDUs sent via MN or SN within a measurement period, when PDCP duplication is enabled. 2) Number of PDCP PDUs sent over MN within a measurement period, when the PDCP duplication is not enabled. 3) Number of PDCP PDUs sent over SN within a measurement period, when the PDCP duplication is not enabled.

Therefore we think we need first check with RAN2 whether the agreement is also applied to the M6 in MDT measurement.

Proposal 5: To check with RAN2 on whether the above-mentioned agreements are also applied to M6 in MDT.

If RAN2 confirmed that M6 for MDT measurement should be also calculated as the minimum value between MN and SN in PDCP duplication case or the weighted average value in without PDCP duplication case, the following scenarios should be studied:

Case 1: PDCP duplication is activated within the collection period of M6

Case 2: PDCP duplication is not activated within the collection period of M6

Case 3: PDCP duplication/non-duplication switch within the collection period of M6

For case 1 and case 2, RAN node needs to report to TCE with a PDCP duplication status indication. Based on that, the TCE may choose the proper way to calculate the final RAN part delay reported by MN and SN. If the indication indicates that PDCP duplication is activated, the TCE may choose the minimum values between MN delay and SN delay. If the indication indicates that PDCP duplication is not activated, the TCE may calculate the delay value by weighted average between MN delay and SN delay. Therefore, for case 2, an additional ratio of packets are transmitted between MN and SN is needed to report to TCE.

For case 3, besides the ratio of duplicated packets and non-duplicated packets, the TCE needs to know the ratio between MN and SN for those non-duplicated packets. E.g., the radio of the number of non-duplicated packets sent via the MN or SN.

Proposal 6: If the TCE needs to calculate M6 for the split bearers, the node hosting the PDCP entities provides the following information to the TCE:

- A PDCP duplication or non-duplication indication (case 1 and 2).

- The ratio of the number of duplication packets sent via MN or SN to the total number of packets sent in the reporting period (case 2).

- The radio of the number of non-duplication packets sent via the MN or SN to the total number of non-duplication packets sent in the collection period (case 3).

3. Conclusion

Based on the discussion in this paper, we propose the following:

Observation 1: For the QoS monitoring and L2 measurement, it is the NG-RAN to combine the whole parts of delay measurements.

Observation 2: In CU-DU, RAN3 does not specify the signalling interaction for L2 measurement configuration.

Observation 3: For the management based immediate MDT, RAN2 has agreed that the MN and SN performs the MDT measurement independently.

Proposal 1: For QoS monitoring of MN terminated SCG and SN terminated MCG bearer, there is no RAN3 impact.

Proposal 2: The current agreements of QoS monitoring in RAN2 does not have impact on RAN3.

Proposal 3: For the L2 measurement, there is no RAN3 impact

Proposal 4: For the management based immediate MDT configuration, there is no RAN3 impact.

Proposal 5: To check with RAN2 on whether the above-mentioned agreements are also applied to M6 in MDT.

Proposal 6: If the TCE needs to calculate M6 for the split bearers, the node hosting the PDCP entities provides the following information to the TCE:

- A PDCP duplication or non-duplication indication (case 1 and 2).

- The ratio of the number of duplication packets sent via MN or SN to the total number of packets sent in the reporting period (case 2).

- The radio of the number of non-duplication packets sent via the MN or SN to the total number of non-duplication packets sent in the collection period (case 3).

The draft LS is provided in the Annex.

4. References

[1] R3-212929, SoD L2 Measurements

Annex – Draft LS to RAN2

3GPP TSG-RAN WG3 Meeting #113-e	<TDoc#>

E-meeting, 16-27 Aug 2021

Title:	[Draft] MDT M6 calculation for split bearers in MR-DC

Response to:

Release:	Rel-17

Work Item:

Source:	Huawei [to be RAN3]

To:	RAN2

Cc:

Contact person:

Name:	Hongzhuo Zhang

E-mail Address:	zhanghongzhuo@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN3 noted that there are some agreements for the RAN part delay measurement calculation for split bearers in MR-DC for Qos monitoring.

7	For QoS monitoring related delay reporting to CN, the minimum value between two legs is defined as the total delay measurement M6 over MCG/SCG for split bearers WITH PDCP duplication.

Agreement:

For QoS monitoring related delay reporting to CN, ‘weighted average (consider the number of packets) over MN and SN’ is used to calculate the total delay measurement M6 over MCG/SCG for split bearers WITHOUT PDCP duplication.

RAN3 would like RAN2 to confirm whether above mentioned agreements are also applied to M6 for split bearers in MR-DC in MDT.

2	Actions

To RAN2

ACTION: RAN3 respectfully asks RAN2 to confirm whether above mentioned agreements are also applied to M6 for split bearers in MR-DC in MDT.

3	Dates of next RAN3 meetings

3GPP TSG RAN3#114e		11/2021	E-Meeting

3GPP TSG RAN3#115e		02/2022	E-Meeting