3GPP TSG-RAN WG3 #113-e							R3-213244

16-26 August 2021                                                                       revision of R3-211588

Online

Source:               ZTE, Qualcomm Incorporated

Title:                    Discussion on Carrier Selection and Carrier Specific Configuration

Agenda item:      12.2

Document for:    Discussion and Decision

Introduction

The Rel-17 WID of additional enhancements for NB-IoT and eMTC have been approved in RAN#86 and revision can be seen in [1]. The following objective is included in the WID:

- Introduce support for NB-IoT carrier selection based on the coverage level, and associated carrier specific configuration (e.g. maximum repetitions UL/DL, DRX configurations, etc.)

In the RAN2 meeting, the following agreements have been agreed[2][3]:

RAN2#111-e agreements: Paging carrier selection Improvements based on CE level is considered Paging carrier selection Improvements based on DRX cycle may be considered - whether DRX cycle is considered as part of CE level (Rmax) or can be also considered separately Enhancements for NPRACH Carrier selection carrier may be considered Paging carrier selection Improvements solely based on WUS or GWUS is not considered FFS service based RAN2#112-e agreements: None RAN2#113-e agreements: Select between one of the options: - Option 1: UE selects a paging carrier based on a rule configured by the network - Option 2: NW configures a specific paging carrier Working assumption: For both options, when coverage changes, mechanism that requires UE to report the update of coverage is not introduced. RAN2#113bis-e agreements:None

In the RAN3 #112-e meeting, the following agreements have been agreed:

WA: Both EPC and 5GC scenarios are supported. - For CE based carrier selection, RAN3 needs to wait for RAN2 further progress.

In this contribution, we will discuss the RAN3 imapcts by this issue and give our proposals.

Discussion

In RAN2, the proposed two options for coverage-based paging carrier selection are depicted in Figure 1.

Figure 1: The two options for coverage-based paging carrier selection

For the Option 1:

- NW provides the information for coverage-based paging carrier selection by SIB, the information mainly includes: the mapping between enhanced coverage level(Rmax-paging, DRX cycle,etc) and paging carrier.

- NW indicates the additional information for paging carrier selection by RRCConnectionRelease message, the additional information for paging carrier selection mainly includes: enhanced coverage level(Estimated Rmax-paging).

- UE selects the paging carrier based on the Estimated Rmax-paging and the mapping between Rmax-	paging and paging carrier.

Since UE selects the paging carrier based on the Estimated Rmax-paging and the mapping between Rmax-paging and paging carrier, eNB should also selects the paging carrier for the UE based on the estimated Rmax-paging and the mapping between Rmax-paging and paging carrier. Although the estimated Rmax-paging(Coverage Enhancement Level) can already be sent to CN and then CN includes it in S1AP PAGING message, but the paging eNB can not differentiate whether the estimated Rmax-paging(Coverage Enhancement Level) will be used for paging carrier selection or not(e.g. if eNB does not send Estimated Rmax-paging to UE by RRCConnectionRelease message, Rmax-paging based paging carrier selection will not be activated). Thus, paging eNB should know whether the Rmax-paging based paging carrier selection is activated or not.

Observation 1: For option 1, based on the current specification, the paging eNB can know the Coverage Enhancement Level estimated in the latest connection, but it cannot know whether the Rmax-paging based paging carrier selection is activated or not.

Proposal 1: If option1 is agreed by RAN2, an Rmax-paging based paging carrier selection activation should be indicated to the paging eNB during CN paging.

Since Coverage Enhancement Level IE is included in Cell Identifier and Coverage Enhancement Level IE in TS36.413 and Paging Assistance Data for CE Capable UE IE in TS38.413, which is necessary for Rmax-paging based paging carrier selection if option 1 is agreed, the Rmax-paging based paging carrier selection activation is also suggested to be included in Cell Identifier and Coverage Enhancement Level IE in TS36.413 and Paging Assistance Data for CE Capable UE IE in TS38.413, which is included in S1AP/NGAP UE CONTEXT RELEASE COMPLETE, UE CONTEXT SUSPEND REQUEST, UE CONTEXT RESUME REQUEST and PAGING meeage.

Proposal 1a: If option1 is agreed by RAN2, an Rmax-paging based paging carrier selection activation is included in S1AP/NGAP Cell Identifier and Coverage Enhancement Level IE in TS36.413 and Paging Assistance Data for CE Capable UE IE in TS38.413.

For the Option 2:

- NW provides the Paging carrier information to UE by RRCConnectionRelease message; the Paging carrier information at least includes: the DARFCN of paging carrier. Whether other Paging carrier configuration information(e.g. Rmax, DRX etc) will be configured by RRCConnectionRelease message has not been discussed yet.

- UE uses the Paging carrier information(e.g. Paging carrier) included in RRCConnectionRelease message for paging monitoring.

Since UE uses the Paging carrier included in RRCConnectionRelease message for paging monitoring, eNB should also use the Paging carrier included in RRCConnectionRelease message for paging transmitting Thus, the paging eNB should also know the Paging carrier information included in RRCConnectionRelease for paging transmitting.

Observation 2: For option 2, the paging eNB should know the Paging carrier information included in RRCConnectionRelease for paging transmitting.

In the last meeting, some companies suggest to include the Paging carrier information in the Cell Identifier and Coverage Enhancement Level IE or UERadioPagingInformation RRC container. Although if Option 2 is agreed, what Paging carrier information will be included in RRCConnectionRelease message and should be provided in S1AP paging has not been decided yet, but since it is used for carrier specific configuration, at least the DARFCN or carrier index of the paging carrier should be included. Obviously, it is not suitable to include the DARFCN or carrier index of the paging carrier in the Cell Identifier and Coverage Enhancement Level IE. Furthermore, UERadioPagingInformation RRC container is associated with UE Radio Capability for Paging IE as follows:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Radio Capability for Paging | M |  | OCTET STRING | Includes either the UERadioPagingInformation message as defined in 10.2.2 of TS 36.331 [16], or the UERadioPagingInformation-NB message as defined in 10.6.2 of TS 36.331 [16]. |

It is also not suitable to include the DARFCN or carrier index of the paging carrier in the UE Radio Capability for Paging IE.

Observation 3: if option 2 is agreed, there is not existing IE suitable to include the Paging carrier information (e.g. the DARFCN or carrier index of the paging carrier).

If option 2 is agreed by RAN2, an RRC container for Paging carrier information will be necessary to be included in S1AP/NGAP UE CONTEXT RELEASE COMPLETE, UE CONTEXT SUSPEND REQUEST, UE CONTEXT RESUME REQUEST(NGAP only) and PAGING message, which will impacts RAN3 specification much.

Observation 4: The option 2 will impacts RAN3 specification much.

Bsed on the following consideration,we prefer the Option 1.

- For Option 1, the paging carrier information is configured by SIB, NW can update it flexibly. But for option 2, paging carrier information is configured by RRCConnectionRelease, it is impossible to update it for UE in RRC_IDLE.

- For option 1, the paging carrier is usually selected by UE, it is also suitable for mobility UEs; But for option 2, the paging carrier is configured by gNB, it is not suitable for mobility UEs

- Option 1 impacts RAN3 specification much less.

Proposal 2: From RAN3 point of view, the Option 1 is preferred.

Based on the proposals, we provide the TS 36.413 draftCR and TS 38.413 draftCR for Option1 [5][6].

Conclusions

In this contribution, we make the following observations and proposals:

Observation 1: For option 1, based on the current specification, the paging eNB can know the Coverage Enhancement Level estimated in the latest connection, but it cannot know whether the Rmax-paging based paging carrier selection is activated or not.

Observation 2: For option 2, the paging eNB should know the Paging carrier information included in RRCConnectionRelease for paging transmitting.

Observation 3: if option 2 is agreed, there is not existing IE suitable to include the Paging carrier information(e.g. the DARFCN or carrier index of the paging carrier).

Observation 4: The option 2 will impacts RAN3 specification much.

Proposal 1: If option1 is agreed by RAN2, an Rmax-paging based paging carrier selection activation should be indicated to the paging eNB during CN paging.

Proposal 1a: If option1 is agreed by RAN2, an Rmax-paging based paging carrier selection activation is included in S1AP/NGAP Cell Identifier and Coverage Enhancement Level IE in TS36.413 and Paging Assistance Data for CE Capable UE IE in TS38.413.

Proposal 2: From RAN3 point of view, the Option 1 is preferred.

Based on the proposals, we provide the TS 36.413 draftCR and TS 38.413 draftCR for Option 1[5][6].

- References

- 3GPP, RP-201064, WID revision: Additional enhancements for NB-IoT and LTE-MTC, June 2020

- 3GPP, R2-2102164, RAN2 agreements for Rel-17 NB-IoT and LTE-MTC, January, 2021

- 3GPP, R2-113bis-e, NB-IoT Brian 2021-04-20-1605 UTC_eom, April, 2021

- 3GPP, R2-2104450, Report of [AT113bis-e][301][NBIOT/eMTC R17] NB-IoT Carrier Selection (Qualcomm), April, 2021

- 3GPP, R3-213245 36.413 (Rel-16) Introduction of CEL based paging carrier selection, August, 2021

- 3GPP, R3-213246 38.413 (Rel-16) Introduction of CEL based paging carrier selection, August, 2021