3GPP TSG-RAN WG3 Meeting #113-e	R3-213245

# Online, 16-26 August 2021                                   			                Revision of R3-211589

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1809 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of CEL based paging carrier selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6-Core |  |  |  |  |  | Date: |  |  | 2021-8-8 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CEL based paging carrier selection feature is introduced for NB-IoT, and paging eNB can know the UE’s CEL, but the paging eNB cannot know whether the CEL is used for paging carrier selection feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include Coverage Enhancement Level based paging carrier indication in Cell Identifier and Coverage Enhancement Level IE. Impact Analysis Impacted functionality: This CR only has an impact on the CEL based paging carrier selection for NB-IoT UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CEL based paging carrier selection cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.109, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R3-211589 |  |  |  |  |  |  |  |  |

Start of the change

9.2.1.109	Cell Identifier and Coverage Enhancement Level

This IE provides information for paging CE capable UEs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Global Cell ID | M |  | E-UTRAN CGI 9.2.1.38 |  |
| Coverage Enhancement Level | M |  | OCTET STRING | Includes either the UEPagingCoverageInformation message as defined in 10.2.2 of TS 36.331 [16], or the UEPagingCoverageInformation-NB message as defined in 10.6.2 of TS 36.331 [16]. |
| Coverage Enhancement Level based paging carrier selection | O |  | ENUMERATED (True, …) | Indicates whether Coverage Enhancement Level based paging carrier selection for NB-IoT UE is used. |

| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |
| --- | --- |

Next change

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

S1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

//SKIP THE UNRELATED PART//

id-EmergencyIndicator,

id-CELBasedPagingCarrierSelection

maxnoofCSGs,

//SKIP THE UNRELATED PART//

CellIdentifierAndCELevelForCECapableUEs ::= SEQUENCE {

global-Cell-ID		EUTRAN-CGI,

cELevel				CELevel,

iE-Extensions		ProtocolExtensionContainer { { CellIdentifierAndCELevelForCECapableUEs-ExtIEs} } OPTIONAL,

...

}

CellIdentifierAndCELevelForCECapableUEs-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CELBasedPagingCarrierSelection	CRITICALITY ignore	EXTENSION  CELBasedPagingCarrierSelection		PRESENCE optional},

...

}

CELBasedPagingCarrierSelection ::= ENUMERATED {

TRUE,

...

}

-- Coverage Enhancement level encoded according to TS 36.331 [16] --

CELevel ::= OCTET STRING

CE-mode-B-SupportIndicator ::= ENUMERATED {

supported,

...

}

//SKIP THE UNRELATED PART//

Next change

9.3.6	Constant Definitions

//SKIP THE UNRELATED PART//

id-EmergencyIndicator								ProtocolIE-ID ::= 326

id-CELBasedPagingCarrierSelection					ProtocolIE-ID ::= 32x

END

-- ASN1STOP

End of the change