3GPP TSG-RAN WG3 #113-e						R3-213272

E-meeting, 16 – 26 August 2021

Agenda Item:  8.1

Source:       NEC

Title:		Proposed response LS to RAN2 on the Inter MN handover without SN change

Document for:	dicision

- Introduction

This contribution discusses the RAN LS in R3-213121(R2-2106682) asking RAN3 the questions:

Question 1: In the inter-MN handover without SN change scenario, is the SN UE X2/XnAP ID always required to be present when target MN sends SN Addition Request to SN?

Question 2: For the same scenario, RAN2 would like to confirm with RAN3 if the receipt of SN UE X2/XnAP ID alone may be interpreted by SN to retrieve the SCG configuration to provide delta configuration?

- 2. Discussion

For the Question 1:

The purpose of giving the SN UE X2/XnAP ID in the SN Addition Request message, is for the inter-MN handover without SN change, then the SN can easily find the UE context. So the proposed answer is :

“Yes, In the inter-MN handover without SN change scenario, the SN UE X2/XnAP ID is always required to be present when target MN sends SN Addition Request to SN”.

For the Question 2:

When the SN UE X2/XnAP ID is present in the SN Addition Request message, as answer to Question1, it is for the handover without SN change, how the SN will use the SN UE X2/Xn AP ID together with other RRC information (sourceConfigSCG, scg-RB-Config) to apply delta configuration, it is within the RAN2 scope. So the proposed answer is:

“The presence of SN UE X2/XnAP ID indicates the request is for inter-MN HO without SN change. RAN3 understands it is RAN2 scope how the SN UE X2/XnAP ID is used alone or together with other RRC IEs (sourceConfigSCG, scg-RB-Config) to apply delta configuration.”

Proposal 1: it is proposed to answer to RAN2 as shown in the Annex of this paper.

- 3. proposal

Proposal 1: it is proposed to answer to RAN2 based on draft in Annex of this paper.

Reference:

R3-213121(R2-2106682) LS on inter-MN handover without SN change (source RAN2)

Annex:

3GPP TSG-RAN WG3 #113-e						R3-21xxxx

E-meeting, 16 – 26 August 2021

Title:	[DRAF] Response LS on inter-MN handover without SN change

Response to:	R2-2106682

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	NEC [To be RAN3]

To:	RAN2

Cc:

Contact Person:

- Name:	Chenghock Ng

- E-mail Address:	ngchenghock.at.nec.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

Regarding the questions asked by RAN2, RAN3 would like to provide the following answers:

RAN2 Question 1: In the inter-MN handover without SN change scenario, is the SN UE X2/XnAP ID always required to be present when target MN sends SN Addition Request to SN?

Answer 1: Yes, In the inter-MN handover without SN change scenario, the SN UE X2/XnAP ID is always required to be present when target MN sends SN Addition Request to SN.

RAN2 Question 2: For the same scenario, RAN2 would like to confirm with RAN3 if the receipt of SN UE X2/XnAP ID alone may be interpreted by SN to retrieve the SCG configuration to provide delta configuration?

Answer 2: The presence of SN UE X2/XnAP ID indicates the request is for inter-MN HO without SN change. RAN3 understands it is RAN2 scope how the SN UE X2/XnAP ID is used alone or together with other RRC IEs (sourceConfigSCG, scg-RB-Config) to apply delta configuration.

2. Actions:

To RAN2.

ACTION: 	RAN3 respectfully asks RAN2 to take the answer above into account.

3. Date of Next TSG-RAN WG3 Meeting:

TSG-RAN3 Meeting #114-e 	Nov 1st – 11th, 2021		Meeting