3GPP TSG-RAN WG3 #113-e						R3-213284

E-meeting, 16 – 26 August 2021

Agenda Item:  14.3.

Source:       NEC

Title:		(TP for CPAC BL CR to TS 36.423 ) Discussion on CPAC (Conditional PSCell Addition and PSCell Change)

Document for:	dicision

- Introduction

This contribution discusses some open issues related with the CPAC, mainly focus on X2 and Xn aspect.

- 2. Discussion

- 2.1 Target SN provide the prepared PSCell ID in X2/Xn level

Last RAN3#112e has reached the agreement and FFS that:

Introduce “List of Prepared PSCell IDs” in SN Addition Request ACK. FFS whether to introduce “List of Prepared PSCell IDs” in SN Change Confirm.

It has been agreed to introduce “List of Prepared PScell IDs” in SN Addition Request Acknowledge message, and it is already reflected in the baseline CR.

For the introduction of the “List of Prepared PSCell IDs” in the SN Change Confirm message, it may have a benefit that when the S-SN at least need to update the measurement configuration, or want to update execution condition corresponding to the prepared PScells that are chosen by the T-SN.

Furthermore, RAN2#114e has a working assumption:

4. S-SN is informed about which candidates were accepted/ rejected by T-SN

It will be likely the working assumption will be agreement as there is no showstopper foreseen. Therefore it would be reasonable to introduce also “List of Prepared PSCell IDs” in the SN Change Confirm message.

Proposal 1: it is beneficial to include the “List of Prepared PSCell IDs” in SN Change Confirm message, for the S-SN to e.g. update the measurement configuration, or want to update execution condition corresponding to the prepared PSCell by T-SN.

- 2.2 Single or multiple RRC containers in SN Addition Request ACK message

Last RAN3#111e meeting has reached working assumption that:

WA: Prepare multiple PSCells in one CPAC procedure. Do not provide Location Information and Resource Coordination information in CPAC, use same parameters for other IEs in the response message for different PSCells, FFS for single RRC container or multiple RRC containers which is pending to RAN2.

A LS was sent to RAN2 in R3-211338 (RAN3#111e). However the RAN2#113bis-e and RAN2#114-e cancelled the reply to RAN3.

Observation 1: whether single / multiple RRC containers is still FFS (still pending on RAN2) in the SN-Node Addition Request Acknowledge message.

- 2.3 Preparation of multiple Target SNs in SN initiated CPC

Regarding the multiple PScells preparation, it has been working assumption in RAN3#111e that:

WA: In case of SN initiated inter-SN CPC, prepare multiple PSCells in one target SN by one SN Change procedure is the baseline.

The above working assumption still hold in last RAN3#112e meeting. Base on this working assumption, last RAN3#112e meeting discussed a possibility that in one SN Initiated SN change procedure whether can prepare multiple T-SNs. This discussion did not reach any status but only in the SOD indicated as FFS (not in the chair note).

SOD in R3-212785: About Multiple Target SNs in SN initiated inter-SN CPC: FFS: Is it allowed to prepare multiple Target SNs in SN initiated inter-SN CPC, by parallel or single SN Change Required procedure(s)? 1)If one SN change procedure can only prepare one target SN, and if parallel preparation is not supported, current SN change required/confirm procedures can be reused. 2)If one SN change procedure can only prepare one target SN, and if parallel preparation is supported, Target S-NG-RAN node ID IE needs to be introduced in SN Change confirm and SN change Refuse. 3)If one SN change procedure can prepare multiple target SNs, a list of Target S-NG-RAN node ID needs to be introduced in SN change required/confirm/refuse, and need to include data forwarding address per Target SN to support early data forwarding.

For the “FFS: Is it allowed to prepare multiple Target SNs in SN initiated inter-SN CPC, by parallel or single SN Change Required procedure(s)?”, we initial think that in order not to complicate the function, it is not to allow multiple T-SNs in SN initiated inter-SN CPC. Then we will have no need to discuss whether to apply either parallel SN change required procedure or to have T-SN node IDs in the SN Change Required/Confirm/Refuse message etc.

However, since the target PSCell ID is NR CGI which will include also the en-gNB ID (e.g. for ENDC), then from the signalling protocol perspective, already by the PSCell ID itself can know the T-SN, even if the multiple T-SN can be prepared, signalling impact may be not much.

Proposal 2: further discuss the complication of X2/Xn signalling perspective in order to prepare multiple T-SN in SN initiated inter-SN CPC.

- 2.4 List of PSCell IDs and RRC Container in SN Change Required message (SN Initiated CPC)

For the SN initiated CPC, even if we will not allow the S-SN to initiate the preparation of the multiple T-SNs, it still need to have a list of PSCell IDs in the SN Change Required message, in order for the MN to explicitly know the target PSCell ID so can avoid the MN to dig into the RRC container, especially for EN-DC, this will help MeNB to avoid digging into NR RRC ASN.1.

By the explicit PSCell ID in the SN Change Required message, the MN can know the T-SN for CPC.

Proposal 3: to include the list of PSCell IDs in SN Change Required message

Following the principle that the MN does not need to dig into the RRC container ( (contains CPC execution condition provided by the SN), the RRC container should then be within the list of PSCell ID in SN Change Required message.

Proposal 4: To include the RRC container (container CPC execution condition provided by the SN) within the list of Target PSCell ID in SN Change Required message.

- 2.5 Support of CPAC updating / Cancel / replacement

For the CPAC updating/ cancel /replacement, last RAN3#112e meeting postponed the discussion but wait for the basic CPAC procedures to be available first.

Last RAN2#114e reached a working assumption that:

5. S-SN can subsequently update the (measurement) configuration. FFS for execution conditions. 6. S-SN can perform this update after the CPC configuration. FFS whether to support updating during the CPC configuration (i.e. solution 2). FFS whether nested procedure is supported

For the updating, following the working assumption in RAN2, updating from S-SN may use the SN Modification Required message, which need to have a list of PSCell IDs to know which PSCell to be updated. However, it is likely that this will need to wait until the RAN2 reach a certain level of agreement.

For the CPAC cancellation from S-SN, for the X2/Xn signalling, S-SN may simply just reuse the SN Modification Required message to indicate the list of PScell IDs to be cancelled. However this may also need to wait until RAN2 reach a certain level of agreement e.g. whether partially cancellation is allowed.

For the CPAC replacement from S-SN, similar to the cancellation, the S-SN may simply just reuse the SN Modification Required message to indicate the prepared PScell to be replaced.

For the CPA and MN initiated CPC, for the updating. Cancellation and replacement, it will be simple to reuse the SN Modification Request message to the prepared T-SN.

Proposal 5 : it is proposed to further hold on the discussion of CPAC updating/ Cancel /replacement, until a RAN2/RAN3 reach a certain level on the basic CPAC procedures.

- 3. proposal

Proposal 1: it is beneficial to include the “List of Prepared PSCell IDs” in SN Change Confirm message, for the S-SN to e.g. update the measurement configuration, or want to update execution condition corresponding to the prepared PSCell by T-SN.

Observation 1: whether single / multiple RRC containers is still FFS (still pending on RAN2) in the SN-Node Addition Request Acknowledge message.

Proposal 2: further discuss the complication of X2/Xn signalling perspective in order to prepare multiple T-SN in SN initiated inter-SN CPC.

Proposal 3: to include the list of PSCell IDs in SN Change Required message

Proposal 4: To include the RRC container (container CPC execution condition provided by the SN) within the list of Target PSCell ID in SN Change Required message.

Proposal 5 : it is proposed to further hold on the discussion of CPAC updating/ Cancel /replacement, until a RAN2/RAN3 reach a certain level on the basic CPAC procedures.

References:

RP-201040 WID on Further Multi-RAT Dual-Connectivity enhancements [LTE_NR_DC_enh2-Core]

R3-211040 (R2-2010850) LS on Conditional PSCell Addition/Change agreements

R3-211133 Summary of Offline Discussion on PSCell_Change_Addition

R3-211423 (R2-2102170) LS on Conditional PSCell Addition/Change agreements

R3-212785 Summary of Offline Discussion on MRDC3-PSCell_Change_Addition

(TP for CPAC BL CR to TS 36.423 )

(as sometime it may be difficult to see which tiny parts have been modified, the TP from this paper are highlighted in green color)

- 8.7.8	SgNB Change

- 8.7.8.1	General

This procedure is used by the en-gNB to change to another en-gNB.

The procedure uses UE-associated signalling.

- 8.7.8.2	Successful Operation

Figure 8.7.8.2-1: SgNB Change, successful operation.

The en-gNB initiates the procedure by sending the SGNB CHANGE REQUIRED message to the MeNB including the Target SgNB ID Information IE. When the en-gNB sends the SGNB CHANGE REQUIRED message, it shall start the timer TDCoverall.

The SGNB CHANGE REQUIRED message may contain

-	the SgNB to MeNB Container IE.

If the MeNB is able to perform the change requested by the en-gNB, the MeNB shall send the SGNB CHANGE CONFIRM message to the en-gNB. For each E-RAB configured with the PDCP entity in the en-gNB, the MeNB may include the DL Forwarding GTP Tunnel Endpoint IE and the UL Forwarding GTP Tunnel Endpoint IE within the E-RABs To Be Released Item IE to indicate that it requests data forwarding of uplink and downlink packets to be performed for that bearer.

The en-gNB may start data forwarding and stop providing user data to the UE and shall stop the timer TDCoverall upon reception of the SGNB CHANGE CONFIRM message.

If the Conditional PSCell Change Information Required IE is included in the SGNB CHANGE REQUIRED message, the MeNB shall consider that the requirement concerns CPAC, as described in TS 37.340 [32]. Accordingly, the MeNB shall include the Conditional PSCell Change Information ConfirmIE in the SGNB CHANGE CONFIRM message.

- 8.7.8.3	Unsuccessful Operation

Figure 8.7.8.3-1: SgNB Change, unsuccessful operation.

In case the request change cannot be performed successfully the MeNB shall respond with the SGNB CHANGE REFUSE message to the en-gNB with an appropriate cause value in the Cause IE.

- 8.7.8.4	Abnormal Conditions

If the timer TDCoverall expires before the en-gNB has received the SGNB CHANGE CONFIRM or the SGNB CHANGE REFUSE message, the en-gNB shall regard the requested change as failed and may take further actions like triggering the SgNB initiated SgNB Release procedure to release all en-gNB resources allocated for the UE.

##### Interaction with the MeNB initiated handover procedure:

If the MeNB, after having initiated the handover procedure, receives the SGNB CHANGE REQUIRED message, the MeNB shall refuse the SgNB change procedure with an appropriate cause value in the Cause IE.

Next change, ommited text not changed

- 9.1.4.2	SGNB ADDITION REQUEST ACKNOWLEDGE

This message is sent by the en-gNB to confirm the MeNB about the SgNB addition preparation.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| E-RABs Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >E-RABs Admitted To Be Added Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |
| >>>>S1 DL GTP Tunnel Endpoint at the SgNB | M |  | GTP Tunnel Endpoint 9.2.1 | en-gNB endpoint of the S1 transport bearer. For delivery of DL PDUs. | – |  |
| >>>>SgNB UL GTP Tunnel Endpoint at PDCP | C-ifMCGpresent |  | GTP Tunnel Endpoint 9.2.1 | en-gNB endpoint of the X2-U transport bearer at PDCP. For delivery of UL PDCP PDUs. | – |  |
| >>>>RLC Mode | C-ifMCGpresent |  | RLC Mode 9.2.119 | Indicates the RLC mode. | – |  |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of DL PDUs | – |  |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint 9.2.1 | Identifies the X2 transport bearer used for forwarding of UL PDUs | – |  |
| >>>>Requested MCG E-RAB Level QoS Parameters | C-ifMCGandSCGpresent_GBRpresent |  | E-RAB Level QoS Parameters 9.2.9 | Includes E-RAB level QoS parameters requested to be provided by the MCG. | – |  |
| >>>>UL Configuration | C-ifMCGandSCGpresent |  | 9.2.118 | Information about UL usage in the MeNB. | – |  |
| >>>>UL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the UL. | YES | ignore |
| >>>>DL PDCP SN Length | O |  | PDCP SN Length 9.2.133 | Indicates the PDCP SN length of the bearer for the DL. | YES | ignore |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>SgNB DL GTP Tunnel Endpoint at SCG | M |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs. | – |  |
| >>>>Secondary SgNB DL GTP Tunnel Endpoint at SCG | O |  | GTP Tunnel Endpoint 9.2.1 | SgNB endpoint of the X2-U transport bearer at the SCG. For delivery of DL PDCP PDUs in case of PDCP duplication | – |  |
| >>>>LCID | O |  | 9.2.138 | LCID for the primary path in case of PDCP duplication | YES | ignore |
| E-RABs Not Admitted List | O |  | E-RAB List 9.2.28 | A value for E-RAB ID shall only be present once in E-RABs Admitted List IE and in E-RABs Not Admitted List IE. | YES | ignore |
| SgNB to MeNB Container | M |  | OCTET STRING | Includes the CG-Config message as defined in TS 38.331[31]. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB | YES | reject |
| Admitted split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |
| SgNB Resource Coordination Information | O |  | 9.2.117 | Information used to coordinate resources utilisation between en-gNB and MeNB. | YES | ignore |
| RRC config indication | O |  | 9.2.132 | Indicates the type of RRC configuration used at the en-gNB. | YES | reject |
| Location Information at SgNB | O |  | 9.2.142 | Contains information to support localisation of the UE | YES | ignore |
| Available fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 isenabled. | YES | ignore |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >CPAC Candidate PSCells Ack List |  | 1 |  |  | - | - |
| >>CPAC Candidate PSCell Ack Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | - | - |
| >>>PSCell ID | M |  | Global en-gNB ID 9.2.112 |  | - | - |
| >>>RRC Container | M |  | OCTET STRING | Includes RRCConnectionReconfiguration message as defined in subclause 6.2.2 of TS 36.331[9]. FFS whether single or multiple RRC containers. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCells for CPAC. Value is FFS. |

| Condition | Explanation |
| --- | --- |
| ifMCGpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources IE in the EN-DC Resource Configuration IE is set to the value "present". |
| ifMCGandSCGpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present". |
| C-ifMCGandSCGpresent_GBRpresent | This IE shall be present if, for the E-RAB admitted to be added, the MCG resources and SCG resources IEs in the EN-DC Resource Configuration IE are set to the value "present", and the GBR QoS Information IE is present in the Requested MCG E-RAB Level QoS Parameters IE. |

Next change, ommited text not changed

- 9.1.4.17	SGNB CHANGE REQUIRED

This message is sent by the en-gNB to the MeNB to request the change of en-gNB for a specific UE.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | reject |
| Target SgNB ID Information | M |  | 9.2.102 |  | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| SgNB to MeNB Container | O |  | OCTET STRING | Includes the CG-Config message as defined in TS 38.331 [31]. | YES | reject |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |
| Conditional PSCell Change Information Required | O |  |  |  | YES | reject |
| >CPAC Indicator | M |  | ENUMERATED (CPAC-initiation, ...) |  | - | - |
| >Maximum Number of PSCells To Prepare | O |  | INTEGER (1..FFS, …) | Indicates the maximum number of PSCells that the target may prepare. | - | - |
| >CPAC Candidate PSCells REQD List |  | 0..1 |  |  | - | - |
| >>Candidate PSCells REQD Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | - | - |
| >>>PSCell ID | M |  | NR CGI 9.2.111 |  | – |  |
| >>>RRC Container | M |  | OCTET STRING | Includes CPCExecutionCondition as defined in subclause 6.2.x of TS 38.331 [31]. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofPScellCandidates | Maximum no. of PSCells for CPAC. Value is 8((FFS). |

- 9.1.4.18	SGNB CHANGE CONFIRM

This message is sent by the MeNB to inform the en-gNB about the successful change.

Direction: MeNB  en-gNB.

| IE/Group Name |  |  | Presence |  |  | Range |  |  | IE type and reference |  |  | Semantics description |  |  | Criticality |  |  | Assigned Criticality |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  |  | M |  |  |  |  |  | 9.2.13 |  |  |  |  |  | YES |  |  | reject |  |  |
| MeNB UE X2AP ID |  |  | M |  |  |  |  |  | eNB UE X2AP ID 9.2.24 |  |  | Allocated at the MeNB. |  |  | YES |  |  | ignore |  |  |
| SgNB UE X2AP ID |  |  | M |  |  |  |  |  | en-gNB UE X2AP ID 9.2.100 |  |  | Allocated at the en-gNB. |  |  | YES |  |  | ignore |  |  |
| E-RABs to be Released List |  |  |  |  |  | 0..1 |  |  |  |  |  |  |  |  | YES |  |  | ignore |  |  |
| >E-RABs To Be Released Item |  |  |  |  |  | 1 .. <maxnoofBearers> |  |  |  |  |  |  |  |  | – |  |  |  |  |  |
| >>E-RAB ID |  |  | M |  |  |  |  |  | 9.2.23 |  |  |  |  |  | – |  |  |  |  |  |
| >>EN-DC Resource Configuration |  |  | M |  |  |  |  |  | EN-DC Resource Configuration 9.2.108 |  |  | Indicates the PDCP and Lower Layer MCG/SCG configuration. |  |  | – |  |  |  |  |  |
| >>CHOICE Resource Configuration |  |  | M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>>PDCP present in SN |  |  |  |  |  |  |  |  |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "present". |  |  |  |  |  |  |  |  |
| >>>>UL Forwarding GTP Tunnel Endpoint |  |  | O |  |  |  |  |  | GTP Tunnel Endpoint 9.2.1 |  |  | Identifies the X2 transport bearer used for forwarding of UL PDUs |  |  | – |  |  |  |  |  |
| >>>>DL Forwarding GTP Tunnel Endpoint |  |  | O |  |  |  |  |  | GTP Tunnel Endpoint 9.2.1 |  |  | Identifies the X2 transport bearer used for forwarding of DL PDUs |  |  | – |  |  |  |  |  |
| >>>PDCP not present in SN |  |  |  |  |  |  |  |  |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |  |  |  |  |  |  |
| Criticality Diagnostics |  |  | O |  |  |  |  |  | 9.2.7 |  |  |  |  |  | YES |  |  | ignore |  |  |
| MeNB UE X2AP ID Extension |  |  | O |  |  |  |  |  | Extended eNB UE X2AP ID 9.2.86 |  |  | Allocated at the MeNB |  |  | YES |  |  | ignore |  |  |
| Conditional PSCell Change Information Confirm |  |  | O |  |  |  |  |  |  |  |  |  |  |  | YES |  |  | reject |  |  |
| >CPAC Candidate PSCells CONF List |  |  |  |  |  | 0..1 |  |  |  |  |  | the SN that initiated CPC is informed of the candidate PSCell IDs |  |  |  |  |  |  |  |  |
| >>Candidate PSCells CONF Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | - | - |
| >> PSCell ID |  |  | M |  |  |  |  |  | NR CGI 9.2.111 |  |  |  |  |  | – |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPScellCandidates | Maximum no. of PSCells for CPAC. Value is 8((FFS). |

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |

Next change, ommited text not changed

Next change, ommited text not changed

CPACcandidatePSCellsAck-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPACcandidatePSCellsAck-item

CPACcandidatePSCellsAck-item ::= SEQUENCE {

pscell-id						NRCGI,

rrc-container					RRCContainer,

iE-Extensions					ProtocolExtensionContainer { {CPACcandidatePSCellsACK-item-ExtIEs}}	OPTIONAL,

...

}

CPACcandidatePSCellsACK-item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CPACcandidatePSCellsREQD-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPACcandidatePSCellsREQD-item

CPACcandidatePSCellsREQD-item ::= SEQUENCE {

pscell-id						NRCGI,

rrc-container					RRCContainer,

iE-Extensions					ProtocolExtensionContainer { {CPACcandidatePSCellsREQD-item-ExtIEs}}	OPTIONAL,

...

}

CPACcandidatePSCellsREQD-item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CPACindicator ::= ENUMERATED {cpac-initiation, ...}

CPCindicator ::= ENUMERATED {cpc-triggered, ...}

CPAinformation-REQ ::= SEQUENCE {

cpac-indicator					CPACindicator,

max-no-of-pscells				INTEGER (1..maxnoofPSCellCandidates)	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {CPAinformation-REQ-ExtIEs} } OPTIONAL,

...

}

CPAinformation-REQ-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CPAinformation-ACK ::= SEQUENCE {

candidate-pscells					CPACcandidatePSCellsACK-list,

iE-Extensions		ProtocolExtensionContainer { {CPAinformation-ACK-ExtIEs} } OPTIONAL,

...

}

CPAinformation-ACK-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CPCinformation-REQD ::= SEQUENCE {

cpac-indicator					CPACindicator,

max-no-of-pscells				INTEGER (1..maxnoofPSCellCandidates)	OPTIONAL,

candidate-pscells				CPACcandidatePSCellsREQD-list,

iE-Extensions		ProtocolExtensionContainer { {CPCinformation-REQD-ExtIEs} } OPTIONAL,

...

}

CPCinformation-REQD-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CPCinformation-CONF ::= SEQUENCE {

candidate-pscells				CPACcandidatePSCellsCONF-list,

iE-Extensions		ProtocolExtensionContainer { {CPCinformation-CONF-ExtIEs} } OPTIONAL,

...

}

CPCinformation-CONF-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CPCinformation-NOTIFY ::= SEQUENCE {

cpc-indicator					CPCindicator,

iE-Extensions		ProtocolExtensionContainer { {CPCinformation-NOTIFY-ExtIEs} } OPTIONAL,

...

}

CPCinformation-NOTIFY-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}

CPACcandidatePSCellsCONF-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPACcandidatePSCellsCONF-item

CPACcandidatePSCellsCONF-item ::= SEQUENCE {

pscell-id						NRCGI,

iE-Extensions					ProtocolExtensionContainer { { CPACcandidatePSCellsCONF-item-ExtIEs}}	OPTIONAL,

...

}

CPACcandidatePSCellsCONF-item-ExtIEs X2AP-PROTOCOL-EXTENSION ::= {

...

}