3GPP TSG-RAN WG3 Meeting #113-e	R3-213316

# Online, Aug 16th – Aug 26th 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0639 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility Support for NR QoE Measurement Collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Mobility support for NR QoE management collection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | QoE-specific IEs are introduced in Handover Preparation and Retrieve UE Context procedures. The new QoE Measurement Configuration IE and IEs therein are defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mobility for NR QoE Management not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 8.2.4.2, 9.2.3.a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Change 1-------------------------------------------

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

If the UE Radio Capability ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

If the QoE Measurement Configuration IE is contained in the HANDOVER REQUEST, the NG-RAN node shall, if supported, take it into account for QoE measurements handling, as described in TS 38.401 [2].

-------------------------------------------Change 2-------------------------------------------

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

If the UE Radio Capability ID IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG- RAN node shall, if supported store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

If the QoE Measurement Configuration IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the NG-RAN node shall, if supported, take it into account for QoE measurements handling, as described in TS 38.401 [2].

-------------------------------------------Change 3-------------------------------------------

### 9.2.3.a QoE Measurement Configuration

The IE defines configuration information for the QoE Measurement Collection (QMC) function.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoE Measurement Configuration List |  | 0..1 |  |  | YES | ignore |
| >QoE Measurement Configuration Item |  | 1 .. <maxnoofQoEMeasConfig> |  |  | - |  |
| >>Service Type | M |  | ENUMERATED (QMC for DASH streaming, QMC for MTSI, QMC for VR, ...) | This IE indicates the service type of QoE measurements. | - | - |
| >>QoE Measurement Type | M |  | ENUMERATED (s-based, m-based, ...) |  | - | - |
| >>QoE Measurement Status | M |  | ENUMERATED (ongoing, ...) |  | - | - |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the trace activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 bytes Octet String. | - | - |
| >>Measurement Configuration Application Layer ID | M |  |  | This IE indicates the identity of the application layer measurement configuration, as defined in TS 38.331 [10]. | - | - |
| >>Alignment With MDT Required | O |  | ENUMERATED (true, false …) |  | - | - |
| >>Measurement Collection Entity IP Address | M |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. | - | - |
| >>Container for QoE measurement configuration | C-ifQoEMeasTypeSignalingBased |  | OCTET STRING (1..FFS) | Contains the QoE measurement configuration. | - | - |
| >>CHOICE Area Scope of QMC | C-ifQoEMeasTypeSignalingBased |  |  |  | - | - |
| >>>Cell based |  |  |  |  |  | - |
| >>>>Cell ID List for QMC |  | 1 .. <maxnoofCellIDforQMC> |  |  |  | - |
| >>>>>NR-CGI | M |  | 9.2.2.7 |  | - | - |
| >>>TA based |  |  |  |  |  | - |
| >>>>TA List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  |  | - |
| >>>>>TAC | M |  | 9.2.2.5 | The TAI is derived using the current serving PLMN. | - | - |
| >>>TAI based |  |  |  |  | - | - |
| >>>>TAI List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  | - | - |
| >>>>>TAI | M |  | 9.2.3.20 |  | - | - |
| >>>PLMN based |  |  |  |  |  | - |
| >>>>PLMN List for QMC |  | 1 .. <maxnoofPLMNforQMC> |  |  |  | - |
| >>>>>PLMN Identity | M |  | 9.2.2.4 |  | - | - |
| >>Slice for QMC List | C-ifQoEMeasTypeSignalingBased | 0..1 |  |  | - | - |
| >>>Slice for QMC Item |  | 1 .. <maxnoofPLMNforQMC> |  |  | - | - |
| >>>>PLMN Identity | M |  | 9.3.1.14 |  | - | - |
| >>>>S-NSSAI List |  | 1 |  |  | - | - |
|  |  | 1 .. <maxnoofSNSSAIforQMC> |  |  | - | - |
| >>>>>S-NSSAI |  |  | S-NSSAI 9.3.1.38 |  | - | - |

| Condition | Explanation |
| --- | --- |
| ifQoEMeasTypeSignalingBased | This IE shall be present if the QoE Measurement Type IE is set to the value “s-based” |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoEMeasConfig | Maximum no. of simultaneous QoE measurement configurations at a UE. Value is FFS. |
| maxnoofCellIDforQMC | Maximum no. of Cell IDs comprising the QMC scope. Value is FFS. |
| maxnoofTAforQMC | Maximum no. of TA comprising the QMC scope. Value is FFS. |
| maxnoofPLMNforQMC | Maximum no. of PLMNs in the PLMN list for QMC scope. Value is FFS. |
| maxnoofSNSSAIforQMC | Maximum no. of S-NSSAIs comprising the QMC scope. Value is FFS. |

-------------------------------------------End of changes-------------------------------------------