3GPP TSG-RAN3 #113-e	R3-213328

E-meeting, 16 – 26 August, 2021

Agenda Item:	13.2.3

Source:	CATT

Title:	CP-UP separation and inter-donor topology redundancy

Document for:	Discussion

Introduction

In this contribution, we further discuss CP-UP separation and inter-donor topology redundancy in detail respectively. Proposals are also provided to address the issues raised at pervious meetings.

Discussion

CP-UP separation

In the last meeting [1], we approached an agreement that “For donor-based IP-address allocation, the MN determines the F1-terminating node.” If IAB MT in NR-DC, both MN and SN could be a normal NG-RAN at this stage. And then, IAB DU would initial F1 setup procedure with donor CU based on MN indication. MN should inform IAB node about who is the donor (MN or SN) via RRC message which include IP address of donor CU. It triggers the F1 setup procedure for IAB DU.

Proposal 1: MN should inform IAB node about who is the donor (MN or SN) via RRC message which also include IP address of donor CU.

Topology redundancy

Mapping

A. Bear mapping at boundary node

The F1-terminating donor sends the QoS information (content FFS) to the non-F1-terminating donor with the granularity of BH RLC CH or F1-U GTP-U tunnel for UP traffic, or non-UP traffic type for non-UP traffic (FFS whether for UP traffic we go for the 1st or the latter option, or both)

We have the following agreement at last meeting [1].

The granularity of QoS depends on whether the BH RLC channel required by F1-termination donor can be accepted by non F1-termination donor. For example, if 10 F1-Us map to 3 BH RLC channel in topology 1 (F1-termination donor) and these 10 F1-Us are also mapped to 3 BH RLC channel in topology 2 (non F1-termination donor) at boundary node. The granularity of QoS can be BH RLC CH.

However, the BH RLC channel required by F1-termination donor may not acceptable by non F1-termination node. For example, there are 10 F1-Us map to 3 BH RLC channels in topology 1. The F1-termination donor sends QoS and the requirement of 3 BH RLC channels to non F1-termination donor for load balance. The non-F1-termination donor may not be able to satisfy the same mapping rules, e.g., 5 BH RLC channels are needed for this QoS in non-F1-termination node. It will lead to remapping at boundary node. The granularity should be F1-U GTP-U tunnel in this scenario.  Namely, the finial mapping decision should be made by non F1-termination node.

Proposal 2: Non F1-termination node determines the traffic mapping (F1-U to BH RLC CH) in non F1-termination topology.

To achieve inter-CU load balance, F1-termination donor should provide the QoS and optionally send mapping information e.g., 10 F1-Us map to 3 BH RLC channels to non F1-termination node. Non F1-termination node provides the BH RLC CH ID(s) for UL to the F1-termination node which is try to follow the UL mapping information in F1-termination donor. If non F1-termination donor cannot satiety the UL mapping in F1-termination donor, a F1-U to BH RLC CH mapping table should be included in Xn signaling from non F1-termination donor to F1-termination donor e.g., F1-U1 and F1-U2 map to BH RLC CH1. The question is how to distinguish different F1-Us at boundary node e.g. via BAP routing ID.

Proposal 3: For bear mapping at boundary node,

F1-termination donor sends the following information to the non-F1 termination donor:

- QoS information (without specific granularity)

- Optional, UL mapping information (F1-U to BH RLC CH) in F1-termination donor

Non F1-termination donor sends the following information to the F1 termination donor:

- BH RLC CH ID(s)

- Optional, UL mapping information (F1-U to BH RLC CH) in non F1-termination donor. FFS on how to distinguish different F1-Us at boundary node e.g. via BAP routing ID.

B. IP to Layer-2 mapping at target donor DU

For inter-donor topology redundancy, the IP to layer-2 mapping should be performed at non F1-termination donor DU. This IP to layer-2 mapping information list is configured by non F1-termination donor CU. Since the F1 connection is between F1-termination donor and boundary node, non F1-termination donor needs to provide the IP header configuration information to F1-termination donor CU. Then the F1-termination node configures the IP header for packets planned to be routed. The destination TNL address and DSCP/flow is included in IP header. For the route between non F1-termination donor DU and boundary node, the destination TNL address should be the destination node’s IP address in non F1-termination donor.

Proposal 4: For IP to Layer-2 mapping at target donor DU for DL,

Non F1-termination donor sends the following information to the F1 termination donor:

- IP address of destination node in non F1-termination donor

- DSCP/flow label in non F1-termination donor

Routing across boundary node

RAN2 said that they prefer option 4 in [2]. However, it does not mean that they have precluded option 5 because option 5 is mainly in RAN3 scope. Generally, the solutions of routing across boundary node are limited in option 4 and 5 at this stage.

- opt4 BAP header rewriting based on BAP routing ID at e.g. the boundary node

- opt5 BAP header rewriting based on IP header at, e.g., the boundary node

Option 4: BAP header rewriting at boundary node requires a mapping table. It allows boundary node to map BAP routing ID 1 to (BAP routing ID 2 [3]. Given an example for DL (UL is similar), once receives the packet with (Path1, BAP3) from redundancy donor CU then boundary node maps it to (Path2, BAP5). BAP3 is the boundary node’s BAP address, BAP5 is the final destination BAP address of packet. Further, if the destination of packet is boundary node, then another path ID need to be introduced e.g., Path5. Actually, Path5 and Path1 is the same path but it represents whether a mapping is required at boundary node. Therefore, option 4 requires more path IDs.

Proposal 5: Boundary node distinguishes whether to perform BAP header mapping procedure via path ID for option 4.

How to option 4 used in topology redundancy:

For DL, the destination IP address of packet is the IP address of descendant node allocated by non F1-termination donor, while the destination BAP address is the boundary node’s BAP address. Boundary node only operates at BAP layer i.e., rewrite the BAP header according to the BAP header mapping table. The destination BAP address is changed to the descendant node’s BAP address. For UL, the destination IP address of packet is the IP address of non F1-termination donor DU, while the destination BAP address is the boundary node’s BAP address.

Proposal 6: Option 4 operation in topology redundancy: destination IP address is the descendant node and non F1-termination donor DU for DL and UL respectively. While the destination BAP address is the boundary node’s BAP address.

Option 5: The issue of this option is that it requires boundary to see IP header which is against R16 principle. The destination IP address in IP header is seen by boundary node should be the real destination (e.g., descendant node). The boundary node writes the BAP routing information base on the IP to layer-2 traffic mapping information configured by F1-termination donor.

Option 4 and option 5 both need to configure a mapping table to boundary node. The difference between them is that one is the BAP routing ID mapping table, the other is IP to layer 2 mapping table. Furthermore, option 4 requires negotiation between two donors while option 5 does not need since the IP address is unique and each donor CU configures their own IP to layer 2 mapping. The main problem of option 5 is how to recognize the real destination IP address in the IP header of packet at boundary node.

Observation 1: Option 4 needs negotiation between two CU, while option 5 does not need because IP address is unique.

Proposal 7: FFS on how to make boundary node see the destination IP address in the IP header of descendant node traffic for option 5.

Conclusion

Proposal 1: MN should inform IAB node about who is the donor (MN or SN) via RRC message which also include IP address of donor CU.

Proposal 2: Non F1-termination node determines the traffic mapping (F1-U to BH RLC CH) in non F1-termination topology.

Proposal 3: For bear mapping at boundary node,

F1-termination donor sends the following information to the non-F1 termination donor:

- QoS information (without specific granularity)

- Optional, UL mapping information (F1-U to BH RLC CH) in F1-termination donor

Non F1-termination donor sends the following information to the F1 termination donor:

- BH RLC CH ID(s)

- Optional, UL mapping information (F1-U to BH RLC CH) in non F1-termination donor. FFS on how to distinguish different F1-Us at boundary node e.g. via BAP routing ID.

Proposal 4: For IP to Layer-2 mapping at target donor DU for DL,

Non F1-termination donor sends the following information to the F1 termination donor:

- IP address of destination node in non F1-termination donor

- DSCP/flow label in non F1-termination donor

Proposal 5: Boundary node distinguishes whether to perform BAP header mapping procedure via path ID for option 4.

Proposal 6: Option 4 operation in topology redundancy: destination IP address is the descendant node and non F1-termination donor DU for DL and UL respectively. While the destination BAP address is the boundary node’s BAP address.

Observation 1: Option 4 needs negotiation between two CU, while option 5 does not need because IP address is unique.

Proposal 7: FFS on how to make boundary node see the destination IP address in the IP header of descendant node traffic for option 5.

References

[1] Chairman notes, R3#112-e

[2] R2-2106707, LS on Topology Adaptation enhancements, Nokia

[3] R2- 2103083, Report [Post113-e][058][IAB17] Inter-donor topology adaptation, QC