3GPP TSG-RAN WG3 Meeting #113-e	R3-213355

# E-meeting, 16-26 Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.473 | CR | 0010 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Paging over W1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_arch_evo-Core |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In case of non-3GPP origin paging, once the 5GC informs the ng-eNB of the Paging Origin of non-3GPP in NGAP: PAGING message, the ng-eNB-CU needs to propagate it to the ng-eNB-DU, but in current W1AP: PAGING message, the Paging Origin IE is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Paging Origin IE in W1AP: PAGING message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support non-3GPP origin paging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.1.2, 9.2.5.1, 9.3.1.xx (new), 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------- Start of the First Change -----------------

8.6.1	Paging

8.6.1.1	General

The purpose of the Paging procedure is used to provide the paging information to enable the ng-eNB-DU to page a UE. The procedure uses non-UE associated signalling.

8.6.1.2	Successful Operation

Figure 8.6.1.2-1: Paging procedure. Successful operation.

The ng-eNB-CU initiates the procedure by sending a PAGING message.

The Paging DRX IE may be included in the PAGING message, and if present the ng-eNB-DU may use it to determine the final paging cycle for the UE.

At the reception of the PAGING message, the ng-eNB-DU shall perform paging of the UE in cells which belong to cells as indicated in the Paging Cell List IE.

The Paging Origin IE may be included in the PAGING message, and if present the ng-eNB-DU shall transfer it to the UE.

8.6.1.3	Abnormal Conditions

Not applicable.

-------------- Start of the Next Change -----------------

9.2.5	Paging messages

9.2.5.1	PAGING

This message is sent by the ng-eNB-CU and is used to request the ng-eNB-DU to page UEs.

Direction: ng-eNB-CU  ng-eNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.3.1.63 |  | YES | reject |
| CHOICE Paging Identity | M |  |  |  | YES | reject |
| >RAN UE Paging identity | M |  | 9.3.1.64 |  | - |  |
| >CN UE paging identity | M |  | 9.3.1.65 |  | - |  |
| Paging DRX | O |  | 9.3.1.66 | It is defined as the minimum between the RAN UE Paging DRX and CN UE Paging DRX | YES | ignore |
| Paging Cell List |  | 1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >> E-UTRAN CGI | M |  | 9.3.1.12 |  | - |  |
| Paging Origin | O |  | 9.3.1.xx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

-------------- Start of the Next Change -----------------

9.3.1.xx	Paging Origin

This IE indicates whether Paging is originated due to the PDU sessions from the non-3GPP access.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Origin | M |  | ENUMERATED (non-3GPP, …) |  |

-------------- Start of the Next Change -----------------

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for W1AP.

--

-- **************************************************************

W1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) w1ap (3) version1 (1) w1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Candidate-SpCell-Item,

Cause,

Cells-Failed-to-be-Activated-List-Item,

Cells-Status-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

C-RNTI,

CriticalityDiagnostics,

CUtoDURRCInformation,

DRB-Activity-Item,

DRBID,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRB-Notify-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DUtoCURRCInformation,

EUTRANQoS,

NGENB-CU-UE-W1AP-ID,

NGENB-DU-UE-W1AP-ID,

NGENB-DU-ID,

NGENB-DU-Served-Cells-Item,

NGENB-DU-System-Information,

InactivityMonitoringRequest,

InactivityMonitoringResponse,

NotificationControl,

EUTRANCGI,

EUTRANPCI,

Potential-SpCell-Item,

RAT-FrequencyPriorityInformation,

ResourceCoordinationTransferContainer,

RRCContainer,

RRCReconfigurationCompleteIndicator,

SCellIndex,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

SCell-FailedtoSetup-Item,

SCell-FailedtoSetupMod-Item,

ServCellIndex,

Served-Cell-Information,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

SRBs-Modified-Item,

SRBs-Setup-Item,

SRBs-SetupMod-Item,

TransactionID,

TransmissionActionIndicator,

DUtoCURRCContainer,

PagingCell-Item,

UEIdentityIndexValue,

UE-associatedLogicalW1-ConnectionItem,

PagingDRX,

PagingIdentity,

PagingOrigin,

PWSSystemInformation,

Broadcast-To-Be-Cancelled-Item,

Cells-Broadcast-Cancelled-Item,

E-UTRAN-CGI-List-For-Restart-Item,

PWS-Failed-E-UTRAN-CGI-Item,

RepetitionPeriod,

NumberofBroadcastRequest,

Cells-To-Be-Broadcast-Item,

Cells-Broadcast-Completed-Item,

Cancel-all-Warning-Messages-Indicator,

NotificationInformation,

EUTRA-NR-CellResourceCoordinationReq-Container,

EUTRA-NR-CellResourceCoordinationReqAck-Container,

RequestType,

PLMN-Identity,

BitRate,

NGENBDUOverloadInformation,

ResourceCoordinationTransferInformation,

IgnoreResourceCoordinationRequestContainer,

SCGIndicator

FROM W1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

W1AP-PRIVATE-IES,

W1AP-PROTOCOL-EXTENSION,

W1AP-PROTOCOL-IES,

W1AP-PROTOCOL-IES-PAIR

FROM W1AP-Containers

id-Candidate-SpCell-List,

id-Cause,

id-Cancel-all-Warning-Messages-Indicator,

id-NotificationInformation,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Status-List,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Deactivated-List,

id-ConfirmedUEID,

id-C-RNTI,

id-CUtoDURRCInformation,

id-CriticalityDiagnostics,

id-DRB-Activity-List,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-List,

id-DRB-Notify-List,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-List,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-ngeNB-CU-UE-W1AP-ID,

id-ngeNB-DU-UE-W1AP-ID,

id-ngeNB-DU-ID,

id-ngeNB-DU-Served-Cells-List,

id-InactivityMonitoringRequest,

id-InactivityMonitoringResponse,

id-oldngeNB-DU-UE-W1AP-ID,

id-Potential-SpCell-List,

id-RAT-FrequencyPriorityInformation,

id-ResetType,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-RRCReconfigurationCompleteIndicator,

id-SCell-FailedtoSetup-List,

id-SCell-FailedtoSetupMod-List,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-List,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-List,

id-ServCellIndex,

id-SpCell-ID,

id-SRBID,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-List,

id-SRBs-Modified-List,

id-SRBs-Setup-List,

id-SRBs-SetupMod-List,

id-TransactionID,

id-TransmissionActionIndicator,

id-UE-associatedLogicalW1-ConnectionListResAck,

id-DUtoCURRCContainer,

id-EUTRANCGI,

id-PagingCell-List,

id-PagingDRX,

id-UEIdentityIndexValue,

id-PagingIdentity,

id-PagingOrigin,

id-PWSSystemInformation,

id-RepetitionPeriod,

id-NumberofBroadcastRequest,

id-Cells-To-Be-Broadcast-List,

id-Cells-Broadcast-Completed-List,

id-Broadcast-To-Be-Cancelled-List,

id-Cells-Broadcast-Cancelled-List,

id-E-UTRAN-CGI-List-For-Restart-List,

id-PWS-Failed-E-UTRAN-CGI-List,

id-EUTRA-NR-CellResourceCoordinationReq-Container,

id-EUTRA-NR-CellResourceCoordinationReqAck-Container,

id-RequestType,

id-ServingPLMN,

id-NGENB-DU-UE-AMBR-UL,

id-NGENBDUOverloadInformation,

id-ResourceCoordinationTransferInformation,

id-IgnoreResourceCoordinationRequestContainer,

id-SCGIndicator,

maxCellinngeNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualW1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs

FROM W1AP-Constants;

//skip the changed part

-- **************************************************************

--

-- Paging PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Paging

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PagingIEs}},

...

}

PagingIEs W1AP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue	CRITICALITY reject	TYPE UEIdentityIndexValue		PRESENCE mandatory	}|

{ ID id-PagingIdentity			CRITICALITY reject	TYPE PagingIdentity				PRESENCE mandatory	}|

{ ID id-PagingDRX				CRITICALITY ignore	TYPE PagingDRX					PRESENCE optional	}|

{ ID id-PagingCell-List			CRITICALITY ignore	TYPE PagingCell-list			PRESENCE mandatory	}|

{ ID id-PagingOrigin			CRITICALITY ignore	TYPE PagingOrigin				PRESENCE optional	},

...

}

PagingCell-list::= SEQUENCE (SIZE(1.. maxnoofPagingCells)) OF PagingCell-Item

-------------- Start of the Next Change -----------------

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

W1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) w1ap (3) version1 (1) w1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-NotificationInformation,

id-AdditionalSIBMessageList,

maxEARFCN,

maxnoofAdditionalSIBs,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofDLUPTNLInformation,

maxnoofE-UTRANCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

maxBandsEutra

FROM W1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM W1AP-CommonDataTypes

ProtocolExtensionContainer{},

W1AP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

W1AP-PROTOCOL-IES

FROM W1AP-Containers;

//skip the changed part

-- P

PacketDelayBudget ::= INTEGER (0..1023,...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar			PER-Scalar,

pER-Exponent		PER-Exponent,

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

PER-Scalar ::= INTEGER (0..9,...)

PER-Exponent ::= INTEGER (0..9,...)

PagingCell-Item ::= SEQUENCE {

eUTRANCGI		EUTRANCGI,

iE-Extensions	ProtocolExtensionContainer { { PagingCell-ItemExtIEs } }	OPTIONAL,

...

}

PagingCell-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

PagingDRX ::= ENUMERATED {

v32,

v64,

v128,

v256,

...

}

PagingIdentity ::=	CHOICE {

rANUEPagingIdentity	RANUEPagingIdentity,

cNUEPagingIdentity	CNUEPagingIdentity,

choice-extension			ProtocolIE-SingleContainer { { PagingIdentity-ExtIEs } }

}

PagingIdentity-ExtIEs W1AP-PROTOCOL-IES::= {

...

}

PagingOrigin ::= ENUMERATED { non-3gpp,	...}

PDCP-SN ::= INTEGER (0..4095)

PDCPSNLength	::= ENUMERATED { twelve-bits,eighteen-bits,...}

PDUSessionID ::= INTEGER (0..255)

Ph-InfoSCG  ::= OCTET STRING

PLMN-Identity ::= OCTET STRING (SIZE(3))

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

PriorityLevel	::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

-------------- Start of the Next Change -----------------

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

W1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) w1ap (3) version1 (1) w1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM W1AP-CommonDataTypes;

//skip the changed part

id-DUtoCURRCContainer									ProtocolIE-ID ::= 66

id-DRB-Notify-List										ProtocolIE-ID ::= 67

id-NotficationControl									ProtocolIE-ID ::= 68

id-PWSSystemInformation									ProtocolIE-ID ::= 69

id-RepetitionPeriod										ProtocolIE-ID ::= 70

id-NumberofBroadcastRequest								ProtocolIE-ID ::= 71

id-Cells-To-Be-Broadcast-List							ProtocolIE-ID ::= 72

id-Cells-Broadcast-Completed-List 						ProtocolIE-ID ::= 73

id-Broadcast-To-Be-Cancelled-List 						ProtocolIE-ID ::= 74

id-Cells-Broadcast-Cancelled-List 						ProtocolIE-ID ::= 75

id-E-UTRAN-CGI-List-For-Restart-List 					ProtocolIE-ID ::= 76

id-PWS-Failed-E-UTRAN-CGI-List 							ProtocolIE-ID ::= 77

id-ConfirmedUEID										ProtocolIE-ID ::= 78

id-Cancel-all-Warning-Messages-Indicator				ProtocolIE-ID ::= 79

id-NGENB-DU-UE-AMBR-UL									ProtocolIE-ID ::= 80

id-ServingPLMN											ProtocolIE-ID ::= 81

id-NGENBDUOverloadInformation							ProtocolIE-ID ::= 82

id-ResourceCoordinationTransferInformation				ProtocolIE-ID ::= 83

id-SRBs-Setup-List										ProtocolIE-ID ::= 85

id-SRBs-SetupMod-List									ProtocolIE-ID ::= 86

id-SRBs-Modified-List									ProtocolIE-ID ::= 87

id-IgnoreResourceCoordinationRequestContainer			ProtocolIE-ID ::= 88

id-NotificationInformation								ProtocolIE-ID ::= 89

id-AdditionalSIBMessageList								ProtocolIE-ID ::= 90

id-MeasGapSharingConfig									ProtocolIE-ID ::= 91

id-SCGIndicator											ProtocolIE-ID ::= 92

id-PagingOrigin											ProtocolIE-ID ::= xx

END

-- ASN1STOP

-------------- End  of the Changes -----------------