3GPP TSG-RAN3 #113-e	R3-213362

16th August – 26th August 2021

Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0164 | rev | 3 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC Re-establishment and inter-vendor CU operation CR 38.401 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Verizon Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently in TS 38.401 in RRC Connection reestablishment the following note is cited. NOTE: If UE accesses from a gNB-DU other than the original one, gNB-CU should trigger UE Context Setup procedure towards this new gNB-DU. Currently, the only case that is mentioned is when the UE accesses from a gNB-DU other than the original one. The question we would like to raise is what happens in the case that the UE access the same gNB-DU but still the gNB-DU was not able to retrieve the previous UE context, how other-vendor CU will operate in this case? Since the gNB-DU was not able to access the context associated to the old F1AP IDs, hence it has issued a UE context release for that context. At the same time, a new F1 signalling connection has been established for the new F1AP IDs. This signalling connection leads to a new UE context setup. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that if UE access the same gNB-DU, then other vendor CU will trigger UE Context Setup procedure towards the same gNB-DU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclarities regarding inter-vendon operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<< NEXT CHANGE >>>>>>

## 8.7	RRC connection reestablishment

This procedure is used for the case that UE tries to reestablish the RRC connection, as shown in Figure 8.7-1.

Figure 8.7-1: RRC connection reestablishment procedure

1.	The UE sends a preamble to the gNB-DU.

2.	The gNB-DU allocates new C-RNTI and responds with RAR.

3.	The UE sends an RRCReestablishmentRequest message to the gNB-DU, which contains old C-RNTI and old PCI.

4.	The gNB-DU includes the RRC message and, if the UE is admitted, the corresponding low layer configuration for the UE in the INITIAL UL RRC MESSAGE TRANSFER message and transfers to the gNB-CU. The INITIAL UL RRC MESSAGE TRANSFER message includes the new C-RNTI.

5.	The gNB-CU includes an RRCReestablishment message and transfers to the gNB-DU. If the UE requests to re-establish RRC connection in the last serving gNB-DU, the DL RRC MESSAGE TRANSFER message shall include old gNB-DU UE F1AP ID.

6.	The gNB-DU retrieves the UE context based on the old gNB-DU UE F1AP ID, and replaces old C-RNTI/PCI with new C-RNTI/PCI. It sends the RRCReestablishment message to UE.

7-8.	The UE sends an RRCReestablishmentComplete message to the gNB-DU. The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends to the gNB-CU.

9-10.	The gNB-CU triggers an UE Context Modification procedure by sending UE CONTEXT MODIFICIATION REQUEST message, which may include DRBs to be modified and released list. The gNB-DU responses with the UE CONTEXT MODIFICATION RESPONSE message.

9'-10'.	The gNB-DU triggers an UE Context Modification procedure by sending UE CONTEXT MODIFICIATION REQUIRED message, which may include DRBs to be modified and released list. The gNB-CU responses with UE CONTEXT MODIFICATION CONFIRM message.

NOTE:	Here it is assumed that the UE accessed the original gNB-DU where the UE context is available for that UE, and either steps 9-10 or steps 9’-10’ may be executed or both could be skipped.

NOTE:	If the UE accessed from a gNB-DU other than the original one, the gNB-CU should trigger the UE Context Setup procedure toward this new gNB-DU. If the original gNB-DU is not capable of retrieving the UE context, the gNB-CU should trigger the UE Context Setup procedure towards the original gNB-DU

11-12.	The gNB-CU includes an RRCReconfiguration message into the DL RRC MESSAGE TRANSFER message and transfers to the gNB-DU. The gNB-DU forwards it to the UE.

13-14.	The UE sends an RRCReconfigurationComplete message to the gNB-DU, and the gNB-DU forwards it to the gNB-CU.

<<<<<< END OF CHANGES >>>>>>