3GPP TSG-RAN WG3#113-e                                             	                     R3-213443

E-Meeting, 16 – 26 August 2021

Agenda item:	9.3.4.1

Source:	Nokia, Nokia Shanghai Bell, Deutsche Telekom

Title: 	Correction of NG handover

Document for:	Approval

# Discussion

NG handover

For NG handover we have the following IE in the Source NG-RAN to Target NG-RAN container IE in section 9.3.1.29:

| >>QoS Flow Information List |  | 1 |  |  | - |  |
| --- | --- | --- | --- | --- | --- | --- |
| >>>QoS Flow Information Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>>>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>>>DL Forwarding | O |  | 9.3.1.33 |  | - |  |
| >>>>UL Forwarding | O |  | 9.3.1.118 |  | YES | reject |
| >>DRBs to QoS Flows Mapping List | O |  | 9.3.1.34 |  | - |  |

And usage in section 8.4.1.2:

If the DL Forwarding IE is included for a given QoS flow in the PDU Session Resource Information Item IE within the Source NG-RAN node to Target NG-RAN node Transparent Container IE of the HANDOVER REQUIRED message and it is set to "DL forwarding proposed", it indicates that the source NG-RAN node proposes forwarding of downlink data for that QoS flow.

The forwarding decision is per QoS flow in the above statement.

However, when the flow-DRB mapping is sent another text states:

If the DRBs to QoS Flows Mapping List IE is included in the PDU Session Resource Information Item IE within the Source NG-RAN node to Target NG-RAN node Transparent Container IE of the HANDOVER REQUIRED message, it implicitly indicates that the source NG-RAN node proposes forwarding of downlink data for those DRBs

The sentence highlighted in yellow associate the sending of flow-DRB mapping to a forwarding proposal.

Observation 1: the highlighted sentence for NG handover introduces a dependency between sending the flow-DRB mapping and data forwarding proposal of associated QoS flows.

However, the DRBs to QoS Flows Mapping List IE includes itself a list of QoS flows as described in section 9.3.1.34:

Observation 2: The sentence highlighted in yellow for NG handovers introduces a redundant requirement for the data forwarding of a QoS flow.

The redundancy creates two possible interpretations:

Interpretation 1

The QoS Flow Information List IE contains the full list of QoS flows proposed for forwarding:

- Some of them are proposed to be forwarded via the associated DRB tunnel in which case the QFI is also present in the DRBs to QoS Flows Mapping List IE.

- Some of them are proposed to be forwarded over the PDU session tunnel in which case the QFI shall not be present in any DRBs to QoS Flows Mapping List IE.

This interpretation assumes that:

- When a DRB forwarding tunnel is setup, not all QoS flows mapped on the DRB may be forwarded;

- When a DRB forwarding tunnel is setup, all of those mapped QoS flows which are proposed to be forwarded shall be forwarded over that DRB tunnel.

Interpretation 2

The QoS Flow Information List IE contains only the list of QoS flows which are proposed for forwarding over the PDU session tunnel.

This interpretation assumes that a QFI present in the QoS Flow Information List IE shall not be present in any DRBs to QoS Flows Mapping List IE.

It is critical for the encoding to say clearly which of these two coding interpretations is valid.

# Conclusion and Proposal

This paper has described that NG handover has a statement including redundant and possibly contradicting data forwarding requirements for a same QoS flow. It makes the following proposal:

Proposal 1: clarify which encoding interpretation is valid for the relationship between the DL Forwarding IE and the DRBs to QoS Flows Mapping List IE. Interpretation 1 is proposed with the CR in [2].

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- R3-213444, Correction of NG handover