3GPP TSG-RAN WG3 Meeting #113-e	R3-213448

Online, 16 – 26 August 2021

Agenda item:	21.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR_IIOT_URLLC_enh BL CR for TS 38.423) Impact of handover on Survival Time

Document for:	Discussion and Decision

# 1	Introduction

RAN3 has agreed to introduce survival time as a new parameter in the TSC Assistance Information (TSCAI). This agreement has been captured in baseline CRs for NGAP [1], XnAP [2], E1AP [3], and F1AP [4].

In this paper, we further analyze the impacts of survival time on RAN3 specifications, in particular the effect of handover on survival time.

# 2	Discussion

Survival time is defined in TS 22.104 as follows:

survival time: the time that an application consuming a communication service may continue without an anticipated message.

Survival time (ST) is applicable to periodic deterministic communication, where a transmission (data burst) occurs every transfer interval. The ST is expressed in units of time and is typically a multiple of the transfer interval. An ST of zero indicates that no data loss can be tolerated.

Consider the following example: Let the ST be equal to 3 times the transfer interval. This means that no more than 3 consecutive packets can be lost. If the transmission of three consecutive packets fail, then the next packet must be transmitted with very high reliability such that it is almost certain to be successfully received by the UE. If we define “available survival time” (AST) as the remaining survival time of the total ST within which the next packet must be transmitted successfully with very high reliability, then in this example:

-	After three consecutive packets fail, the AST is 0.

-	After two consecutive packets fail, the AST is 1.

-	If the previous packet is received successfully, then the AST is equal to the ST (i.e. 3).

It is essential for the target gNB to know the AST, so it can determine the level of reliability that is needed when transmitting the first packet following handover. In the above example:

-	If the AST received by the target gNB is 3 (i.e. the last packet was successfully delivered at the source gNB), then the target gNB does not need to send the first packet following handover with very high reliability.

-	If the AST received by the target gNB is 0 (i.e. the last 3 packets were not successfully delivered at the source gNB) or e.g. 1, then the target gNB must send the first packet following handover with very high reliability.

Without signaling the AST from source to target via Xn, the target gNB must always assume that it must transmit the next packet with very high reliability since it does not know if previous tranmission(s) failed, i.e. the target gNB must assume that AST is 0.

Proposal: 	Introduce Available Survival Time within the TSC Assistance Information IE transferred over Xn and F1.

# 3	Conclusions

In this paper, we discussed the effects of handover on survival time.  The following is proposed:

Proposal: 	Introduce Available Survival Time within the TSC Assistance Information IE transferred over Xn and F1.

A TP for XnAP reflecting the above proposal is provided in Appendix A.

# References

R3-212900 Introduction of Enhanced IIoT support over NG, CATT, Nokia, Nokia Shanghai Bell, Samsung, Huawei, Ericsson, ZTE

R3-212901 Introduction of Enhanced IIoT support over Xn, Ericsson, Samsung, Huawei, ZTE, CATT, Nokia, Nokia Shanghai Bell,

R3-212902 Introduction of Enhanced IIoT support over E1, ZTE, Nokia, Nokia Shanghai Bell, Samsung, CATT, Huawei, Ericsson

R3-212903 Introduction of Enhanced IIoT support over F1, Huawei, Nokia, Nokia Shanghai Bell, CATT, Ericsson, ZTE, Samsung

# A	Appendix: Text Proposal for TS 38.423 BL CR

Start of Text Proposal for TS 38.423 BL CR

#### 9.2.3.xxx	Survival Time

This IE provides the Survival Time for a TSC QoS flow (see TS 23.501 [7]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0..6400000 (FFS), …) | Expressed in units of 1 us (FFS). |
| Available Survival Time | M |  | INTEGER (0..6400000 (FFS), …) | Survival time that remains following handover. Expressed in units of 1 us (FFS). |

Editor’s note: The maximum value and the time unit granularity for the Survival Time are FFS.

End of Text Proposal for TS 38.423 BL CR