3GPP TSG-RAN3 Meeting #113-e	R3-213501

16 - 26 Aug 2021 E-Meeting

Title: 	(TP on SN change failure for 38.423) Addition of SCG Failure procedure for SN change failure

Source: 	CATT

Agenda item:	10.2.1.6

Document Type:	other

# 1. Introduction

This contribution is the TP for TS38.423 v16.6.0 with addition of SCG Failure procedure for SN change failure.

# 2. Discussion

As discussed in R3-213500, a class-1 procedure needs to be introduced for SN to inform MN whether to make optimization when transmitting SCGFailureInformation from the MN to the SN.

# 3. Annex – TP

Start of the first change

8	XnAP procedures

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| SCG Failure Information | SCG FAILURE INFORMATION REQUEST | SCG FAILURE INFORMATION RESPONSE |  |

8.3.X	SCG Failure Information

8.3.X.1	General

The purpose of the SCG Failure Information procedure is to transfer SCG failure related information from M-NG-RAN to S-NG-RAN. S-NG-RAN informs M-NG-RAN whether makes optimization for SCG failure.

The procedure uses UE-associated signalling.

8.3.X.2	Successful Operation

Figure 8.3.X.2-1: SCG Failure Information procedure, successful operation.

The M-NG-RAN-NODE initiates the procedure by sending the SCG FAILURE INFORMATION message to the S-NG-RAN-NODE.

If MN Optimization Indicator IE is included in SCG FAILURE INFORMATION RESPONSE message, M-NG-RAN-NODE shall make optimization as the indication.

8.3.X.3	Abnormal Conditions

Not applicable.

9.1.2.X	SCG FAILURE INFORMATION REQUEST

This message is sent by M-NG-RAN-NODE to transfer SCG failure related information to S-NG-RAN-NODE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| SCG Failure Information Container | M |  | OCTET STRING | scgFailureInfo as defined in subclause 11.2.2 in TS 38.331 [10]. | YES | ignore |
| Next PSCell | M |  | 9.2.3.25 Target Cell Global ID | The next suitable PSCell | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |

9.1.2.X	SCG FAILURE INFORMATION RESPONSE

This message is sent by the S-NG-RAN node to inform the M-NG-RAN node that M-NG-RAN node shall make optimization for SCG failure.

Direction: S-NG-RAN node  M-NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| MN Optimization Indicator | O |  | ENUMERATED (true, …) | Indicates whether MN needs make optimization | YES | ignore |

End of the last change