3GPP TSG-RAN WG3 #113 electronic	R3-213545

Online, 16-26 Aug, 2021

Agenda Item:	22.2.4

Source:	CATT, CBN

Title:	Consideration on MBS Session Management at F1E1 interface

Document for:	Discussion, Decision

1	Introduction

On MBS session resource over F1/E1, it has been discussed in RAN3 for three meetings and some progress has been made, as described below:

RAN3#110-e meeting:

Use a shared F1-U tunnel for PTM transmission of an MBS radio bearer for an MBS Session within one cell.

If multiple MBS radio bearers could be established for an MBS Session, a shared F1-U tunnel should be used for PTM transmission of these MBS radio bearers of the same MBS Session within one cell.

Use non-UE associated F1/E1 procedures to setup the shared F1-U tunnel.

Support the method that gNB-DU assigns the DL F1-U GTP-U tunnel info, provides it to gNB-CU-CP and then gNB-CU-CP forwards it to gNB-CU-UP.

Provide the MBS Session id, Qos profile from gNB-CU to gNB-DU.

Provide the MBS Session id, Qos profile from gNB-CU-CP to gNB-CU-UP.

F1/E1 MBS Bearer management procedure can be discussed, but details on e.g. information to signal are pending RAN2/SA2 progress

WA: gNB DU assignes the G-RNTI, pending to RAN2 confirmation.

RAN3#112-e meeting:

WA: Standard shall enable a one to one mapping between an MRB and a shared F1-U tunnel

WA: F1-U multicast transport is not supported

Use non-UE associated F1/E1 procedures to set up the MBS context and shared F1-U tunnel(s) for a broadcast session (MBS context is used in analogy to UE context)

For broadcast, an MBS context ID may be associated to one or more MRB IDs, to be included in the non-UE-associated F1AP procedure (procedure and IEs are FFS)

Flow control should be enabled for an MRB established for a broadcast MBS session.

WA: For broadcast session, agree to introduce the following gNB-CU-CP triggered F1AP procedures: MBS Context Setup, MBS Context Modification, MBS Context Release. Message name, scope, association with other F1AP procedures and potential alignment with multicast F1AP procedures are FFS

WA: For broadcast session, agree to introduce the following gNB-CU-CP triggered E1AP procedures: MBS Bearer Setup, MBS Bearer Modification, MBS Bearer Release. Message name, scope, association with other E1AP procedures and potential alignment with multicast E1AP procedures are FFS.

However, on the topics related to session management over F1/E1, only a little progress was made in RAN3, many issues need to be addressed, as described below：

FFS on whether to introduce new type of signaling connection.

FFS on whether to introduce the gNB-DU triggered F1AP procedures for broadcast session: MBS Context Modification, MBS Context Release, further justification is needed.

FFS on whether to introduce the gNB-CU-UP triggered E1AP procedures for broadcast session: MBS Bearer Modification, MBS Bearer Release, further justification is needed.

FFS on which name should be used for the non-UE associated F1/E1 procedures for broadcast session, and alignment with TS 23.247 is recommended.

FFS on how to setup the corresponding MBS Bearer for a multicast session.

FFS on signaling details of setup the corresponding MBS Bearer for a multicast session.

FFS on whether to include UE join information within the F1AP UE Context signalling.

FFS on how to establish the shared F1-U tunnel for a multicast session

FFS on introducing a non UE associated Class1 E1AP procedure to support Shared NG-U transport.

In this contribution, we provide further analysis and proposals on these issues. Accordingly TPs for TS 38.473 and TS 38.463 are attached in chapter 5.

2	Discussion

Open issue 1: Whether to introduce new type of signaling connection？

According to the email discussion of last meeting [1], there are two points on this issue：

- Option 1：Introduce a MBS session associated signaling connection

- Option 2：Identify different signaling connection based on MBS session id

Option 2 may be difficult to identify a signaling connection in some scenarios, e.g, for same MBS session, if one gNB-CU connects to multiple gNB-DUs. Then in case of gNB-DU initiated MBS context modification procedure, the gNB-CU cannot identify the corresponding F1AP signaling connection only based on MBS session ID. Of course, it is possible for gNB-CU to identify different gNB-DUs by information from lower layer, but such design is not good due to across protocol layer. Additionally, in 4G MBMS, a dedicated association pair: MCE MBMS M2AP ID / eNB MBMS M2AP ID is used to uniquely identifies the MBMS Service association over the M2 interface. Therefore, it is proposed for RAN3 to support Option 1 for MBS context management procedure.

Proposal 1: RAN3 to introduce a MBS session associated signaling connection for MBS context management procedure.

Open issue 2: Whether to introduce the gNB-DU triggered F1AP procedures for broadcast session: MBS Context Modification, MBS Context Release？

For F1AP: MBS context management procedure of broadcast session, gNB-DU should be allowed to initiate MBS Context modification procedure. In some scenarios, gNB-DU needs to change the lower layer configuration of PTM bearer, e.g, RLC, MAC/PHY parameters, etc. In addition, according to the conclusion of last meeting, F1-U supports only unicast transport tunnel, then it is possible for gNB-DU to initiate the change of DL F1-U tunnel address. On the other hand, gNB-DU should be allowed to initiate MBS Context release procedure, as for legacy UE context release procedure (gNB-DU initiated), such procedure generally is used for some abnormal case, e.g, buffer overflow or other faults, etc.

Proposal 2: RAN3 to introduce gNB-DU triggered F1AP procedures for broadcast session: MBS Context Modification, MBS Context Release.

Open issue 3: Whether to introduce the gNB-CU-UP triggered E1AP procedures for broadcast session: MBS Bearer Modification, MBS Bearer Release？

For E1AP: MBS bearer management procedure of broadcast session, gNB-CU-UP should be allowed to initiate MBS Bearer modification procedure. In some scenarios, gNB-CU-UP needs to change the PDCP configuration of PTM bearer, or the mapping relation of flow to MRB, etc. Additionally, gNB-CU-UP maybe change DL NG-U tunnel address. On the other hand, gNB-CU-UP should be allowed to initiate MBS Bearer release procedure, as for legacy Bearer Context Release Request (gNB-CU-UP initiated), such procedure can be used for some abnormal case, e.g, PDCP buffer overflow or other faults, etc.

Proposal 3: RAN3 to introduce gNB-CU-UP triggered E1AP procedures for broadcast session: MBS Bearer Modification, MBS Bearer Release.

Open issue 4: Which name should be used for the non-UE associated F1/E1 procedures for broadcast session？

The MBS procedure for broadcast session in TS 23.247 is named as: MBS Session Resource Setup Request procedure, MBS Session Resource Update Request procedure, MBS Session Resource Release Request procedure, etc. To align with TS 23.247, in F1AP, a series of similar name are considered for broadcast session: MBS Session Resource Setup Request/Response procedure, MBS Session Resource Modification Request/Response procedure, MBS Session Resource Release Request/Response procedure. Here, "Update" is not adopted, considering that F1AP may not only updates the MBS session attributes, but also modifies the radio resource of the MBS session, so it is replaced with "Modification". Accordingly, a series of similar name are used for E1AP, i.e, MBS Session Resource Setup Request/Response procedure, MBS Session Resource Modification Request/Response procedure, MBS Session Resource Release Request/Response procedure.

Proposal 4: To align with TS 23.247, in F1AP/E1AP, the following names can be used for broadcast session: MBS Session Resource Setup Request/Response procedure, MBS Session Resource Modification Request/Response procedure, MBS Session Resource Release Request/Response procedure.

Open issue 5: How to setup the corresponding MBS context for a multicast session？

On this issue, according to the last email discussion [1], there are two options to be considered：

- Option 1: Define new non-UE associated F1/E1 procedures

- Option 2: Reuse UE-associated E1AP and F1AP procedures

In fact, for the MBS context of multicast session, it contains two parts, one is per UE information, and the other is per session information, especially in case of split MRB with PTP and PTM. Since the reception of only RRC connected UE is supported for multicast session in R17, the UE associated procedure seems to be mandatory for the establishment of MBS context for the UE part over F1 interface. Additionally, according to the last email discussion [1], most companies support Option 2.

Proposal 5: For multicast session, reuse UE-associated E1AP and F1AP procedures for the establishment of MBS context for the UE part over F1/E1 interface.

Open issue 6: Whether to include UE join information within the F1AP UE Context signalling？

Currently, the PDU session management procedure over NG interface contains UE join information, as follows: “The reference to the MBS Session which the UE has joined. and, if applicable, the associated QoS flows, are included in a PDU Session Resources Item and maintained within the NG-RAN UE Context.” As mentioned in P5, the UE associated procedure is mandatory to complete the establishment of multicast session over F1 interface. Thus the F1AP UE context signaling should contain the UE join information information.

Proposal 6: For multicast session, UE join information should be included in the F1AP UE context signaling.

Open issue 7: How to establish the shared F1-U tunnel for a multicast session？

On this issue, according to the last email discussion [1], there are two options to be considered：

- Option 1: A non UE associated Class1 F1AP/E1AP procedure, e.g. named as Multicast Distribution Setup procedure

- Option 2: Reuse UE associated F1/E1 procedures

In fact, the shared F1-U tunnel itself is non UE associated, so it is more suitable to use non UE associated signaling procedure, i.e, Option 1 seems reasonable. But even with Option 1, there are different sub-options:

- Option 1.1: Use the class 1 signaling procedure initiated by gNB-DU, e.g. Multicast Distribution Setup procedure [2]

- Option 1.2: Use the class 1 signaling process initiated by gNB-CU, e.g, MBS Session Resource Context Setup Request/Response procedure [3]

The advantage of Option 1.2 is that the response message can carry admitting result, but Option 1.1 needs to address the issue. In this regard, the NG interface faces the same issue. So in last meeting, most companies assume that it should be align with NG interface.

Proposal 7: The establishment of shared F1-U tunnel for multicast session is align with the NG interface.

Open issue 8: Signaling details of setup the corresponding MBS Bearer for a multicast session

According to the last email discussion [1], there are some points involving signaling details of setup the corresponding MBS Bearer for a multicast session over F1AP/E1AP as below:

- Option 1: Enhance DRB by adding MBS info;

- Option 2: Introduce new IE to configure the MRB;

From our understanding, RAN3 is still in stage 2 of the establishment of multicast session. Additionally, the configuration of MRB involves a lot of RAN2 protocol design, e.g, the definition of various MRB types, the relationship between MRB ID and DRB ID, and so on. Therefore, this topic needs to wait for further progress of RAN2.

Proposal 8: Signaling details of MBS Bearer for a multicast session subjects to RAN2 progress.

Open issue 9: Whether to introduce a non UE associated Class1 E1AP procedure to support Shared NG-U transport.？

According to the last email discussion, one company proposed to introduce a class 1 process in E1AP to support the establishment of shared Ng-U Transport [2], in fact, the issue is related to how to design a non-UE associated procedure to establish the MBS bearer for multicast session over E1AP. A signaling example of the establishment of the MBS bearer for multicast session is shown as follows:

Figure 1 The establishment of the MBS bearer for multicast session

Anyway, we think that only after the signaling procedure for multicast session is completed over NG interface, the full picture for signaling flow of the MBS bearer for multicast session can be considered in RAN3.

Proposal 9: How to use a non UE associated Class1 E1AP procedure to support Shared NG-U transport is considered only after the signaling procedure for multicast session is completed over NG interface.

3	Conclusion

In the previous sections we made the following observations and proposals:

Proposal 1: RAN3 to introduce a MBS session associated signaling connection for MBS context management procedure.

Proposal 2: RAN3 to introduce gNB-DU triggered F1AP procedures for broadcast session: MBS Context Modification, MBS Context Release.

Proposal 3: RAN3 to introduce gNB-CU-UP triggered E1AP procedures for broadcast session: MBS Bearer Modification, MBS Bearer Release.

Proposal 4: To align with TS 23.247, in F1AP/E1AP, the following names can be used for broadcast session: MBS Session Resource Setup Request/Response procedure, MBS Session Resource Modification Request/Response procedure, MBS Session Resource Release Request/Response procedure.

Proposal 5: For multicast session, reuse UE-associated E1AP and F1AP procedures for the establishment of MBS context for the UE part over F1/E1 interface.

Proposal 6: For multicast session, UE join information should be included in the F1AP UE context signaling.

Proposal 7: The establishment of shared F1-U tunnel for multicast session is align with the NG interface.

Proposal 8: Signaling details of MBS Bearer for a multicast session subjects to RAN2 progress.

Proposal 9: How to use a non UE associated Class1 E1AP procedure to support Shared NG-U transport is considered only after the signaling procedure for multicast session is completed over NG interface.

4	Reference

- R3-212791	Summary of discussion on MBS Bearer Management over F1/E1, CATT

- R3-212436	Bearer management over F1 and E1, Huawei, CBN

- R3-212104	On F1 and E1 protocol aspects for NR MBS, Ericsson

5	Text Proposals for stage 3

5.1	TP for TS 38.473

--------------------------------Start of the First Change-----------------------------

8	F1AP procedures

8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| F1 Setup |  | F1 SETUP REQUEST |  | F1 SETUP RESPONSE |  | F1 SETUP FAILURE |  |
| gNB-DU Configuration Update |  | GNB-DU CONFIGURATION UPDATE |  | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-DU CONFIGURATION UPDATE FAILURE |  |
| gNB-CU Configuration Update |  | GNB-CU CONFIGURATION UPDATE |  | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU CONFIGURATION UPDATE FAILURE |  |
| UE Context Setup |  | UE CONTEXT SETUP REQUEST |  | UE CONTEXT SETUP RESPONSE |  | UE CONTEXT SETUP FAILURE |  |
| UE Context Release (gNB-CU initiated) |  | UE CONTEXT RELEASE COMMAND |  | UE CONTEXT RELEASE COMPLETE |  |  |  |
| UE Context Modification (gNB-CU initiated) |  | UE CONTEXT MODIFICATION REQUEST |  | UE CONTEXT MODIFICATION RESPONSE |  | UE CONTEXT MODIFICATION FAILURE |  |
| UE Context Modification Required (gNB-DU initiated) |  | UE CONTEXT MODIFICATION REQUIRED |  | UE CONTEXT MODIFICATION CONFIRM |  | UE CONTEXT MODIFICATION REFUSE |  |
| Write-Replace Warning |  | WRITE-REPLACE WARNING REQUEST |  | WRITE-REPLACE WARNING RESPONSE |  |  |  |
| PWS Cancel |  | PWS CANCEL REQUEST |  | PWS CANCEL RESPONSE |  |  |  |
| gNB-DU Resource Coordination |  | GNB-DU RESOURCE COORDINATION REQUEST |  | GNB-DU RESOURCE COORDINATION RESPONSE |  |  |  |
| F1 Removal |  | F1 REMOVAL REQUEST |  | F1 REMOVAL RESPONSE |  | F1 REMOVAL FAILURE |  |
| BAP Mapping Configuration |  | BAP MAPPING CONFIGURATION |  | BAP MAPPING CONFIGURATION ACKNOWLEDGE |  |  |  |
| GNB-DU Resource Configuration |  | GNB-DU RESOURCE CONFIGURATION |  | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE |  |  |  |
| IAB TNL Address Allocation |  | IAB TNL ADDRESS REQUEST |  | IAB TNL ADDRESS RESPONSE |  |  |  |
| IAB UP Configuration Update |  | IAB UP CONFIGURATION UPDATE REQUEST |  | IAB UP CONFIGURATION UPDATE RESPONSE |  | IAB UP CONFIGURATION UPDATE FAILURE |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| Positioning Measurement |  | POSITIONING MEASUREMENT REQUEST |  | POSITIONING MEASUREMENT RESPONSE |  | POSITIONING MEASUREMENT FAILURE |  |
| Positioning Information Exchange |  | POSITIONING INFORMATION REQUEST |  | POSITIONING INFORMATION RESPONSE |  | POSITIONING INFORMATION FAILURE |  |
| TRP Information Exchange |  | TRP INFORMATION REQUEST |  | TRP INFORMATION RESPONSE |  | TRP INFORMATION FAILURE |  |
| Positioning Activation |  | POSITIONING ACTIVATION REQUEST |  | POSITIONING ACTIVATION RESPONSE |  | POSITIONING ACTIVATION FAILURE |  |
| E-CID Measurement Initiation |  | E-CID MEASUREMENT INITIATION REQUEST |  | E-CID MEASUREMENT INITIATION RESPONSE |  | E-CID MEASUREMENT INITIATION FAILURE |  |
| MBS Session Resource Setup |  | MBS SESSION RESOURCE SETUP REQUEST |  | MBS SESSION RESOURCE SETUP RESPONSE |  | MBS SESSION RESOURCE SETUP FAILURE |  |
| MBS Session Resource Release (gNB-CU initiated) |  | MBS SESSION RESOURCE RELEASE COMMAND |  | MBS SESSION RESOURCE RELEASE COMPLETE |  |  |  |
| MBS Session Resource Modification (gNB-CU initiated) |  | MBS SESSION RESOURCE MODIFICATION REQUEST |  | MBS SESSION RESOURCE MODIFICATION RESPONSE |  | MBS SESSION RESOURCE MODIFICATION FAILURE |  |
| MBS Session Resource Modification Required (gNB-DU initiated) |  | MBS SESSION RESOURCE MODIFICATION REQUIRED |  | MBS SESSION RESOURCE MODIFICATION CONFIRM |  | MBS SESSION RESOURCE MODIFICATION REFUSE |  |

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |
| MBS Session Resource Release Request (gNB-DU initiated) |  | MBS SESSION RESOURCE RELEASE REQUEST |  |

------------------------------------------Next Change---------------------------------------

8.X	MBS Session Resource Management procedures

8.X.1	MBS Session Resource Setup

8.X.1.1	General

The purpose of the MBS Session Resource Setup procedure is to establish the MBS Context including MRB configuration. The procedure uses MBS-Session-associated signaling.

8.X.1.2	Successful Operation

TBD

8.X.1.3	Unsuccessful Operation

TBD

8.3.X.4	Abnormal Conditions

TBD

8.X.2	MBS Session Resource Modification (gNB-CU initiated)

8.X.2.1	General

The purpose of the MBS Session Resource Modification procedure is to modify the established MBS Context, e.g., establishing, modifying and releasing radio resources for PTM bearer. The procedure uses MBS-Session-associated signalling

8.X.2.2	Successful Operation

TBD

8.X.2.3	Unsuccessful Operation

TBD

8.X.2.4	Abnormal Conditions

TBD

8.X.3	MBS Session Resource Modification Required (gNB-DU initiated)

8.X.3.1	General

The purpose of the MBS Session Resource Modification Required procedure is to modify the established MBS Context, e.g., modifying and releasing radio resources for PTM bearer. The procedure uses MBS-Session-associated signalling.

8. X.3.2	Successful Operation

TBD

8. X.3.3	Abnormal Conditions

TBD

8.X.4	MBS Session Resource Release (gNB-CU initiated)

8.X.4.1	General

The purpose of the MBS Session Resource Release procedure is to enable the gNB-CU to order the release of the MBS context associated with a specific MBS session. The procedure uses MBS-Session-associated signalling.

8.X.4.2	Successful Operation

TBD

8.X.4.3	Abnormal Conditions

TBD

8.X.5	MBS Session Resource Release Request (gNB-DU initiated)

8.X.5.1	General

The purpose of the MBS Session Resource Release Request procedure is to enable the gNB-DU to request the gNB-CU to release the MBS context associated with a specific MBS session. The procedure uses MBS-Session-associated signalling.

8.X.5.2	Successful Operation

TBD

8.X.5.3	Abnormal Conditions

TBD

------------------------------------------Next Change---------------------------------------

9.2.X	MBS Session Resource Management messages

9.2.X.1	MBS SESSION RESOURCE SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a MBS context.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | FFS |  | YES | reject |
| MBS Session ID | M |  | FFS |  |  |  |
| TBD |  |  |  |  |  |  |
| MBS Session Level QoS parameters | M |  |  |  |  |  |
| TBD |  |  |  |  |  |  |

9.2.X.2	MBS SESSION RESOURCE SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a MBS context.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | FFS |  | YES | reject |
| TBD |  |  |  |  |  |  |

9.2.X.3	MBS SESSION RESOURCE SETUP FAILURE

This message is sent by the gNB-DU to indicate that the setup of the MBS context was unsuccessful.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | FFS |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.X.4	MBS SESSION RESOURCE MODIFICATION REQUEST

This message is sent by the gNB-CU to provide MBS Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | FFS |  | YES | reject |
| MBS Session ID | M |  | FFS |  |  |  |
| MBS Session Level QoS parameters | O |  |  |  |  |  |
| TBD |  |  |  |  |  |  |

9.2.X.5	MBS SESSION RESOURCE MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a MBS context.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | FFS |  | YES | reject |
| TBD |  |  |  |  |  |  |

9.2.X.6	MBS SESSION RESOURCE MODIFICATION FAILURE

This message is sent by the gNB-DU to indicate a MBS context modification failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name |  |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  |  | M |  |  |  | 9.3.1.1 |  |  |  | YES |  | reject |
| gNB-CU MBS F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | FFS |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | Cause |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | Criticality Diagnostics |

9.2.X.7	MBS SESSION RESOURCE MODIFICATION REQUIRED

This message is sent by the gNB-DU to request the modification of a MBS context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | FFS |  | YES | reject |
| TBD |  |  |  |  |  |  |

9.2.X.8	MBS SESSION RESOURCE MODIFICATION CONFIRM

This message is sent by the gNB-CU to inform the gNB-DU the successful modification.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | FFS |  | YES | reject |
| TBD |  |  |  |  |  |  |

9.2.X.9	MBS SESSION RESOURCE RELEASE COMMAND

This message is sent by the gNB-CU to request the gNB-DU to release the MBS context associated with a specific MBS session.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | FFS |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

9.2.X.10	MBS SESSION RESOURCE RELEASE COMPLETE

This message is sent by the gNB-DU to confirm the release of the MBS context associated with a specific MBS session.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | FFS |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.X.11	MBS SESSION RESOURCE RELEASE REQUEST

This message is sent by the gNB-DU to request the gNB-CU to release the MBS context associated with a specific MBS session.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | FFS |  | YES | reject |
| TBD |  |  |  |  |  |  |

- -------------------------------------End of the Changes----------------------------------

5.2	TP for TS 38.463

--------------------------------Start of the First Change-----------------------------

8	E1AP procedures

8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| gNB-CU-UP E1 Setup |  | GNB-CU-UP E1 SETUP REQUEST |  | GNB-CU-UP E1 SETUP RESPONSE |  | GNB-CU-UP E1 SETUP FAILURE |  |
| gNB-CU-CP E1 Setup |  | GNB-CU-CP E1 SETUP REQUEST |  | GNB-CU-CP E1 SETUP RESPONSE |  | GNB-CU-CP E1 SETUP FAILURE |  |
| gNB-CU-UP Configuration Update |  | GNB-CU-UP CONFIGURATION UPDATE |  | GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-UP CONFIGURATION UPDATE FAILURE |  |
| gNB-CU-CP Configuration Update |  | GNB-CU-CP CONFIGURATION UPDATE |  | GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-CP CONFIGURATION UPDATE FAILURE |  |
| E1 Release |  | E1 RELEASE REQUEST |  | E1 RELEASE RESPONSE |  |  |  |
| Bearer Context Setup |  | BEARER CONTEXT SETUP REQUEST |  | BEARER CONTEXT SETUP RESPONSE |  | BEARER CONTEXT SETUP FAILURE |  |
| Bearer Context Modification (gNB-CU-CP initiated) |  | BEARER CONTEXT MODIFICATION REQUEST |  | BEARER CONTEXT MODIFICATION RESPONSE |  | BEARER CONTEXT MODIFICATION FAILURE |  |
| Bearer Context Modification Required (gNB-CU-UP initiated) |  | BEARER CONTEXT MODIFICATION REQUIRED |  | BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| Bearer Context Release (gNB-CU-CP initiated) |  | BEARER CONTEXT RELEASE COMMAND |  | BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| IAB UP TNL Address Update |  | IAB UP TNL ADDRESS UPDATE |  | IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE |  | IAB UP TNL ADDRESS UPDATE FAILURE |  |
| MBS Session Resource Setup |  | MBS SESSION RESOURCE SETUP REQUEST |  | MBS SESSION RESOURCE MODIFICATION RESPONSE |  | MBS SESSION RESOURCE MODIFICATION FAILURE |  |
| MBS Session Resource Modification (gNB-CU-CP initiated) |  | MBS SESSION RESOURCE MODIFICATION REQUEST |  | MBS SESSION RESOURCE SETUP RESPONSE |  | MBS SESSION RESOURCE SETUP FAILURE |  |
| MBS Session Resource Modification Required (gNB-CU-UP initiated) |  | MBS SESSION RESOURCE MODIFICATION REQUIRED |  | MBS SESSION RESOURCE MODIFICATION CONFIRM |  |  |  |
| MBS Session Resource Release (gNB-CU-CP initiated) |  | MBS SESSION RESOURCE RELEASE COMMAND |  | MBS SESSION RESOURCE RELEASE COMPLETE |  |  |  |

Table 2: Class 2 procedures

| Elementary Procedure |  |  | Message |  |  |
| --- | --- | --- | --- | --- | --- |
| Error Indication |  |  | ERROR INDICATION |  |  |
| Bearer Context Release Request (gNB-CU-UP initiated) |  |  | BEARER CONTEXT RELEASE REQUEST |  |  |
| Bearer Context Inactivity Notification |  |  | BEARER CONTEXT INACTIVITY NOTIFICATION |  |  |
| DL Data Notification |  |  | DL DATA NOTIFICATION |  |  |
| UL Data Notification |  |  | UL DATA NOTIFICATION |  |  |
| Data Usage Report |  |  | DATA USAGE REPORT |  |  |
| gNB-CU-UP Counter Check |  |  | GNB-CU-UP COUNTER CHECK |  |  |
| gNB-CU-UP Status Indication |  |  | GNB-CU-UP STATUS INDICATION |  |  |
| MR-DC Data Usage Report |  |  | MR-DC DATA USAGE REPORT |  |  |
| Trace Start |  |  | TRACE START |  |  |
| Deactivate Trace |  |  | DEACTIVATE TRACE |  |  |
| Resource Status Reporting |  |  | RESOURCE STATUS UPDATE |  |  |
| Early Forwarding SN Transfer |  |  | EARLY FORWARDING SN TRANSFER |  |  |
| GNB-CU-CP Measurement Results Information |  |  | GNB-CU-CP MEASUREMENT RESULTS INFORMATION |  |  |
| MBS Session Resource Release Request (gNB-CU-UP initiated) |  |  | MBS SESSION RESOURCE RELEASE REQUEST |  |  |

------------------------------------------Next Change---------------------------------------

8.3.X	MBS Session Resource Setup

8.3.X.1	General

The purpose of the MBS Session Resource Setup procedure is to allow the gNB-CU-CP to establish a MBS bearer context in the gNB-CU-UP. The procedure uses MBS-session-associated signaling.

8.3.X.2	Successful Operation

TBD

8.3.X.3	Unsuccessful Operation

TBD

8.3.X.4	Abnormal Conditions

TBD

8.3.Y	MBS Session Resource Modification (gNB-CU-CP initiated)

8.3.Y.1	General

The purpose of the MBS Session Resource Modification procedure is to allow the gNB-CU-CP to modify a MBS bearer context in the gNB-CU-UP. The procedure uses MBS-session-associated signaling.

8.3.Y.2	Successful Operation

TBD

8.3.Y.3	Unsuccessful Operation

TBD

8.3.Y.4	Abnormal Conditions

TBD

8.3.Z	MBS Session Resource Modification Required (gNB-CU-UP initiated)

8.3.Z.1	General

The purpose of the MBS Session Resource Modification Required procedure is to allow the gNB-CU-UP to modify a MBS bearer context (e.g., due to local problems) and inform the gNB-CU-CP. The procedure uses MBS-session-associated signaling.

8.3.Z.2	Successful Operation

TBD

8.3.Z.3	Abnormal Conditions

TBD

8.3.XX	MBS Session Resource Release (gNB-CU-CP initiated)

8.3.XX.1	General

The purpose of the MBS Session Resource Release procedure is to allow the gNB-CU-CP to command the release of all MBS bearer context in the gNB-CU-UP. The procedure uses MBS-session-associated signaling.

8.3.XX.2	Successful Operation

TBD

8.3.XX.3	Abnormal Conditions

TBD

8.3.YY	MBS Session Resource Release Request (gNB-CU-UP initiated)

8.3.YY.1	General

The purpose of the MBS Session Resource Release Request procedure is to allow the gNB-CU-UP to request the gNB-CU-CP to release all MBS bearer context. The procedure uses MBS-session-associated signaling.

8.3.YY.2	Successful Operation

TBD

8.3.YY.3	Abnormal Conditions

TBD

------------------------------------------Next Change---------------------------------------

9.2.X.1	MBS SESSION RESOURCE SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a MBS bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| MBS Session ID | M |  | FFS |  |  |  |
| >MRB To Setup Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| >>QoS Flows Information To Be Setup | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |

9.2.X.2	MBS SESSION RESOURCE SETUP RESPONSE

This message is sent by the gNB-CU-UP to confirm the setup of the requested MBS bearer context.

Direction:gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | FFS |  | YES | reject |
| MRB Setup List |  | 1 |  |  | - | - |
| >MRB Setup Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| TBD |  |  |  |  |  |  |
| MRB Failed List |  | 0.. 1 |  |  | - | - |
| >MRB Failed Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |

9.2.X.3	MBS SESSION RESOURCE SETUP FAILURE

This message is sent by the gNB-CU-UP to indicate that the setup of the MBS bearer context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.X.4	MBS SESSION RESOURCE MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify a MBS bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | FFS |  | YES | reject |
| MBS Session ID | M |  | FFS |  |  |  |
| MRB To Setup List |  | 1 |  |  | - | - |
| >MRB To Setup Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| >>QoS Flows Information To Be Setup | M |  | FFS |  | - | - |
| MRB To Modify List |  | 1 |  |  | - | - |
| >MRB To Modify Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB To Remove List |  | 1 |  |  | - | - |
| >MRB To Remove Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.2.X.5	MBS SESSION RESOURCE MODIFICATION RESPONSE

This message is sent by the gNB-CU-UP to confirm the modification of the requested MBS bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | FFS |  | YES | reject |
| MRB Setup List |  | 1 |  |  | - | - |
| >MRB Setup Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB Failed List |  | 1 |  |  | - | - |
| >MRB Failed Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB Modified List |  | 1 |  |  | - | - |
| >MRB Modified Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB Failed to Modify List |  | 1 |  |  | - | - |
| >MRB Failed to Modify Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |

9.2.X.6	MBS SESSION RESOURCE MODIFICATION FAILURE

This message is sent by the gNB-CU-UP to indicate that the modification of the MBS bearer context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  |  | YES |  | reject |  |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | FFS |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | Cause |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | Criticality Diagnostics |

9.2.X.7	MBS SESSION RESOURCE MODIFICATION REQUIRED

This message is sent by the gNB-CU-UP to inform the gNB-CU-CP that a modification of a MBS bearer context is required (e.g., due to local problems at the gNB-CU-UP).

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | FFS |  | YES | reject |
| MRB To Modify List |  | 1 |  |  | - | - |
| >MRB To Modify Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| MRB To Remove List |  | 1 |  |  | - | - |
| >MRB To Remove Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| TBD |  |  |  |  |  |  |

9.2.X.8	MBS SESSION RESOURCE MODIFICATION CONFIRM

This message is sent by the gNB-CU-CP to confirm the modification of the requested MBS bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | FFS |  | YES | reject |
| MRB Modified List |  | 1 |  |  | - | - |
| >MRB Modified Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD | M |  | FFS |  | - | - |
| TBD |  |  |  |  |  |  |

9.2.X.9	MBS SESSION RESOURCE RELEASE COMMAND

This message is sent by the gNB-CU-CP to command the gNB-CU-UP to release all MBS bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | FFS |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

9.2.X.10	MBS SESSION RESOURCE RELEASE COMPLETE

This message is sent by the gNB-CU-UP to confirm the release of all MBS bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | FFS |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.X.11	MBS SESSION RESOURCE RELEASE REQUEST

This message is sent by the gNB-CU-UP to request the release of all MBS bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | FFS |  | YES | reject |
| MRB Status List |  | 0.. 1 |  |  | YES | ignore |
| >MRB Status Item |  | 1..<maxnoofMRBs> |  |  | - | - |
| >> TBD |  |  |  |  |  |  |
| TBD |  |  |  |  |  |  |

- -------------------------------------End of the Changes----------------------------------