3GPP TSG-RAN WG3 Meeting #113-e	R3-213568

E-meeting, 16-26 Aug 2021

Title: 	Consideration on inter-MN handover without SN change

Source: 	Huawei

Agenda item:	8.1

Document Type:	Discussion

1. Introduction

In May meeting, RAN2 sends an LS [1] to RAN3 on inter-MN handover without SN change, with the following description and Actions:

1. Overall Description: For the inter-MN handover without SN change scenario, RAN2 discussed how MN can indicate whether SN should apply delta signalling or full configuration. For one of the options discussed, the signalling of the fields is as shown by the example below: - Inter-MN HO without SN change (implies SN may provide delta configuration) - SN UE X2/XnAP ID present - sourceConfigSCG not present - scg-RB-Config not present - Inter-MN HO without SN change (implies SN must apply full configuration) - SN UE X2/XnAP ID not present - sourceConfigSCG not present - scg-RB-Config not present RAN2 would like to ask RAN3 the following questions: Question 1: In the inter-MN handover without SN change scenario, is the SN UE X2/XnAP ID always required to be present when target MN sends SN Addition Request to SN? Question 2: For the same scenario, RAN2 would like to confirm with RAN3 if the receipt of SN UE X2/XnAP ID alone may be interpreted by SN to retrieve the SCG configuration to provide delta configuration? 2. Actions: To RAN3 group. ACTION: RAN2 respectfully asks RAN3 to answer the above questions.

In this contribution, we provide our analyses on these questions.

3. Discussion

Case 1: SN UE X2/XnAP ID is present

In case the SN UE X2/XnAP ID is provided from the MN to the SN in SN Addition Request, the SN will be able to find the UE context in the SN, therefore the SN is able/allowed to use delta configuration, the first bullet from RAN2 is straight forward.

About the Question 2 from RAN2 “For the same scenario, RAN2 would like to confirm with RAN3 if the receipt of SN UE X2/XnAP ID alone may be interpreted by SN to retrieve the SCG configuration to provide delta configuration?”, we propose to feedback to RAN2 to confirm their understanding.

Proposal 1: Confirm the RAN2 understanding in RAN2 Question 2.

Case 2: SN UE X2/XnAP ID is not present

In case the SN UE X2/XnAP ID is not provided the MN to the SN in SN Addition Request, the UE will be regarded as a new coming UE from the SN point of view.

If the sourceConfigSCG and SCG-RB-Config are provided, the SN able to use delta configuration.

If the sourceConfigSCG and SCG-RB-Config are not provided, the SN has to apply full configuration.

Therefore the 2nd bullet in RAN2 LS is also a correct interpretation of the receiving IEs.

It could be understand that for the inter-MN handover without SN change scenario, if the SN UE X2/XnAP ID is not included, although the source SN and the target SN are the same SN, the SN will treat the UE as a new coming UE. Then about RAN2 Question 1 “In the inter-MN handover without SN change scenario, is the SN UE X2/XnAP ID always required to be present when target MN sends SN Addition Request to SN?”, the answer should be no.

Proposal 2: Feedback to RAN2 that in the inter-MN handover without SN change scenario, the SN UE X2/XnAP ID may not be provided, and then the UE will be regarded as a new coming UE by the SN.

With the analyses above, when we look at the current stage 2 text in TS 37.340 section 10.7, it can be found that it does not elaborate the case when both the SN UE X2/XnAP ID and UE Context in the SN are not provided, and in such case full RRC configuration has to be applied:

EN-DC 2. If the target MN decides to keep the SN, the target MN sends SN Addition Request to the SN including the SN UE X2AP ID as a reference to the UE context in the SN that was established by the source MN. If the target MN decides to change the SN, the target MN sends the SgNB Addition Request to the target SN including the UE context in the source SN that was established by the source MN. 3. The (target) SN replies with SN Addition Request Acknowledge. The (target) SN may include the indication of the full or delta RRC configuration.

MR-DC with 5GC 2. If the target MN decides to keep the source SN, the target MN sends SN Addition Request to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the target MN decides to change the SN, the target MN sends the SN Addition Request to the target SN including the UE context in the source SN that was established by the source MN. 3. The (target) SN replies with SN Addition Request Acknowledge. The (target) SN may include the indication of the full or delta RRC configuration.

Proposal 3: Elaborate in TS 37.340 that in case the SN UE X2/XnAP ID is not included in the SN Addition Request, if the UE context in the source SN is not provided as well, full RRC configuration has to be applied by the SN.

4. Conclusion

In this contribution, we analyse the RAN2 incoming LS, get the following proposals:

Proposal 1: Confirm the RAN2 understanding in RAN2 Question 2.

Proposal 2: Feedback to RAN2 that in the inter-MN handover without SN change scenario, the SN UE X2/XnAP ID may not be provided, and then the UE will be regarded as a new coming UE by the SN.

Proposal 3: Elaborate in TS 37.340 that in case the SN UE X2/XnAP ID is not included in the SN Addition Request, if the UE context in the source SN is not provided as well, full RRC configuration has to be applied by the SN.

The corresponding Draft Reply LS to RAN2 and the draftCRs to TS37.340 are provided in [2], [3] and [4].

5. Reference

- R2-2106682 LS on inter-MN handover without SN change, Source:	 TSG RAN WG2, To: TSG RAN3

- R3-213571 [Draft]Reply LS on inter-MN handover without SN change, To: TSG RAN2, Huawei

- R3-213569 Clarification on delta or full RRC configuration in inter-MN handover without SN change, draft CR to TS 37.340, Rel-15

- R3-213570 Clarification on delta or full RRC configuration in inter-MN handover without SN change, draft CR to TS 37.340, Rel-16