3GPP TSG-RAN WG3 Meeting #113-e	R3-213614

E-meeting, 16-27 Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR |  | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for POS BL CR for TS 38.455): Support of UL-AOA/ZOA assistance information signalling for NR positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2021-07-23 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To introduce the support for UL-AOA/ZOA assistance information signalling for multiple positioning methods. |  |  |  |  |  |  |  |  |
| R3-20 |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new IE of Angle assistance information in Measurement Request message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UL-AOA/ZOA assistance information signalling cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4.1, 9.2.x, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR… CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms) | The codepoint 60min is not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16] | - |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Angle Assistance Information | O |  | 9.2.x |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

9.2.x	Angle Assistance Information

This information element contains the UL AoA/ZoA assistance information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Expected Azimuth Angle of Arrival | O |  | INTEGER(0..3599) | TS 38.133 [16] |
| Expected Zenith Angle of Arrival | O |  | INTEGER(0..1799) | TS 38.133 [16] |
| Angle Uncertainty | M |  |  |  |
| >Azimuth Uncertainty | O |  | INTEGER(0..255) |  |
| >Zenith Uncertainty | O |  | INTEGER(0..255) |  |
| >Resolution | M |  | ENUMERATED (0.1deg, …) |  |
| LCS to GCS Translation |  | 0..1 |  | If absent, the azimuth and zenith are provided in GCS. |
| >Alpha | M |  | INTEGER (0..3599) |  |
| >Beta | M |  | INTEGER (0..3599) |  |
| >Gamma | M |  | INTEGER (0..3599) |  |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for NRPPa

--

-- **************************************************************

NRPPA-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

E-CID-MeasurementResult,

OTDOACells,

OTDOA-Information-Item,

Measurement-ID,

UE-Measurement-ID,

MeasurementPeriodicity,

MeasurementQuantities,

ReportCharacteristics,

RequestedSRSTransmissionCharacteristics,

Cell-Portion-ID,

OtherRATMeasurementQuantities,

OtherRATMeasurementResult,

WLANMeasurementQuantities,

WLANMeasurementResult,

Assistance-Information,

Broadcast,

AssistanceInformationFailureList,

SRSConfiguration,

TRPMeasurementQuantities,

TrpMeasurementResult,

TRP-ID,

TRPInformationTypeListTRPReq,

TRPInformationListTRPResp,

TRP-MeasurementRequestList,

TRP-MeasurementResponseList,

MeasurementBeamInfoRequest,

PositioningBroadcastCells,

SRSResourceSetID,

SpatialRelationInfo,

SRSResourceTrigger,

TRPList,

AbortTransmission,

SystemFrameNumber,

SlotNumber,

RelativeTime1900,

AngleAssistanceInformation

FROM NRPPA-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-Single-Container{},

NRPPA-PRIVATE-IES,

NRPPA-PROTOCOL-EXTENSION,

NRPPA-PROTOCOL-IES

FROM NRPPA-Containers

maxnoOTDOAtypes,

id-Cause,

id-CriticalityDiagnostics,

id-LMF-Measurement-ID,

id-LMF-UE-Measurement-ID,

id-OTDOACells,

id-OTDOA-Information-Type-Group,

id-OTDOA-Information-Type-Item,

id-ReportCharacteristics,

id-MeasurementPeriodicity,

id-MeasurementQuantities,

id-RAN-Measurement-ID,

id-RAN-UE-Measurement-ID,

id-E-CID-MeasurementResult,

id-RequestedSRSTransmissionCharacteristics,

id-Cell-Portion-ID,

id-OtherRATMeasurementQuantities,

id-OtherRATMeasurementResult,

id-WLANMeasurementQuantities,

id-WLANMeasurementResult,

id-Assistance-Information,

id-Broadcast,

id-AssistanceInformationFailureList,

id-SRSConfiguration,

id-TRPMeasurementQuantities,

id-MeasurementResult,

id-TRP-ID,

id-TRPInformationTypeListTRPReq,

id-TRPInformationListTRPResp,

id-TRP-MeasurementRequestList,

id-TRP-MeasurementResponseList,

id-TRP-MeasurementReportList,

id-MeasurementBeamInfoRequest,

id-PositioningBroadcastCells,

id-SRSType,

id-ActivationTime,

id-SRSResourceSetID,

id-TRPList,

id-SRSSpatialRelation,

id-AbortTransmission,

id-SystemFrameNumber,

id-SlotNumber,

id-SRSResourceTrigger,

id-SFNInitialisationTime,

id-AngleAssistanceInformation

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- MEASUREMENT REQUEST

--

-- **************************************************************

MeasurementRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementRequest-IEs}},

...

}

MeasurementRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-TRP-MeasurementRequestList		CRITICALITY reject	TYPE TRP-MeasurementRequestList PRESENCE mandatory}|

{ ID id-ReportCharacteristics			CRITICALITY reject	TYPE ReportCharacteristics		PRESENCE mandatory}|

{ ID id-MeasurementPeriodicity			CRITICALITY reject	TYPE MeasurementPeriodicity		PRESENCE conditional}|

-- The IE shall be present if the Report Characteritics IE is set to “periodic” –

{ ID id-TRPMeasurementQuantities			CRITICALITY reject	TYPE TRPMeasurementQuantities		PRESENCE mandatory}|

{ ID id-SFNInitialisationTime			CRITICALITY ignore	TYPE RelativeTime1900		PRESENCE optional}|

{ ID id-SRSConfiguration				CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-MeasurementBeamInfoRequest		CRITICALITY ignore	TYPE MeasurementBeamInfoRequest	PRESENCE optional}|

{ ID id-SystemFrameNumber				CRITICALITY ignore	TYPE SystemFrameNumber			PRESENCE optional}|

{ ID id-SlotNumber						CRITICALITY ignore	TYPE SlotNumber					PRESENCE optional}|

{ ID id-AngleAssistanceInformation		CRITICALITY ignore	TYPE AngleAssistanceInformation	PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

AngleAssistanceInformation ::= SEQUENCE {

expectedAzimuthAoA					INTEGER (0..3599)				OPTIONAL,

expectedZenithAoA					INTEGER (0..1799)				OPTIONAL,

angleUncertainty					AngleUncertainty,

lCS-to-GCS-Translation				LCS-to-GCS-TranslationAoA		OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { AngleAssistanceInformation-ExtIEs } }	OPTIONAL,

...

}

AngleAssistanceInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

AngleUncertainty ::= SEQUENCE {

azimuthUncertainty		INTEGER (0..255) 	OPTIONAL,

zenithUncertainty		INTEGER (0..255)	OPTIONAL,

resolution				ENUMERATED {deg0dot1, ...},

iE-extensions		ProtocolExtensionContainer { { AngleUncertainty-ExtIEs } } OPTIONAL,

...

}

AngleUncertainty-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-LMF-UE-Measurement-ID										ProtocolIE-ID ::= 2

id-ReportCharacteristics										ProtocolIE-ID ::= 3

id-MeasurementPeriodicity										ProtocolIE-ID ::= 4

id-MeasurementQuantities										ProtocolIE-ID ::= 5

id-RAN-UE-Measurement-ID										ProtocolIE-ID ::= 6

id-E-CID-MeasurementResult										ProtocolIE-ID ::= 7

id-OTDOACells													ProtocolIE-ID ::= 8

id-OTDOA-Information-Type-Group									ProtocolIE-ID ::= 9

id-OTDOA-Information-Type-Item									ProtocolIE-ID ::= 10

id-MeasurementQuantities-Item									ProtocolIE-ID ::= 11

id-RequestedSRSTransmissionCharacteristics						ProtocolIE-ID ::= 12

id-Cell-Portion-ID												ProtocolIE-ID ::= 14

id-OtherRATMeasurementQuantities								ProtocolIE-ID ::= 15

id-OtherRATMeasurementQuantities-Item							ProtocolIE-ID ::= 16

id-OtherRATMeasurementResult									ProtocolIE-ID ::= 17

id-WLANMeasurementQuantities									ProtocolIE-ID ::= 19

id-WLANMeasurementQuantities-Item								ProtocolIE-ID ::= 20

id-WLANMeasurementResult										ProtocolIE-ID ::= 21

id-TDD-Config-EUTRA-Item										ProtocolIE-ID ::= 22

id-Assistance-Information										ProtocolIE-ID ::= 23

id-Broadcast													ProtocolIE-ID ::= 24

id-AssistanceInformationFailureList								ProtocolIE-ID ::= 25

id-SRSConfiguration												ProtocolIE-ID ::= 26

id-MeasurementResult											ProtocolIE-ID ::= 27

id-TRP-ID														ProtocolIE-ID ::= 28

id-TRPInformationTypeListTRPReq									ProtocolIE-ID ::= 29

id-TRPInformationListTRPResp									ProtocolIE-ID ::= 30

id-MeasurementBeamInfoRequest									ProtocolIE-ID ::= 31

id-ResultSS-RSRP												ProtocolIE-ID ::= 32

id-ResultSS-RSRQ												ProtocolIE-ID ::= 33

id-ResultCSI-RSRP												ProtocolIE-ID ::= 34

id-ResultCSI-RSRQ												ProtocolIE-ID ::= 35

id-AngleOfArrivalNR												ProtocolIE-ID ::= 36

id-GeographicalCoordinates										ProtocolIE-ID ::= 37

id-PositioningBroadcastCells									ProtocolIE-ID ::= 38

id-LMF-Measurement-ID											ProtocolIE-ID ::= 39

id-RAN-Measurement-ID											ProtocolIE-ID ::= 40

id-TRP-MeasurementRequestList									ProtocolIE-ID ::= 41

id-TRP-MeasurementResponseList									ProtocolIE-ID ::= 42

id-TRP-MeasurementReportList									ProtocolIE-ID ::= 43

id-SRSType														ProtocolIE-ID ::= 44

id-ActivationTime												ProtocolIE-ID ::= 45

id-SRSResourceSetID												ProtocolIE-ID ::= 46

id-TRPList														ProtocolIE-ID ::= 47

id-SRSSpatialRelation											ProtocolIE-ID ::= 48

id-SystemFrameNumber											ProtocolIE-ID ::= 49

id-SlotNumber													ProtocolIE-ID ::= 50

id-SRSResourceTrigger											ProtocolIE-ID ::= 51

id-TRPMeasurementQuantities										ProtocolIE-ID ::= 52

id-AbortTransmission											ProtocolIE-ID ::= 53

id-SFNInitialisationTime										ProtocolIE-ID ::= 54

id-ResultNR														ProtocolIE-ID ::= 55

id-ResultEUTRA													ProtocolIE-ID ::= 56

id-TRPInformationTypeItem										ProtocolIE-ID ::= 57

id-CGI-NR														ProtocolIE-ID ::= 58

id-SFNInitialisationTime-NR										ProtocolIE-ID ::= 59

id-Cell-ID														ProtocolIE-ID ::= 60

id-SrsFrequency													ProtocolIE-ID ::= 61

id-AngleAssistanceInformation									ProtocolIE-ID ::= zz

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>