3GPP TSG-RAN WG3 Meeting #113-e	R3-213648

16 – 26 Aug. 2021, E-Meeting

Agenda item:		18.4.3

Source:	LG Electronics Inc.

Title:	Support of AI/ML enabled Mobility Optimization for NG-RAN and EN-DC

Document for:		Discussion

1. Introduction

In the past meetings, the use case description was captured in TR on mobility optimization for AI/ML enabled RAN. This paper is targeted for analyzing the impacts. The corresponding proposals are provided as well.

2. Discussion

In RAN3 #112e, the following agreements were captured on mobility optimization for TR 37.817:

- UE Location/Mobility/Performance Prediction

Predicting UE’s location is a key part for mobility optimisation, as many RRM actions related to mobility (e.g. selecting handover target cells) can benefit from the predicted UE location/trajectory. UE mobility prediction is also one key factor in the optimization of early data forwarding particularly for CHO. UE Performance prediction when the UE is served by certain cells is a key factor in determining, which is the best mobility target for maximisation of efficiency and performance.

- Traffic Steering

Efficient resource handling can be achieved adjusting handover trigger points and selecting optimal combination of Pcell/PSCell/Scells to serve a user.

Existing traffic steering can also be improved by providing a RAN node with information related to mobility or dual connectivity.

For example, before initiating a handover, the source gNB, could use feedbacks on UE performance collected for successful handovers occurred in the past and received from neighboring gNBs.

Similarly, for the case of dual connectivity, before triggering the addition of a secondary gNB or triggering SN change, an eNB could use information (feedbacks) received in the past from the gNB for successfully completed SN Addition or SN Change procedures.

In the two reported examples, the source RAN node of a mobility event, or the RAN node acting as Master Node (a eNB for EN-DC, a gNB for NR-DC) can use feedbacks received from the other RAN node, as input to an AI/ML function supporting traffic related decisions (e.g. selection of target cell in case of mobility, selection of a PSCell / Scell(s) in the other case), so that future decisions can be optimized.

Based on the use case description above, this paper is to analyze the impacts for mobility optimization.

It is assumed that the function locates on the side of NG-RAN or near the NG-RAN. With this function, it is targeted to help RAN to decide the handover target or the target for EN-DC offloading, optimization of early data forwarding particularly for CHO, adjusting handover trigger points and selecting optimal combination of Pcell/PSCell/Scells to serve a user etc.

The important work is to identify the input and output of the function for mobility optimization, which will also be the impacts from standard point of view. The following initial thinking are listed on the input:

Information from CN (the input can be based on the information from AI based CN function):

- UE mobility statistics parameters, e.g., UE location statistics (duration of the time slot)

- UE mobility predications, e.g., predicated UE location information in the analytical period

Information from UE:

- Current and past location statistical information, e.g. GPS, GNSS, cell and UE’s staying duration information

- UE may also have the training model on its locations, thus UE can report the predicated location to RAN

Information from the neighbor RAN nodes:

- UE’s successful handover information in the past and received from neighboring RAN nodes

- UE’s successful DC offloading information in the past and received from neighboring RAN nodes

- UE’s history information from neighbor

Based on the input from CN, UE and neighbor RAN node, the training model in RAN can perform the long or short term training.

Output of the training model:

- the predicated UE’s location with the confidence of the predication

- the predicated target cell IDs or gNB IDs for handover together with the confidence of the predication

- the predicated target SN node IDs for DC together with the confidence of the predication

For a specific NG-RAN handover or EN-DC offloading, the source gNB/MN will take the input from training mode into account and decide the target node. In the Handover Request message or SN Addition Request message, the output from the training model can also be included, i.e., the predicated target cell IDs or gNB IDs together with the confidence of the predication, which will be used by the target node or SN on the future handover or SN change decisions.

For the CHO or CPAC, the source node or the SN can decide whether/when to perform early data forwarding or not based on the predication results.

Proposal): To consider the input and output information above for mobility optimization in NG-RAN and EN-DC. The corresponding impacts to Uu, NG and X2/Xn interfaces should be standardized.

3. Conclusion

In this contribution, the issues were investigated for support of AI enabled Mobility in NG-RAN and EN-DC. The following proposal is suggested to RAN3:

Proposal): To consider the following input and output information for mobility optimization in NG-RAN and EN-DC. The corresponding impacts to Uu, NG and X2/Xn interfaces should be standardized.

Input Information from CN (the input can be based on the information from AI based CN function):

- UE mobility statistics parameters, e.g., UE location statistics (duration of the time slot)

- UE mobility predications, e.g., predicated UE location information in the analytical period

Input Information from UE:

- Current and past location statistical information, e.g. GPS, GNSS, cell and UE’s staying duration information

- UE may also have the training model on its locations, thus UE can report the predicated location to RAN

Input Information from the neighbor RAN nodes:

- UE’s successful handover information in the past and received from neighboring RAN nodes

- UE’s successful DC offloading information in the past and received from neighboring RAN nodes

- UE’s history information from neighbor

Output of the training model:

- the predicated UE’s location with the confidence of the predication

- the predicated target cell IDs or gNB IDs for handover together with the confidence of the predication

- the predicated target SN node IDs for DC together with the confidence of the predication

4. References

- RP-201620, “Study on enhancement for data collection for NR and ENDC”, CMCC