3GPP TSG-RAN WG3 #113-e	R3-213855
Online, 16-26 August 2021

Agenda Item:	19.2.3

Source:	Ericsson

Title:		TP to F1AP BL CR: Addition of On-demand DL-PRS information

Document for:	Other

# Introduction

RAN3#112-e has the following notes on F1AP Positioning BL CR:

2352 rev in R3-212779 CR0037r1 (E///) Endorsed as BL

2239 rev in R3-212780 CR0767r1 (HW) noted

F1AP: To be continued on this basis...

F1AP: To be continued on this basis...

Consensus to work on NRPPa and F1AP in parallel

We understand that the F1AP CR in [1] will be resubmitted in RAN3#113e to be reviewed and endorsed as baseline. We propose bellow the parallel changes of our NRPPa TP in [2] to the future F1AP BL CR.

# References

R3-212780, Introduction of NR Positioning enhancements to F1AP, Huawei

R3-213854, TP to NRPPa BL CR: Addition of On-demand DL-PRS information, Ericsson

# TP to F1AP BL CR

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

### 8.13.x	PRS Configuration Exchange

#### 8.13.x.1	General

The PRS Configuration Exchange procedure is initiated by the gNB-CU to request the gNB-DU to configure or update the PRS transmissions.

The procedure uses non-UE-associated signalling.

#### 8.13.x.2	Successful Operation

Figure 8.13.X.2-1: PRS Configuration Exchange procedure, successful operation

The gNB-CU initiates the procedure by sending a PRS CONFIGURATION REQUEST message to the gNB-DU node. The gNB-DU shall respond with the PRS CONFIGURATION RESPONSE message. If the List of PRS Configurations IE is included in the PRS CONFIGURATION REQUEST message, the gNB-DU may take it into account when configuring the DL-PRS transmission for each of its TRPs.

[FFS]

#### 8.13.x.3	Unsuccessful Operation

Figure 8.13.X.2-1: PRS Configuration Exhcange procedure, unsuccessful operation

If the gNB-DU is unable to configure any PRS transmission, it shall reply with a PRS CONFIGURATION FAILURE message.

[FFS]

#### 8.13.X.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

#### 9.2.12.x	PRS CONFIGURATION REQUEST

This message is sent by a gNB-CU to request a gNB-DU to configure or updating PRS transmissions.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| PRS TRP List |  | [FFS] |  |  | YES | ignore |
| >PRS TRP Item |  | 1 .. <maxnoofTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.3.1.197 |  | - |  |
| >>List of PRS Configurations |  | 0..1 |  | List of the proposed PRS configurations for this TRP | YES | ignore |
| >>> PRS configuration item |  | 1 .. <maxnoPRSindex> |  |  |  |  |
| >>> PRS configuration index | M |  | ENUMERATED(1,..,FFS) | Index of the PRS configuration |  |  |
| >>> PRS configuration | M |  | FFS |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPs | Maximum no. of TRPs in a gNB-DU Value is 65535 |
| maxnoPRSindex | Maximum no of PRS configurations. Value is FFS |

#### 9.2.12.x	PRS CONFIGURATION RESPONSE

This message is sent by a gNB-DU to ackknowledge the PRS transmissions.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| PRS Transmission TRP List |  | [FFS] |  |  | YES | ignore |
| >PRS Transmission TRP Item |  | 1 .. <maxnoofTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.3.1.197 |  | - |  |
| >>On-demand PRS information [FFS] | [FFS] |  | 9.3.1.b |  |  |  |
| >> PRS configuration Index | O |  | ENUMERATED(1..FFS) | The chosen PRS configuration index for this TRP | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPs | Maximum no. of TRPs in a gNB-DU Value is 65535. |

#### 9.2.12.x	PRS CONFIGURATION FAILURE

This message is sent by the gNB-DU to indicate that it cannot configure any PRS transmission..

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.b	On-demand PRS information [FFS]

This IE contains the updated PRS configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| FFS |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< END CHANGE >>>>>>>>>>>>>>>>>>>>