3GPP TSG-RAN3 Meeting # 113-e	   R3-213856

# 16th - 26th  August 2021

Agenda Item:	19.3

Source:	Ericsson

Title: 	Discussion on latency improvements for Rel-17 positioning

Document for:	Discussion

# 1	Introduction

As we have previously presented in our contribution in [1], the current positioning RAN architecture is going to stay for Rel-17. Thus, the impacts for latency improvements can be considered minimal and focused mainly on RAN2. Nevertheless, we believe that some points can be worth considered by RAN3.

# 2	Discussion

In last e-meeting, the following agreements were taken:

RAN3 should not discuss the schedule time unless there is significant progress in SA2

RAN3 wait for progress in other groups before discussing the Configuration Grant based optimization

We provide below a sequence diagram for the UL location reporting when UE is in RRC_INACTIVE state (no NRPPa signalling):

- The LMF configures the necessary positioning measurement and sets a response time by which the UE is required to provide the measurement response to the network.

- The gNB configures the CG-SDT.

- The UE while in inactive state performs the positioning measurement and decides in which RRC state the UE should reply.

- The performed measurement is then provided to the LMF within the set response time.

From the above, we can consider that the gNB is oblivious to the UE CG information, as it does not know what sort of configured grant should be applicable to the UE related to its positioning reporting requirements during step 2.  As we have highlighted in our previous contribution in [1], during the UL transmissions, the LMF configures the UE measurement reporting periodicity for location information reporting via LPP during step 1 using the ‘periodic LPP reporting’ IE. This information is defined in TS 37.355 as below:

#### CommonIEsRequestLocationInformation

The CommonIEsRequestLocationInformation carries common IEs for a Request Location Information LPP message Type.

-- ASN1START

CommonIEsRequestLocationInformation ::= SEQUENCE {

locationInformationType		LocationInformationType,

triggeredReporting			TriggeredReportingCriteria	OPTIONAL,	-- Cond ECID

periodicalReporting			PeriodicalReportingCriteria OPTIONAL,	-- Need ON

additionalInformation		AdditionalInformation		OPTIONAL,	-- Need ON

qos							QoS							OPTIONAL,	-- Need ON

environment					Environment					OPTIONAL,	-- Need ON

locationCoordinateTypes		LocationCoordinateTypes		OPTIONAL,	-- Need ON

velocityTypes				VelocityTypes				OPTIONAL,	-- Need ON

...,

[[

messageSizeLimitNB-r14	MessageSizeLimitNB-r14		OPTIONAL	-- Need ON

]],

[[

segmentationInfo-r14	SegmentationInfo-r14		OPTIONAL	-- Need ON

]]

}

...

PeriodicalReportingCriteria ::=		SEQUENCE {

reportingAmount						ENUMERATED {

ra1, ra2, ra4, ra8, ra16, ra32,

ra64, ra-Infinity

} DEFAULT ra-Infinity,

reportingInterval					ENUMERATED {

noPeriodicalReporting, ri0-25,

ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64

}

}

The gNB however does not know anything of the LPP periodic reporting for the UE. Because of this, gNB may not be able to configure UE’s UL transmissions in accordance with the UE reporting periodicity, during which the UE may provide periodic location information to LMF. There may be issues with other pre-configured UL transmissions of the UE (such as PUR) if the gNB is not aware of such.

Observation 1:	The CG-SDT mechanism may be inefficient as RAN node is unaware of the positioning reporting requirements

To enhance the CG-based signalling, one way would be that LMF provides some assistance information to gNB between steps 1 and 2 above, so that gNB can decide for the appropriate CG resources (to activate or deactivate) for the UE for reporting the positioning measurements or location estimate. This will add extra NRPPa signalling, but it provides necessary information to gNB for avoiding issues when UE has several UL CG reportings.

Proposal 1 : In order to help gNB decide to whether configure or release the CG resources for UE transmitting location measurements, the LMF needs to signal over NRPPa the UE’s positioning reporting information to gNB.

RAN2 is currently discussing and evaluating the benefits of this extra NRPPa signalling. RAN3 should also discuss the above and decide whether to define new NRPPa procedure for this purpose or re-use existing ones. We propose in the annex an example of enhancing the Positioning Information Exchange Procedure.

# 3	Conclusions

Observation 1:	The CG-SDT mechanism may be inefficient as RAN node is unaware of the positioning reporting requirements

Proposal 1 : In order to help gNB decide to whether configure or release the CG resources for UE transmitting location measurements, the LMF needs to signal over NRPPa the UE’s positioning reporting information to gNB.

# 4	References

- RP-212354, Discussion on latency improvements for Rel-17 positioning, Ericsson

# 5	Annex - Proposal for NRPPa BL CR

#### 9.1.1.10	POSITIONING INFORMATION REQUEST

This message is sent by LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |
| UE Reporting Information | O |  | 9.2.x | Corresponds to the UE LPP reporting periodicity | YES | ignore |

### 9.2.x UE Reporting Information (FFS)

This IE contains UE’s LPP periodic time transmissions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reporting Amount | M |  | ENUMERATED ( ra1, ra2, ra4, ra8, ra16, ra32, ra64, ra-Infinity) |  |
| Reporting Interval | M |  | ENUMERATED ( noPeriodicalReporting, ri0-25, ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64) |  |