3GPP TSG-RAN WG3 Meeting #113-e	R3-213917

# E-meeting, 16-26 Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0125 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Assistance information for UL duplication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Unicom, China Telecom, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIoT-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Rel-16 IIOT WI, it was agreed that “RAN3 will not introduce the network coordination in this release”. Currenlty in Rel-17, it is benefical to introduce the UL assistance information between RAN nodes, so that each node can control all Secondary RLC entities either at the MN or at the SN side. In order to do this, there is need to exchange the UL radio channel conditions in between. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the ASSISTANCE INFORMATION DATA (PDU Type 2) frame: - Add UL Assistance Info. Ind per RLC and LCH ID In the DL USER DATA (PDU Type 0) frame: - Add the Radio Quality Assistance Information and LCH ID |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UL duplication is not fully supported for the multi-connectivity scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.4.3, 5.5.2.1, 5.5.2.3, 5.5.3.Y, 5.5.3.Z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

5.4.1	Transfer of Downlink User Data

5.4.1.1	Successful operation

<Unchanged Text Omitted>

If the User data existence flag is set to 1, the corresponding node assumes that the node hosting the NR PDCP entity has some user data for the concerned data radio bearer. The corresponding node decides whether and when to use DRX for the UE (i.e. the corresponding node may indicate the UE to use DRX even if the flag is set to 1 and the received DL USER DATA frame contains no user data).

The node hosting the NR PDCP entity can indicate to the corresponding node the Radio Quality Assistance Information and the logical channel ID. The corresponding node shall, if supported, take this information into account for UL packet duplication.

<Unchanged Text Omitted>

5.4.3	Transfer of Assistance Information

5.4.3.1	Successful operation

<Unchanged Text Omitted>

The ASSISTANCE INFORMATION DATA frame may include the UL Delay or/and DL Delay measured by the corresponding node. The node hosting the NR PDCP entity may take this information into account to calculate the whole UL or/and DL delay of RAN.

The ASSISTANCE INFORMATION DATA frame may include the Logical Channel ID and the Radio Quality Assistance Information. The node hosting the NR PDCP entity shall, if supported, take this information into account for UL packet duplication.

<Unchanged Text Omitted>

5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined to allow the corresponding node to detect lost NR-U packets and is associated with the transfer of a Downlink NR PDCP PDU.

The following shows the respective DL USER DATA frame.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 |  | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  |  | Spare | DL Discard Blocks | DL Flush | Report polling | 1 |
| Spare |  |  | UL PDCP Dupl. assistance Info. Ind. per RLC Entity | Request OutofSeq Report | Report Delivered | User data existence flag | Assistance Info. Report Polling Flag | Retransmission flag | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  |  | 0 or 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  |  | 0 or 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  |  | 0 or 1 |
| DL report NR PDCP PDU SN |  |  |  |  |  |  |  |  | 0 or 3 |
| Number of Assistance Information Fields |  |  |  |  |  |  |  |  | 0 or 1 |
| Assistance Information Type |  |  |  |  |  |  |  |  | 0 or (2*Number of Assistance Info Fields + sum of Number of octets for Radio Quality Assistance Information Fields +1) |
| Number of octets for Radio Quality Assistance Information Fields |  |  |  |  |  |  |  |  |  |
| Radio Quality Assistance Information |  |  |  |  |  |  |  |  |  |
| LCH ID |  |  |  |  |  |  |  |  | 0 or 1 |
| Padding |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

<Unchanged Text Omitted>

5.5.2.3	ASSISTANCE INFORMATION DATA (PDU Type 2)

This frame format is defined to allow the node hosting the NR PDCP entity to receive assistance information.

The following shows the respective ASSISTANCE INFORMATION DATA frame.

| Bits |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 |  | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=2) |  |  |  | PDCP Dupl. Ind. |  | Assistance Info. Ind. |  | UL Delay Ind. | DL Delay Ind. | 1 |
| Spare |  |  |  |  |  |  |  | UL PDCP Dupl. assistance Info. Ind. Per RLC Entity | PDCP Duplication Activation Suggestion | 1 |
| Number of Assistance Information Fields |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Assistance Information Type |  |  |  |  |  |  |  |  |  | 0 or (2*Number of Assistance Info Fields + sum of Number of octets for Radio Quality Assistance Information Fields) |
| Number of octets for Radio Quality Assistance Information Fields |  |  |  |  |  |  |  |  |  |  |
| Radio Quality Assistance Information |  |  |  |  |  |  |  |  |  |  |
| UL Delay DU Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| DL Delay DU Result |  |  |  |  |  |  |  |  |  | 0 or 4 |
| LCH ID |  |  |  |  |  |  |  |  |  | 0 or 1 |

<Unchanged Text Omitted>

5.5.3.Y	UL PDCP Duplication Assistance Information Indicaton Per RLC Entity

Description: This field indicates the presence of the Number of Assistance Information Fields and the LCH ID.

Value range: {0= Number of Assistance Information Fields and LCH ID not present, 1= Number of Assistance Information Fields and LCH ID present}.

5.5.3.Z	LCH ID

Description: This field indicates the logical channel for the corresponding RLC.

Value range: {1..25}..

Field length: 6 bit.

<Unchanged Text Omitted>

Change Ends