3GPP TSG-RAN WG3 Meeting #113-e	R3-214066

- 16-26 Aug 2021

- Online

Agenda item:	10.2.4

Source: 	ZTE, China Telecom, China Unicom

Title: 	(TP for SON BL CR 36.300) Inter-System Load Balancing

Document for:	Agreement

Introduction

This contribution provides a TP for SON BL CR 36.300 on the Inter-System Load Balancing.

Text Proposal for TS 36.300

22.4.1.2.2	Load reporting for inter-RAT scenario

The load information consists of:

-	Cell Capacity Class value (UL/DL relative capacity indicator: the same scale shall apply to E-UTRAN, UTRAN, GERAN and eHRPD cells when mapping cell capacities on this value);

-	Capacity value (UL/DL available capacity for load balancing as percentage of total cell capacity).

-	Radio Resource Usage (UL/DL GBR PRB usage, UL/DL non-GBR PRB usage, UL/DL total PRB usage);

NOTE 1:	Capacity value is expressed in available E-UTRAN resources.

NOTE 2:	A cell is expected to accept traffic corresponding to the indicated available capacity.

Event-triggered inter-RAT load reports are sent when the reporting node detects crossing of cell load thresholds.

Load information shall be provided in a procedure separated from existing active mode mobility procedures, which shall be used infrequently and with lower priority with respect to the UE dedicated signalling.

To achieve load reporting function between LTE and NR, eNB Configuration Transfer & MME Configuration Transfer procedures are used.